| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0394-r1 |
| --- | --- |

| Title: | CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this proposal, a unified MPM list derivation is proposed, which is combination of CE3-3.3.2/CE3-3.3.3 and CE3-3.5.1. With the proposed method, the angular MPMs used in CE3-3.3.2/CE3-3.3.3 are replaced by first 5 or 6 angular MPMs derived in CE3-3.5.1. Furthermore, the modification of MPM list construction together with the encoder optimization employed in proposal JVET-N0450 are also combined in this proposal. It is reported that the combined test 7 obtains 0.25% coding gain for the AI configuration on top of VTM-4.0 with around 96% encoding time and 100% decoding time.

Introduction

In VTM-4.0, three different MPM list generation processes are employed for zero reference line with ISP off, ISP on, and non-zero reference lines, respectively. For zero reference line with ISP off, Planar and DC modes are always included in the MPM list, and 4 angular MPM candidates are derived. For ISP on, Planar mode is always included from the MPM list, DC mode is always excluded from the MPM list, and 5 angular MPM candidates are derived. For non-zero reference lines, Planar and DC modes are always excluded from the MPM list, and 6 angular MPM candidates are derived.

Harmonization of MPM list has been studied in CE3-3. For CE3-3.3 and CE3-3.5, only one MPM list is generated. Specifically, in CE3-3.3.2/3, Planar and DC modes are always the first two MPMs and 4 angular MPMs are derived using the zero-reference line MPM list generation process. In CE3-3.5.1, Planar and DC modes are coded separately, and 5/6 angular MPMs are derived by using the non-zero reference lines MPM list generation process.

Proposed methods

In this proposal, only one unified MPM list is generated, wherein Planar and DC modes are always first two MPMs in this unified MPM list. All the MPM candidates in this unified MPM list are used for zero reference line with ISP off, MPM candidates excluding DC mode in this unified MPM list are used for ISP on, and only the angular MPM candidates in this unified MPM list are used for non-zero reference lines.

5 or 6 angular MPM candidates are derived by the method in CE3-3.5.1, and the number of the unified MPM list is 7 or 8. For combined test 1 and combined test 3, there is one context for each of the first two bins of MPM index, which is the same as that in CE3-3.3.2. For combined test 2 and test 4, there are 3 and 1 contexts for the first two bins of MPM index respectively, which is the same as that in CE3-3.3.3.

To be specific, the number of angular MPMs and the number of contexts used for different combined tests are compared in Table 1.

Table I: Comparisons between different combined tests

| Test | Number of angular MPMs | Number of Contexts for MPM index |
| --- | --- | --- |
| Combined Test 1 | 5 angular modes | 2 |
| Combined Test 2 | 5 angular modes | 4 |
| Combined Test 3 | 6 angular modes | 2 |
| Combined Test 4 | 6 angular modes | 4 |

Furthermore, on top of the combined test 4, four features from JVET-N0450 are also combined, which are listed below.

- The lossless encoder speedup for the intra mode search, which simplifies the intra mode bits calculation process. (namely feature #1)

- The encoder search in section 2.1 of JVET-N0450 is applied. (namely feature #2)

- The lambda value for the initial mode search is doubled (namely feature #3)

- Only the first entry in the MPM list may be equal to VDIA_IDX. (namely feature #4)

Experimental results

The tests are implemented on top of VTM4.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-M1010 [2]. 7 combined tests are conducted to verify the performance of the proposed methods. The description of combined test 1 ~4 are illustrated in Table 1. And combined test 5 ~ 7 is based on combined test 4, and the details are listed below:

- Combined test 5: Combined test 4 + feature #1+ feature #2

- Combined test 6: Combined test 4 + feature #1 + feature #2 + feature #3

- Combined test 7: Combined test 4 + feature #1 + feature #2 + feature #3 + feature #4

Table 2: Results of combined test 1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.25% | -0.30% | 98% | 100% |  |  |  |  |  |
| Class A2 | -0.10% | 0.00% | -0.10% | 98% | 97% |  |  |  |  |  |
| Class B | -0.13% | -0.06% | -0.03% | 101% | 102% | -0.05% | -0.09% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.03% | 0.01% | 101% | 98% | 0.00% | 0.08% | -0.15% | 100% | 101% |
| Class E | -0.09% | 0.11% | -0.10% | 99% | 98% |  |  |  |  |  |
| Overall | -0.13% | -0.05% | -0.09% | 100% | 99% |  |  |  |  |  |
| Class D | -0.09% | -0.03% | -0.16% | 100% | 101% | 0.04% | -0.23% | -0.30% | 100% | 103% |
| Class F | 0.11% | 0.05% | 0.13% | 101% | 101% |  |  |  |  |  |

Table 3: Results of combined test 2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.31% | -0.34% | 100% | 103% |  |  |  |  |  |
| Class A2 | -0.14% | 0.01% | -0.18% | 98% | 98% |  |  |  |  |  |
| Class B | -0.17% | -0.14% | -0.11% | 102% | 102% |  |  |  |  |  |
| Class C | -0.05% | -0.11% | -0.04% | 99% | 100% | 0.03% | -0.01% | -0.06% | 101% | 101% |
| Class E | -0.16% | -0.08% | -0.08% | 99% | 98% |  |  |  |  |  |
| Overall | -0.18% | -0.13% | -0.14% | 100% | 101% |  |  |  |  |  |
| Class D | -0.09% | -0.03% | -0.19% | 100% | 100% | 0.00% | -0.22% | -0.12% | 100% | 101% |
| Class F | 0.10% | -0.06% | 0.01% | 100% | 100% |  |  |  |  |  |

Table 4: Results of combined test 3 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.33% | -0.33% | 99% | 101% |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.14% | -0.07% | -0.05% | 103% | 102% | -0.06% | 0.00% | -0.24% | 100% | 100% |
| Class C | -0.04% | -0.07% | -0.01% | 100% | 98% | 0.02% | 0.11% | -0.02% | 101% | 102% |
| Class E | -0.12% | 0.06% | 0.05% | 100% | 101% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.10% | 0.01% | -0.09% | 102% | 100% | 0.02% | -0.28% | -0.25% | 101% | 102% |
| Class F | 0.07% | 0.17% | 0.07% | 104% | 104% | 0.02% | 0.02% | 0.00% | 101% | 101% |

Table 5: Results of combined test 4 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.41% | -0.35% | 99% | 102% |  |  |  |  |  |
| Class A2 | -0.15% | -0.03% | -0.11% | 99% | 98% |  |  |  |  |  |
| Class B | -0.18% | -0.08% | -0.11% | 101% | 100% | -0.06% | -0.10% | -0.22% | 100% | 99% |
| Class C | -0.06% | -0.09% | -0.02% | 100% | 95% | -0.03% | -0.03% | -0.18% | 101% | 100% |
| Class E | -0.18% | -0.08% | -0.07% | 100% | 98% |  |  |  |  |  |
| Overall | -0.19% | -0.13% | -0.12% | 100% | 99% |  |  |  |  |  |
| Class D | -0.09% | -0.09% | -0.18% | 103% | 100% | -0.01% | -0.12% | 0.11% | 100% | 100% |
| Class F | 0.07% | -0.07% | -0.06% | 101% | 99% | -0.01% | -0.02% | 0.00% | 100% | 99% |

Table 6: Results of combined test 5 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.47% | -0.35% | 95% | 101% |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.18% | -0.17% | -0.05% | 97% | 101% |  |  |  |  |  |
| Class C | -0.06% | -0.08% | -0.04% | 97% | 101% | -0.01% | -0.08% | -0.14% | 101% | 103% |
| Class E | -0.15% | 0.00% | -0.02% | 98% | 102% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.07% | -0.04% | -0.13% | 99% | 103% | 0.02% | -0.35% | -0.08% | 101% | 103% |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 7: Results of combined test 6 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.23% | -0.16% | -0.13% | 96% | 102% | -0.09% | -0.02% | -0.24% | 100% | 99% |
| Class C | -0.12% | -0.10% | -0.04% | 96% | 100% | -0.03% | 0.05% | -0.31% | 100% | 101% |
| Class E | -0.23% | -0.04% | -0.06% | 96% | 100% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.14% | -0.03% | -0.20% | 97% | 104% | -0.04% | -0.39% | -0.12% | 100% | 103% |
| Class F | -0.02% | -0.05% | 0.11% | 98% | 100% | -0.03% | -0.13% | 0.04% | 101% | 102% |

Table 8: Results of combined test 7 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.42% | -0.35% | 96% | 102% |  |  |  |  |  |
| Class A2 | -0.21% | -0.07% | -0.15% | 96% | 99% |  |  |  |  |  |
| Class B | -0.24% | -0.13% | -0.15% | 96% | 101% | -0.08% | 0.14% | -0.29% | 100% | 100% |
| Class C | -0.15% | -0.07% | -0.07% | 95% | 99% | 0.00% | -0.06% | -0.16% | 100% | 103% |
| Class E | -0.23% | -0.10% | -0.20% | 96% | 99% |  |  |  |  |  |
| Overall | -0.25% | -0.15% | -0.17% | 96% | 100% |  |  |  |  |  |
| Class D | -0.14% | -0.17% | -0.09% | 97% | 99% | -0.03% | -0.16% | 0.01% | 100% | 103% |
| Class F | -0.03% | -0.05% | -0.10% | 99% | 101% | -0.03% | -0.11% | -0.02% | 101% | 102% |

Conclusion

This contribution reports the combined CE tests between CE3-3.3 and CE3-3.5.1 on MPM list harmonization. The proposed methods harmonize and simplify the MPM list generation process with no encoding and decoding time increase, and combined test 7 achieves 0.25% coding gain in AI configuration on top of VTM-4.0 with 96% encoding time. It is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, K. Misra “CE3-3.4-related: unified MPM list construction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0450, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).