| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0394-r2 |
| --- | --- |

| Title: | CE3-related: Unified MPM list based on CE3-3.3 and CE3-3.5.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this proposal, a unified MPM list derivation is proposed, which is combination of CE3-3.3.2/CE3-3.3.3 and CE3-3.5.1. With the proposed method, the angular MPMs used in CE3-3.3.2/CE3-3.3.3 are replaced by first 5 or 6 angular MPMs derived in CE3-3.5.1. Furthermore, the modification of MPM list construction together with the encoder optimization employed in proposal JVET-N0450 are also combined in this proposal. It is reported that the combined test 7 obtains 0.25% coding gain for the AI configuration on top of VTM-4.0 with around 96% encoding time and 100% decoding time.

Introduction

In VTM-4.0, three different MPM list generation processes are employed for zero reference line with ISP off, ISP on, and non-zero reference lines, respectively. For zero reference line with ISP off, Planar and DC modes are always included in the MPM list, and 4 angular MPM candidates are derived. For ISP on, Planar mode is always included from the MPM list, DC mode is always excluded from the MPM list, and 5 angular MPM candidates are derived. For non-zero reference lines, Planar and DC modes are always excluded from the MPM list, and 6 angular MPM candidates are derived.

Harmonization of MPM list has been studied in CE3-3. For CE3-3.3 and CE3-3.5, only one MPM list is generated. Specifically, in CE3-3.3.2/3, Planar and DC modes are always the first two MPMs and 4 angular MPMs are derived using the zero-reference line MPM list generation process. In CE3-3.5.1, Planar and DC modes are coded separately, and 5/6 angular MPMs are derived by using the non-zero reference lines MPM list generation process.

Proposed methods

In this proposal, only one unified MPM list is generated, wherein Planar and DC modes are always first two MPMs in this unified MPM list. All the MPM candidates in this unified MPM list are used for zero reference line with ISP off, MPM candidates excluding DC mode in this unified MPM list are used for ISP on, and only the angular MPM candidates in this unified MPM list are used for non-zero reference lines.

5 or 6 angular MPM candidates are derived by the method in CE3-3.5.1, and the number of the unified MPM list is 7 or 8. For combined test 1 and combined test 3, there is one context for each of the first two bins of MPM index, which is the same as that in CE3-3.3.2. For combined test 2 and test 4, there are 3 and 1 contexts for the first two bins of MPM index respectively, which is the same as that in CE3-3.3.3.

To be specific, the number of angular MPMs and the number of contexts used for different combined tests are compared in Table 1.

Table I: Comparisons between different combined tests

| Test | Number of angular MPMs | Number of Contexts for MPM index |
| --- | --- | --- |
| Combined Test 1 | 5 angular modes | 2 |
| Combined Test 2 | 5 angular modes | 4 |
| Combined Test 3 | 6 angular modes | 2 |
| Combined Test 4 | 6 angular modes | 4 |

Furthermore, on top of the combined test 4, four features from JVET-N0450 are also combined, which are listed below.

- The lossless encoder speedup for the intra mode search, which simplifies the intra mode bits calculation process. (namely feature #1)

- The encoder search in section 2.1 of JVET-N0450 is applied. (namely feature #2)

- The lambda value for the initial mode search is doubled (namely feature #3)

- Only the first entry in the MPM list may be equal to VDIA_IDX. (namely feature #4)

Experimental results

The tests are implemented on top of VTM4.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-M1010 [2]. 7 combined tests are conducted to verify the performance of the proposed methods. The description of combined test 1 ~4 are illustrated in Table 1. And combined test 5 ~ 7 is based on combined test 4, and the details are listed below:

- Combined test 5: Combined test 4 + feature #1+ feature #2

- Combined test 6: Combined test 4 + feature #1 + feature #2 + feature #3

- Combined test 7: Combined test 4 + feature #1 + feature #2 + feature #3 + feature #4

Table 2: Results of combined test 1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.25% | -0.30% | 98% | 100% | -0.11% | -0.08% | -0.20% | 100% | 100% |
| Class A2 | -0.10% | 0.00% | -0.10% | 98% | 97% | -0.03% | 0.06% | -0.04% | 100% | 100% |
| Class B | -0.13% | -0.06% | -0.03% | 101% | 102% | -0.05% | -0.09% | 0.07% | 100% | 100% |
| Class C | -0.01% | -0.03% | 0.01% | 101% | 98% | 0.00% | 0.08% | -0.15% | 100% | 101% |
| Class E | -0.09% | 0.11% | -0.10% | 99% | 98% |  |  |  |  |  |
| Overall | -0.13% | -0.05% | -0.09% | 100% | 99% | -0.04% | -0.01% | -0.06% | 100% | 100% |
| Class D | -0.09% | -0.03% | -0.16% | 100% | 101% | 0.04% | -0.23% | -0.30% | 100% | 101% |
| Class F | 0.11% | 0.05% | 0.13% | 101% | 101% | 0.03% | -0.06% | -0.03% | 101% | 101% |

Table 3: Results of combined test 2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.31% | -0.34% | 100% | 103% | -0.12% | -0.04% | -0.15% | 100% | 100% |
| Class A2 | -0.14% | 0.01% | -0.18% | 98% | 98% | -0.03% | 0.24% | 0.09% | 100% | 99% |
| Class B | -0.17% | -0.14% | -0.11% | 102% | 102% | -0.05% | 0.03% | -0.24% | 101% | 100% |
| Class C | -0.05% | -0.11% | -0.04% | 99% | 100% | 0.03% | -0.01% | -0.06% | 101% | 101% |
| Class E | -0.16% | -0.08% | -0.08% | 99% | 98% |  |  |  |  |  |
| Overall | -0.18% | -0.13% | -0.14% | 100% | 101% | -0.04% | 0.05% | -0.11% | 100% | 100% |
| Class D | -0.09% | -0.03% | -0.19% | 100% | 100% | 0.00% | -0.22% | -0.12% | 100% | 101% |
| Class F | 0.10% | -0.06% | 0.01% | 100% | 100% | -0.01% | -0.02% | -0.14% | 100% | 101% |

Table 4: Results of combined test 3 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | -0.33% | -0.33% | 99% | 101% | -0.13% | -0.22% | -0.16% | 100% | 100% |
| Class A2 | -0.10% | -0.02% | -0.09% | 101% | 99% | -0.03% | 0.16% | 0.12% | 102% | 100% |
| Class B | -0.14% | -0.07% | -0.05% | 103% | 102% | -0.06% | 0.00% | -0.24% | 100% | 100% |
| Class C | -0.04% | -0.07% | -0.01% | 100% | 98% | 0.02% | 0.11% | -0.02% | 101% | 102% |
| Class E | -0.12% | 0.06% | 0.05% | 100% | 101% |  |  |  |  |  |
| Overall | -0.15% | -0.08% | -0.08% | 101% | 100% | -0.04% | 0.02% | -0.09% | 101% | 101% |
| Class D | -0.10% | 0.01% | -0.09% | 102% | 100% | 0.02% | -0.28% | -0.25% | 101% | 102% |
| Class F | 0.07% | 0.17% | 0.07% | 104% | 104% | 0.02% | 0.02% | 0.00% | 101% | 101% |

Table 5: Results of combined test 4 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.41% | -0.35% | 99% | 102% | -0.14% | -0.38% | -0.10% | 99% | 100% |
| Class A2 | -0.15% | -0.03% | -0.11% | 99% | 98% | -0.06% | 0.02% | -0.01% | 100% | 99% |
| Class B | -0.18% | -0.08% | -0.11% | 101% | 100% | -0.06% | -0.10% | -0.22% | 100% | 99% |
| Class C | -0.06% | -0.09% | -0.02% | 100% | 95% | -0.03% | -0.03% | -0.18% | 101% | 100% |
| Class E | -0.18% | -0.08% | -0.07% | 100% | 98% |  |  |  |  |  |
| Overall | -0.19% | -0.13% | -0.12% | 100% | 99% | -0.07% | -0.11% | -0.15% | 100% | 100% |
| Class D | -0.09% | -0.09% | -0.18% | 103% | 100% | -0.01% | -0.12% | 0.11% | 100% | 100% |
| Class F | 0.07% | -0.07% | -0.06% | 101% | 99% | -0.01% | -0.02% | 0.00% | 100% | 99% |

Table 6: Results of combined test 5 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.47% | -0.35% | 95% | 101% | -0.15% | -0.22% | -0.25% | 99% | 99% |
| Class A2 | -0.14% | -0.01% | -0.14% | 95% | 97% | -0.07% | 0.07% | 0.04% | 99% | 99% |
| Class B | -0.18% | -0.17% | -0.05% | 97% | 101% | -0.04% | -0.05% | -0.06% | 100% | 99% |
| Class C | -0.06% | -0.08% | -0.04% | 97% | 101% | -0.01% | -0.08% | -0.14% | 101% | 103% |
| Class E | -0.15% | 0.00% | -0.02% | 98% | 102% |  |  |  |  |  |
| Overall | -0.18% | -0.15% | -0.11% | 97% | 100% | -0.06% | -0.07% | -0.10% | 100% | 100% |
| Class D | -0.07% | -0.04% | -0.13% | 99% | 103% | 0.02% | -0.35% | -0.08% | 101% | 103% |
| Class F | 0.04% | 0.05% | -0.10% | 98% | 100% | -0.03% | 0.03% | -0.09% | 101% | 101% |

Table 7: Results of combined test 6 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.48% | -0.42% | -0.33% | 94% | 102% | -0.12% | -0.26% | -0.24% | 99% | 98% |
| Class A2 | -0.19% | 0.00% | -0.16% | 95% | 96% | -0.04% | 0.21% | 0.01% | 99% | 99% |
| Class B | -0.23% | -0.16% | -0.13% | 96% | 102% | -0.09% | -0.02% | -0.24% | 100% | 99% |
| Class C | -0.12% | -0.10% | -0.04% | 96% | 100% | -0.03% | 0.05% | -0.31% | 100% | 101% |
| Class E | -0.23% | -0.04% | -0.06% | 96% | 100% |  |  |  |  |  |
| Overall | -0.24% | -0.14% | -0.14% | 96% | 100% | -0.07% | 0.00% | -0.21% | 99% | 100% |
| Class D | -0.14% | -0.03% | -0.20% | 97% | 104% | -0.04% | -0.39% | -0.12% | 100% | 103% |
| Class F | -0.02% | -0.05% | 0.11% | 98% | 100% | -0.03% | -0.13% | 0.04% | 101% | 102% |

Table 8: Results of combined test 7 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.42% | -0.35% | 96% | 102% | -0.12% | -0.17% | -0.14% | 99% | 99% |
| Class A2 | -0.21% | -0.07% | -0.15% | 96% | 99% | -0.09% | -0.08% | 0.06% | 100% | 98% |
| Class B | -0.24% | -0.13% | -0.15% | 96% | 101% | -0.08% | 0.14% | -0.29% | 100% | 100% |
| Class C | -0.15% | -0.07% | -0.07% | 95% | 99% | 0.00% | -0.06% | -0.16% | 100% | 103% |
| Class E | -0.23% | -0.10% | -0.20% | 96% | 99% |  |  |  |  |  |
| Overall | -0.25% | -0.15% | -0.17% | 96% | 100% | -0.07% | -0.02% | -0.16% | 100% | 100% |
| Class D | -0.14% | -0.17% | -0.09% | 97% | 99% | -0.03% | -0.16% | 0.01% | 100% | 103% |
| Class F | -0.03% | -0.05% | -0.10% | 99% | 101% | -0.03% | -0.11% | -0.02% | 101% | 102% |

Spec text changes

The following describes spec text modification of combined test 7. It is based on VVC draft5 v7 [3]. The added texts are highlighted with in green, and the deleted texts are marked with strikethrough.

8.4.2 Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The variables ispDefaultMode1 and ispDefaultMode2 are defined as follows:

- If IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, ispDefaultMode1 is set equal to INTRA_ANGULAR18 and ispDefaultMode2 is set equal to INTRA_ANGULAR5.

- Otherwise, ispDefaultMode1 is set equal to INTRA_ANGULAR50 and ispDefaultMode2 is set equal to INTRA_ANGULAR63.

- The candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-9)

candModeList[ 1 ] = INTRA_DC		(8-10)

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-9)

candModeList[ 1 ] = INTRA_PLANAR	(8-10)

candModeList[ 2 ] = INTRA_DC		(8-11)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-12)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-13)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-14)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 02 ] = candIntraPredModeA	(8-15)

candModeList[ 13 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-16)

candModeList[ 24 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-17)

  - If one of the following conditions is true,

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and candIntraPredModeA  is less than INTRA_ANGULAR34,

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and candIntraPredModeA  is greater than or equal to INTRA_ANGULAR34,

  - IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0,

the following applies:

candModeList[ 35 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-18)

candModeList[ 46 ] = 2 + ( candIntraPredModeA % 64 )	(8-19)

candModeList[ 57 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-20)

  - Otherwise, the following applies:

candModeList[ 3 ] = ispDefaultMode1	(8-21)

candModeList[ 4 ] = ispDefaultMode2	(8-22)

candModeList[ 5 ] = INTRA_PLANAR	(8-23)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-24)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-25)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 02 ] = candIntraPredModeAmaxAB	(8-26)

candModeList[ 13 ] = candIntraPredModeBminAB	(8-27)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-28)

candModeList[ 3 ] = INTRA_DC		(8-29)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-30)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-31)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-33)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0 or IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

  - When IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, and abs( candIntraPredModeB − ispDefaultMode1 ) is less than abs( candIntraPredModeA − ispDefaultMode1 ), the following applies:

candModeList[ 0 ] = candIntraPredModeB	(8-34)

candModeList[ 1 ] = candIntraPredModeA	(8-35)

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 24 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-36)

candModeList[ 35 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-37)

candModeList[ 46 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-38)

candModeList[ 57 ] = 2 + ( maxAB % 64 )	(8-39)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 24 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-40)

candModeList[ 35 ] = 2 + ( ( minAB + 61 maxAB − 1) % 64 )	(8-41)

candModeList[ 46 ] = 2 + ( ( maxAB − 1 minAB + 60 ) % 64 )	(8-42)

candModeList[ 57 ] = 2 + ( ( minAB + 60 maxAB ) % 64 )	(8-43)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 24 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-44)

candModeList[ 35 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 46 ] = 2 + ( minAB % 64 )	(8-46)

candModeList[ 57 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise, the following applies:

candModeList[ 24 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-48)

candModeList[ 35 ] = 2 + ( ( minAB − 1 maxAB + 61  ) % 64 )	(8-49)

candModeList[ 46 ] = 2 + ( ( maxAB + 61 minAB − 1) % 64 )	(8-50)

candModeList[ 57 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-51)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-52)

candModeList[ 1 ] = candIntraPredModeB	(8-53)

candModeList[ 2 ] = 1 − minAB		(8-54)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-55)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-56)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-57)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 02 ] = maxAB		(8-58)

candModeList[ 13 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-59)

candModeList[ 24 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-60)

candModeList[ 35 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-61)

candModeList[ 46 ] = 2 + ( maxAB % 64 )	(8-62)

candModeList[ 57 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-63)

  - Otherwise (IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT), the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-64)

candModeList[ 1 ] = maxAB		(8-65)

candModeList[ 2 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-66)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-67)

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-68)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-69)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0 and IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-70)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-71) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-72)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-73)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-74)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-75)

  - Otherwise, if IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, the following applies:

candModeList[ 02 ] = INTRA_ANGULAR50	(8-76)

candModeList[ 13 ] = INTRA_ANGULAR18	(8-77)

candModeList[ 24 ] = INTRA_ANGULAR2	(8-78)

candModeList[ 35 ] = INTRA_ANGULAR34	(8-79)

candModeList[ 46 ] = INTRA_ANGULAR66	(8-80)

candModeList[ 57 ] = INTRA_ANGULAR26	(8-81)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-82)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-83)

candModeList[ 2 ] = INTRA_ANGULAR25	(8-84)

candModeList[ 3 ] = INTRA_ANGULAR10	(8-85)

candModeList[ 4 ] = INTRA_ANGULAR65	(8-86)

candModeList[ 5 ] = INTRA_ANGULAR50	(8-87)

  - Otherwise, if IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT, the following applies:

candModeList[ 0 ] = INTRA_PLANAR	(8-88)

candModeList[ 1 ] = INTRA_ANGULAR50	(8-89)

candModeList[ 2 ] = INTRA_ANGULAR43	(8-90)

candModeList[ 3 ] = INTRA_ANGULAR60	(8-91)

candModeList[ 4 ] = INTRA_ANGULAR3	(8-92)

candModeList[ 5 ] = INTRA_ANGULAR18	(8-93)

- If IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0, for i = 0..5, the following applies:

candModeList[ i] = candModeList[i+2] 	(8-94)

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT, for i = 1..6, the following applies:

candModeList[ i] = candModeList[i + 1] 	(8-95)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 02..46 and for each i, j = ( i + 1 )..57, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 57, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

| cu_skip_flag[ ][ ] | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |
| pred_mode_flag | 0,1 (clause 9.5.4.2.2) | na | na | na | na | na |
| pred_mode_ibc_flag | 0,1,2 (clause 9.5.4.2.2) | na | na | na | na | na |
| pcm_flag[ ][ ] | terminate | na | na | na | na | na |
| intra_luma_ref_idx[ ][ ] | 0 | 1 | na | na | na | na |
| intra_subpartitions_mode_flag | 0 | na | na | na | na | na |
| intra_subpartition_split_flag | 0 | na | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] | 0 | na | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] | Bypass0,1,2 (clause a.b.c.d.e) | Bypass3 | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |

a.b.c.d.e Derivation process of ctxInc using left-bottom and above-right syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, and the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 − 1, y0 + cbHeight − 1 ) as inputs, and the output is assigned to availableLB.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to availableAR.

The same CTU checking process for a block as specified in clause 6.Y.X [Same CTU checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to sameCtuAR.

The assignment of ctxInc is specified as follows with condL, condLB, condA and condAR for the syntax elements intra_luma_angular_mode_flag[ x0 ][ y0 ] specified in Table 9-xx:

ctxInc = ( condLB  &&  availableLB ) + ( condAR  &&  availableAR && sameCtuAR )	(9-yy)

Table 9-xx – Specification of ctxInc using left-bottom and above-right syntax elements

| condLB | condAR |
| --- | --- |
| intra_luma_mpm_flag [ xNbLB ][ yNbLB ] is false or ( intra_luma_mpm_flag [ xNbLB ][ yNbLB ] is true && intra_luma_mpm_idx [ xNbLB ][ yNbLB ] is greater than 0) | intra_luma_mpm_flag [ xNbAR ][ yNbAR ] is equal to false or ( intra_luma_mpm_flag [ xNbAR ][ yNbAR ] is equal to true && intra_luma_mpm_idx [ xNbAR ][ yNbAR ] is greater than 0) |

Conclusion

This contribution reports the combined CE tests between CE3-3.3 and CE3-3.5.1 on MPM list harmonization. The proposed methods harmonize and simplify the MPM list generation process, and combined test 7 achieves 0.25% coding gain in AI configuration on top of VTM-4.0 with 96% encoding time. It is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, K. Misra “CE3-3.4-related: unified MPM list construction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0450, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).