| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0396-v1 |
| --- | --- |

| Title: | CE3-related: Enabling parallel reconstruction of small intra-coded chroma blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luong Pham Van Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Han Huang Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 | Email: | lphamvan@qti.qualcomm.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a method allowing multiple chroma intra coded blocks within a parallel-processable region (PPR) of size larger than or equal to 16 chroma samples to be reconstructed in parallel. In the dual-tree case, the block sizes of 2x2 and 2x4/4x2 are disabled. In the single-tree case, the intra-prediction dependency between the blocks in a 16-sample PPR is eliminated by marking all the blocks within the PPR as “unavailable” for predicting other blocks contained in this PPR. The VVC reference sample substitution process automatically pads the unavailable reference samples for prediction. The average (Y, U, V) BD-rate losses are reportedly (0.02%, 0.37%, 0.49%), (0.07%, 0.xx%, 0.xx%) and (0.01%, 0.xx%, 0.xx%) for the AI, RA, and LDB configurations, respectively. In addition, the worst-case processing throughput for chroma intra prediction is increased 4x compared to that in VTM4.0.

# Introduction

In VTM4.0, it is possible to have small chroma blocks of size smaller than 16 samples including 2x2, 2x4 and 4x2. In a typical hardware video codec, the processing throughput is reduced when a picture is coded using many small blocks. This reduction mainly comes from small intra blocks, because intra blocks have data dependency between neighbouring blocks (e.g., the prediction of a block uses top and left boundary reconstructed samples from neighbouring blocks as reference); hence the reconstruction must be performed sequentially.

The worst-case processing throughput of HEVC occurs when 4x4 chroma intra blocks are processed. In VTM4.0, the size of the smallest chroma intra block is 2x2, and the reconstruction process of a chroma intra block becomes very complex after adopting several tools. It is asserted that the worst-case processing throughput of VTM4.0 is lower than that of HEVC by 4 times.

# Proposed Method

In this proposal, two techniques are proposed to increase the worst-case processing throughput of VTM4.0.

## Small size restriction in dual-tree:

In the first solution, small block sizes including 2x2, 2x4, and 4x2 are restricted in dual-tree. This restriction is performed by prohibiting the following splits: splitting is not allowed for 4x4, 2x8, and 8x2 blocks while ternary splits are not allowed for 8x4 and 4x8 blocks.

## Removal of intra-prediction dependency in single tree:

This solution enables parallel generation of multiple chroma intra coding blocks within a parallel-processable region (PPR) in the single tree case. All the data dependencies between the chroma blocks in the same PPR are eliminated, thus they can be reconstructed in parallel. To achieve this, parallel-processable regions are first defined.

Definition of PPR:

When a chroma block of size larger than or equal to threshold chroma samples has any children blocks of size less than threshold or is not further split, the chroma block of size larger than or equal to threshold is treated as a parallel-processable region. In this proposal, threshold is set equal to 16 to achieve a worst-case processing throughput of 4x that of VTM4.0. This definition can be also found in JVET-M0169. The shape of some example of PPR is depicted in Figure 1.

Figure 1. PPR example with the minimum number of chroma samples of 16

Dependency removal:

The intra-prediction dependency among the blocks in a 16-sample PPR is eliminated by marking all the blocks within the PPR as “unavailable” for predicting all contained blocks in this PPR. It should be noted that, the substitution of reference samples is automatically performed by the reference substitution process in VVC when a neighbour block is unavailable. In addition, this proposed method is simple, because it does not need any change in other parts of intra prediction.

# Experimental Results

The proposed technique has been implemented on top of VTM4.0. The experimental results under the AI, RA, and LDB configuration are summarized in Table 1.

With an increase of the worst-case processing throughput by 4x, the proposed technique results in average (Y, U, V) BD-rates of (0.02%, 0.37%, 0.49%), (0.07%, 0.xx%, 0.xx%) and (0.01%, 0.xx%, 0.xx%) for the AI, RA, and LDB configurations, respectively.

Table 1 Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.16% | 0.11% |  |  |
| Class A2 | 0.04% | 0.39% | 0.23% |  |  |
| Class B | 0.00% | 0.18% | 0.29% |  |  |
| Class C | 0.05% | 1.06% | 1.56% |  |  |
| Class E | -0.02% | -0.05% | 0.05% |  |  |
| Overall | 0.04% | 0.39% | 0.23% |  |  |
| Class D | 0.12% | 1.80% | 2.48% |  |  |
| Class F | 0.19% | 1.56% | 1.96% |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 | 0.03% | 0.31% | 0.19% |  |  |
| Class B | 0.04% | 0.24% | 0.29% |  |  |
| Class C | 0.16% | 0.90% | 1.23% |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.20% | 1.43% | 1.83% |  |  |
| Class F | 0.21% | 1.10% | 1.42% |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.02% | 0.36% | 0.42% |  |  |
| Class E | 0.03% | 0.93% | 0.46% |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.05% | 0.88% | -0.06% |  |  |
| Class F |  |  |  |  |  |

# Conclusion

This contribution proposed a method allowing multiple chroma intra coding blocks within a parallel-processable region of size larger than or equal to 16 chroma samples to be reconstructed in parallel. The block sizes of 2x2 and 2x4/4x2 are prohibited in the dual-tree case. In single-tree, the intra-prediction dependency among the blocks in a 16-sample PPR is eliminated. With an increase of the worst-case processing throughput by 4x, the average (Y, U, V) BD-rate losses is reported to be (0.02%, 0.37%, 0.49%), (0.07%, 0.xx%, 0.xx%) and (0.01%, 0.xx%, 0.xx%) for the AI, RA, and LDB configurations, respectively.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).