| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0397 |
| --- | --- |

| Title: | AHG16: Memory bandwidth reduction for small CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Guichun Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xlxiangli, guichunli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

To reduce worst-case memory bandwidth for small CUs, it is proposed to use size dependent interpolation filter in motion compensation. A shorter filter (4-tap chroma interpolation filter or 2-tap bilinear filter) is used in luma motion compensation for the direction in which the length of a block is 4 luma pixel. Compared to VTM-4.0, 0.05% RA loss and 0.08% LDB loss are reported when 4-tap filter is used; while 0.13% RA loss and 0.24% LDB loss are reported when 2-tap filter is used.

# Introduction

In current VVC, bi-predicted 4x16, 16x4, 4x8 and 8x4 coding units (CUs) and uni-predicted 4x4 CUs require higher memory bandwidth than 8x8 bi-predicted CUs. The memory bandwidth required by these CUs are summarized in Table 1 for YCbCr 4:2:0 case, wherein both luma and chroma components are considered.

Table 1: bandwidth comparison of different CUs

| CU | Bandwidth per luma pixel | Bandwidth / bandwidth of 8x8 bi-predicted CUs |
| --- | --- | --- |
| 8x8 Bi | ((8 + 7) × (8 + 7) + (4 + 3) × (4 + 3) × 2) × 2 / 64 = 10.09375 | 100% |
| 4x4 Uni | ((4 + 7) × (4 + 7) + (2 + 3) × (2 + 3) × 2) / 16 = 10.6875 | 106% |
| 4x8/8x4 Bi | ((4 + 7) × (8 + 7) + (2 + 3) × (4 + 3) × 2) × 2 / 32 = 14.6875 | 146% |
| 4x16/16x4 Bi | ((4 + 7) × (16 + 7) + (2 + 3) × (8 + 3) × 2) × 2 / 64 = 11.34375 | 112% |

# Proposed methods

To save both worst-case memory bandwidth and interpolation complexity while keeping coding performance, it is proposed to use shorter filter for small CUs. To avoid introducing additional filters and complex operations, existing filters and logics in VTM-4 are reused. Basically, an existing shorter interpolation filter (4-tap chroma interpolation filter, or 2-tap bilinear interpolation filter) is used for the direction in which block length is 4. For example, for an 8x4 block current 8-tap interpolation filter is used for horizontal direction, but one of the proposed shorter filters is used for vertical direction.

With the proposed method, the bandwidth assessment table is updated as below. It can be observed that the worst-case memory bandwidth is reduced significantly, i.e., from 146% of 8x8 Bi to 108% of 8x8 Bi

Table 2: bandwidth comparison between proposed methods and VTM-4.0

| CU | VTM-4 | Proposed 4-tap | Proposed 2-tap |
| --- | --- | --- | --- |
| 8x8 Bi | 10.09375 | 10.09375 | 10.09375 |
| 4x4 Uni | 10.6875 | ((4 + 3) × (4 + 3) + (2 + 3) × (2 + 3) × 2) / 16 = 6.1875 | ((4 + 1) × (4 + 1) + (2 + 3) × (2 + 3) × 2) / 16 = 4.6875 |
| 4x8/8x4 Bi | 14.6875 | ((4 + 3) × (8 + 7) + (2 + 3) × (4 + 3) × 2) × 2 / 32 = 10.9375 | ((4 + 1) × (8 + 7) + (2 + 3) × (4 + 3) × 2) × 2 / 32 = 9.0625 |
| 4x16/16x4 Bi | 11.34375 | ((4 + 3) × (16 + 7) + (2 + 3) × (8 + 3) × 2) × 2 / 64 = 8.46875 | ((4 + 1) × (16 + 7) + (2 + 3) × (8 + 3) × 2) × 2 / 64 = 7.03125 |

# Simulation results

The proposed methods are tested on top of VTM-4.0 under common test conditions defined in JVET-M1010. The results are summarized as follows.

## 4-tap filter

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.12% | 0.00% | 103% | 103% |
| Class A2 | -0.02% | -0.09% | -0.01% | 103% | 103% |
| Class B | 0.02% | -0.18% | -0.11% | 102% | 104% |
| Class C | 0.19% | 0.03% | -0.05% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.10% | -0.05% | 102% | 103% |
| Class D | 0.37% | 0.10% | 0.13% | 100% | 100% |
| Class F | 0.00% | -0.01% | -0.04% | 101% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.06% | -0.18% | 102% | 103% |
| Class C | 0.18% | 0.53% | 0.33% | 103% | 108% |
| Class E | 0.02% | 0.29% | 0.02% | 102% | 106% |
| Overall | 0.08% | 0.22% | 0.04% | 102% | 106% |
| Class D | 0.58% | 1.20% | 1.18% | 103% | 102% |
| Class F | -0.15% | 0.10% | 0.06% | 103% | 107% |
|  |  |  | Low Delay P Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.56% | -0.86% | 101% | 103% |
| Class C | 0.14% | -0.14% | 0.13% | 102% | 105% |
| Class E | -0.12% | -0.22% | 0.27% | 100% | 102% |
| Overall | -0.01% | -0.33% | -0.25% | 101% | 103% |
| Class D | 0.53% | 0.43% | 1.19% | 100% | 99% |
| Class F | -0.06% | -0.27% | -0.40% | 106% | 108% |

## 2-tap filter

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | -0.13% | 103% | 103% |
| Class A2 | 0.01% | -0.38% | -0.15% | 102% | 102% |
| Class B | 0.09% | -0.18% | -0.06% | 102% | 104% |
| Class C | 0.36% | 0.21% | 0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | -0.11% | -0.04% | 102% | 103% |
| Class D | 0.69% | 0.35% | 0.44% | 100% | 99% |
| Class F | -0.03% | -0.05% | -0.14% | 101% | 101% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.07% | 0.23% | 102% | 104% |
| Class C | 0.43% | 0.92% | 1.06% | 104% | 107% |
| Class E | 0.09% | 0.38% | 0.00% | 104% | 109% |
| Overall | 0.24% | 0.43% | 0.45% | 103% | 106% |
| Class D | 1.04% | 1.62% | 1.30% | 103% | 101% |
| Class F | -0.12% | -0.55% | -0.35% | 103% | 106% |
|  |  |  | Low Delay P Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.37% | -0.22% | 102% | 104% |
| Class C | 0.23% | 0.34% | 0.46% | 104% | 106% |
| Class E | -0.37% | -0.03% | -0.46% | 100% | 104% |
| Overall | -0.02% | -0.05% | -0.05% | 102% | 105% |
| Class D | 1.03% | 1.99% | 2.00% | 101% | 98% |
| Class F | -0.27% | -0.14% | -0.04% | 103% | 105% |

# Conclusions

In this proposal, CU size dependent interpolation filter is proposed to reduce both worst-case memory bandwidth and interpolation complexity for small CUs. When width/height of a CU is 4, shorter interpolation filter is used for the direction of length 4. Analysis and simulations show that significant worst-case memory bandwidth reduction is achieved at the cost of minor average coding loss.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).