| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0398 |
| --- | --- |

| Title: | CE2-4.4: Affine block memory bandwidth reduction by MV clip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Guichun Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xlxiangli, guichunli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of CE2-4.4. To reduce the memory bandwidth in affine prediction, the MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. It is reported that the worst-case memory bandwidth requirement by affine prediction is reduced from 15.125 luma sample per pixel to 8 luma sample per pixel at the cost of minor BD-rate increase (0.01% and -0.02% for RA and LDB configurations, respectively).

# Introduction

In affine prediction, the uni-directional inter prediction for 4x4 block needs to load up to (4+7)×(4+7)=121 luma samples. The per-pixel memory bandwidth is 7.6 luma samples per pixel. For bi-directional affine prediction, the per-pixel memory bandwidth is up to 15.125 luma samples per pixel. To reduce the per-pixel memory bandwidth in affine prediction, the MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel. Consequently, the worst-case memory bandwidth is reduced to (8+7+1) × (8+7+1)/(8x8) ×2=8 luma samples per pixel.

The clipping process applied to each 8x8 block is described as follows:

- The minimal and maximal values for the MV components, MVmin_x, MVmin_y, MVmax_x, MVmax_y are firstly determined for each 8x8 block as follows:

  - Get the minimal MV component among the four 4x4 sub-block MVs

MVmin_x = min(min(MVx0, MVx1), min(MVx2, MVx3))

MVmin_y = min(min(MVy0, MVy1), min(MVy2, MVy3))

  - Use the integer part of MVminx and MVminy as the minimal MV component

MVmin_x = MVmin_x & 0xfffffff0

MVmin_y = MVmin_y & 0xfffffff0

  - The maximal MV component is calculated as follows:

MVmax_x = MVmin_x + (2 << MV_precision) – 1

MVmax_y = MVmin_y + (2 << MV_precision) – 1

- The MV components of each 4x4 block inside this 8x8 block are clipped as follows:

MVxi = min(MVmax_x, MVxi)

MVyi = min(MVmax_y, MVyi)

where (MVxi, MVyi) is the MV of ith sub-block within one 8x8 block, where i is 0, 1, 2, 3; MV_precision is equal to 4 corresponding to 1/16 motion vector fraction accuracy. Since the difference between integer parts of MVmin_x and MVmax_x (MVmin_y and MVmax_y) is 1 pixel, the max difference between integer parts of the four 4x4 sub-block MVs is no more than 1 pixel.

In order to provide more tradeoff points, it is further allowed to have separate constraints along vertical and horizontal directions. In one sub-test, max difference in horizontal direction is constrained as 1 pixel while the max difference in vertical direction is constrained as 0 pixel, namely Tx=1, Ty=0. Accordingly, the worst-case memory bandwidth is further reduced to (8+7+1) × (8+7+0)/(8x8) ×2=7.5 luma sample per pixel.

While keeping the max difference, distance based MVmin and MVmax determination may be used to further improve the coding performance when the max distance is set to 0 [1].

# Simulation results

In this CE, we have three sub-tests as follows

- CE2-4.4.a: Tx=1, Ty=0 (cfg setting: --AffineIntx=1 --AffineInty=1 --DistBasedClip=1)

- CE2-4.4.b: Tx=1, Ty=1 (cfg setting: --AffineIntx=1 --AffineInty=1 --DistBasedClip=1)

- CE2-4.4.c: Tx=1, Ty=1 with distance based MVmin and MVmax determination off (cfg setting: --AffineIntx=1 --AffineInty=1)

## CE2-4.4.a

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.02% | -0.03% | 102% | 100% |
| Class A2 | 0.23% | 0.08% | 0.13% | 101% | 100% |
| Class B | 0.08% | -0.01% | 0.03% | 102% | 100% |
| Class C | 0.03% | -0.08% | -0.10% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | -0.01% | 0.00% | 102% | 100% |
| Class D | 0.02% | -0.09% | 0.03% | 101% | 97% |
| Class F | 0.12% | 0.13% | 0.03% | 101% | 99% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.23% | -0.30% | 102% | 102% |
| Class C | -0.01% | 0.28% | 0.03% | 102% | 98% |
| Class E | -0.04% | 0.40% | -0.21% | 103% | 96% |
| Overall | -0.01% | 0.10% | -0.17% | 103% | 99% |
| Class D | -0.02% | 0.33% | -0.51% | 102% | 98% |
| Class F | -0.02% | -0.20% | 0.28% | 103% | 102% |

## CE2-4.4.b

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.03% | 102% | 100% |
| Class A2 | 0.02% | -0.05% | -0.05% | 101% | 100% |
| Class B | 0.01% | -0.04% | -0.05% | 101% | 99% |
| Class C | 0.00% | 0.00% | -0.10% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.05% | 102% | 99% |
| Class D | 0.01% | -0.15% | -0.13% | 101% | 97% |
| Class F | 0.03% | 0.05% | -0.03% | 102% | 98% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.39% | -0.28% | 103% | 102% |
| Class C | -0.05% | 0.00% | 0.00% | 103% | 99% |
| Class E | -0.01% | 0.07% | -0.66% | 104% | 98% |
| Overall | -0.02% | -0.14% | -0.28% | 103% | 100% |
| Class D | -0.04% | 0.31% | -0.28% | 103% | 98% |
| Class F | -0.07% | -0.04% | 0.16% | 102% | 100% |

## CE2-4.4.c

|  |  |  | Random Access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.03% | 104% | 100% |
| Class A2 | 0.02% | -0.05% | -0.05% | 105% | 101% |
| Class B | 0.01% | -0.04% | -0.05% | 103% | 103% |
| Class C | 0.00% | 0.00% | -0.10% | 103% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.05% | 104% | 102% |
| Class D | 0.01% | -0.15% | -0.13% | 101% | 102% |
| Class F | 0.03% | 0.05% | -0.03% | 103% | 102% |
|  |  |  | Low Delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.39% | -0.28% | 105% | 99% |
| Class C | -0.05% | 0.00% | 0.00% | 105% | 107% |
| Class E | -0.01% | 0.07% | -0.66% | 107% | 105% |
| Overall | -0.02% | -0.14% | -0.28% | 105% | 103% |
| Class D | -0.04% | 0.31% | -0.28% | 103% | 104% |
| Class F | -0.07% | -0.04% | 0.16% | 105% | 105% |

# Conclusions

This contribution reports the CE2-4.4 results on affine block memory bandwidth reduction. With the proposed methods, MVs of four 4x4 sub-blocks within one 8x8 block are constrained such that the max difference between integer parts of the four 4x4 sub-block MVs is no more than a threshold. It is reported that the worst-case memory bandwidth in affine prediction is significantly reduced at the cost of minor BD-rate increase.

# References

- X. Li, M. Gao, S. Liu, “CE2-related: Adaptive sub-block MV clip for affine blocks,” JVET-M0702, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).