| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0399 |
| --- | --- |

| Title: | CE2-related: Simplifications of interweaved affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li, Guichun Li, Xiaozhong Xu, Shan Liu | Tel: Email: | {xlxiangli, guichunli, xiaozhongxu, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Interweaved affine mode was proposed to improve coding efficiency. It converts uni-predicted affine block to bi-predicted block by introducing a uni-predicted auxiliary affine block. The final prediction is obtained by weighted averaging the two uni-predicted blocks. In this contribution, it is proposed to regard interweaved affine mode as a special bi-predicted mode so that weighted bi-prediction process is reused. Furthermore, it is proposed to simplify the weighting process. It is reported that minor impact on coding performance is observed by the proposed methods.

# Introduction

Interweaved affine mode was proposed in JVET-M0268 [1]. First, a coding block is divided into sub-blocks with two different dividing patterns. The first dividing pattern is the same as that in VTM, while the second dividing pattern also divides the coding-block into 4×4 sub-blocks but with a 2×2 offset. Then, two auxiliary predictions are generated by affine motion compensation. The MV for each sub-block in a dividing pattern is derived from CPMVs by the affine model. The final prediction is calculated as a weighted-sum of the two auxiliary predictions, formulated as:

| . |  |
| --- | --- |

The weighting is position based. Basically, an auxiliary prediction sample located at the center of a sub-block is associated with a weighting value 3, while an auxiliary prediction sample located at the boundary of a sub-block is associated with a weighting value 1.

# Proposed method

In the interweaved affine mode, the two predictions are both uni-predicted, and then a weighted averaging process is invoked to generate the final prediction. To keep higher interpolation accuracy and simple control logic, it is proposed to regard interweaved affine mode as a special bi-predicted mode such that the weighted averaging may reuse the current weighted bi-prediction process with interweaved affine weighting table as input.

Moreover, it is proposed to further simplify the weighting process: Always using 2:2 weighting for all pixels except for the shaded region (as shown in Fig. 1) which keeps using 4:0 weighting as only one prediction is available for such a region.

Fig. 1 Interweaved prediction is not applied on the shaded regions.

# Simulation results

The proposed methods are tested on top of CE-4-1-1-2 [2] under common test conditions defined in JVET-M1010 [3]. The results are summarized as follows

## Test 1

In this test, the aspect of reusing weighed averaging of bi-prediction is tested. From the results below, minor impact on coding performance is observed.

## Test 2

The simplification on weighting is tested on top of test 1. Comparing the results of test 1 and test 2, the further simplification of weighting table does not affect the coding performance much.

# Conclusions

In this contribution, simplifications of interweaved affine mode are proposed. It is reported that minor coding impact is observed by the proposed methods.

# References

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong , “Non-CE2: Interweaved Prediction for Affine Motion Compen,” JVET-M0268, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

- K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong, “CE2-1.1: Interweaved Prediction for Affine Motion Compensation,” JVET-N0261, Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).