| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0402-v2 |
| --- | --- |

| Title: | CE12-related: MCTS improvement by modifying motion compensation filter coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou, Alireza Zare, Miska M. Hannuksela | Tel: Email: | firstname.lastname@nokia.com. |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Motion constraint tile set technique, as an encoder side method, restricts selection of motion vectors and merge modes in a way that no sample is used outside the current tile in motion compensation process of each block. According to JVET-M0445, this imposes 10.49% YBD-Rate loss for 49 frames using the 360° CTC sequences with CMP projection format and a 6x4 uniform tile grid, i.e. 2x2 tiles per cube face. Instead of encoder side motion vector constraints of MCTS, this document proposes modifying motion compensation filter coefficients according to the position of the current tile, block, and length of motion vector to improve RD performance of the MCTS-coded video. For each block, based on the number of samples needed from other tiles, some of the coefficients of the motion compensation filter are changed to zero, and their original values are added to the first coefficient next to those coefficients. This removes the effect of those samples in the other tiles in derivation of the samples of the prediction block.

# Introduction and Problem Definition

Motion compensation process for a block may need samples from other tiles. When motion vector has fractional part, an n-tap interpolation filter is used, which increases the chance of using samples from other tiles in motion compensation process. So, to guarantee the MCTS coding, at the encoder side, these motion vectors should be avoided in motion estimation and merge candidate selection. As a result, either a suboptimal motion vector should be selected (which increases residual values), or the block should be split into subblocks to have different motion vectors for different subblocks. Each of the above solution results in degrading the RD performance.

# Proposed method

The proposed method has been shown in figure below in one dimensional case. In this figure, squares (indicated by ri) are samples of the reference frame, and circles (indicated by pi) are the predicted samples for a block with associated motion vector. There are two tiles in this example, and their common boundary is shown in dashed line. This figure shows the case that the motion vectors is fractional. So, each prediction sample is derived using a motion compensation filter (8-tap filter in this figure with filter coefficients from C0 to C7). In this example, the calculation of p0 needs two samples (i.e. r-2 and r-1) from the other tile (i.e., Tile0).

The goal of the proposed method is to avoid using any reference sample from other tiles. In the above example, r-2 and r-1 shall not have effect in the generation of the predicted sample p0. So, the effect of these reference samples is eliminated by changing the corresponding coefficients of MC filter (i.e. C0 and C1 for calculation of p0) to zero. As a result, however these reference samples are accessed during motion compensation, they will not have contribution in derivation of the predicted sample p0. When the corresponding coefficients (i.e. C0 and C1 in this example) are set to zero, their original values are added to other coefficients (i.e. C2 in this example) to keep the sum of coefficients of the new filter the same as the original one. The modified filter is used to derive all the samples in this prediction block (i.e. p1, p2, etc.).

- This modification is applied for horizontal and vertical direction separately, but similarly.

- If the number of coefficients that needs to be set to zero is larger than the length of the motion compensation filter, all the filter coefficients are set to zero.

- If the motion vector of the block is integer and it is pointing outside of the current tile, all the samples of the prediction block are set to zero.

## Detailed description

| Parameter | Description |
| --- | --- |
| NTAP | NTAPS_LUMA, NTAPS_CHROMA or NTAPS_BILINEAR |
| (xInt, yInt) | Integer part of the motion vector of the block |
| (xFrac, yFrac) | Fractional part of the motion vector of the block |
| PULeftX, PUTopY | Top-left coordinate of the current block |
| PURightX, PUBottomY | Bottom-right coordinate of the current block |
| TileLeftX, TileTopY | Top-left coordinate of the current tile |
| TileRightX, TileBottomY | Bottom-right coordinate of the current tile |
| numReplaceFilterLeftHor | Number of coefficients to be set to zero from the left side of motion compensation filter for horizontal direction |
| numReplaceFilterRightHor | Number of coefficients to be set to zero from the right side of motion compensation filter for horizontal direction |
| numReplaceFilterLeftVer | Number of coefficients to be set to zero from the left side of motion compensation filter for vertical direction |
| numReplaceFilterRightVer | Number of coefficients to be set to zero from the right side of motion compensation filter for vertical direction |

if (xFrac)

{

numReplaceFilterLeftHor = (NTAP >> 1) - xInt - 1 - (PULeftX - TileLeftX);

numReplaceFilterRightHor = (NTAP >> 1) + xInt + (PURightX - TileRightX);

}

else

{

numReplaceFilterLeftHor = -xInt - (PULeftX - TileLeftX);

numReplaceFilterRightHor = +xInt + (PURightX - TileRightX);

}

if (yFrac)

{

numReplaceFilterLeftVer = (NTAP >> 1) - yInt - 1 - (PUTopY - TileTopY);

numReplaceFilterRightVer = (NTAP >> 1) + yInt + (PUBottomY - TileBottomY);

}

else

{

numReplaceFilterLeftVer = -yInt - (PUTopY - TileTopY);

numReplaceFilterRightVer = +yInt + (PUBottomY - TileBottomY);

}

numReplaceFilterLeftHor = clip3(0, NTAP, numReplaceFilterLeftHor)

numReplaceFilterLeftVer = clip3(0, NTAP, numReplaceFilterLeftVer)

numReplaceFilterRightHor = clip3(0, NTAP, numReplaceFilterRightHor)

numReplaceFilterRightVer = clip3(0, NTAP, numReplaceFilterRightVer)

## Benefits

Compared to CE12-test1 ‎[1]  (i.e. padding tile group boundary and allow motion vectors to point to samples
outside the boundaries in the reference pictures), the proposed method does not need padding the reference frame in tile group boundaries. Hence, sample and memory access in the reference frame is performed as easily as before.

# Results

According to the CE12 description in JVET-M1032 ‎[1] and the common test conditions in JVET-M0870 ‎[2], the proposed method implemented on top of VTM4.0.1+MCTS. However, all the restrictions in the MCTS implementation are removed from the code. The simulation result for 17-frame random-access has been presented below.

| Reference: | VTM-4.0.1+MCTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tested: | VTM-4.0.1+MCTS+Proposed (MC filter modification) |  |  |  |  |  |
|  | Random Access Main 10 |  |  | Random Access Main 10 |  |  |
|  | 24 Tiles |  |  | 96 Tiles |  |  |
|  | Y | U | V | Y | U | V |
| ChairliftRide | -2.48 % | -1.80 % | -1.72 % | -3.68 % | -2.99 % | -2.81 % |
| SkateboardInLot | -0.81 % | -1.22 % | -0.17 % | -0.98 % | -1.51 % | -1.22 % |
| Gaslamp | -0.30 % | -0.25 % | -0.20 % | -0.49 % | -0.33 % | -0.31 % |
| Harbor | -0.43 % | -0.27 % | -0.55 % | -0.55 % | -0.40 % | -0.73 % |
| KiteFlite | -0.38 % | -0.32 % | -0.47 % | -0.58 % | -0.45 % | -0.52 % |
| Trolley | -0.15 % | -0.10 % | -0.09 % | -0.17 % | -0.07 % | -0.09 % |
| Overall | -0.76 % | -0.66 % | -0.53 % | -1.07 % | -0.96 % | -0.95 % |

| Reference: | VTM-4.0.1+noMCTS (CE12 Anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tested: | VTM-4.0.1+MCTS+Proposed (MC filter modification) |  |  |  |  |  |
|  | Random Access Main 10 |  |  | Random Access Main 10 |  |  |
|  | 24 Tiles |  |  | 96 Tiles |  |  |
|  | Y | U | V | Y | U | V |
| ChairliftRide | 5.55 % | 5.80 % | 5.37 % | 14.53 % | 18.88 % | 15.84 % |
| SkateboardInLot | 6.71 % | 10.03 % | 12.74 % | 14.48 % | 24.79 % | 26.73 % |
| Gaslamp | 0.46 % | -0.07 % | 0.56 % | 0.54 % | 0.48 % | 1.05 % |
| Harbor | 0.39 % | 0.24 % | -0.56 % | 0.83 % | 1.47 % | 0.26 % |
| KiteFlite | 1.02 % | 1.36 % | 1.12 % | 1.61 % | 2.96 % | 2.76 % |
| Trolley | 0.25 % | 0.14 % | 0.19 % | 0.32 % | 0.69 % | 0.80 % |
| Overall | 2.40 % | 2.92 % | 3.24 % | 5.38 % | 8.21 % | 7.91 % |

| Reference: | VTM-4.0.1+noMCTS (CE12 Anchor) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tested: | VTM-4.0.1+MCTS |  |  |  |  |  |
|  | Random Access Main 10 |  |  | Random Access Main 10 |  |  |
|  | 24 Tiles |  |  | 96 Tiles |  |  |
|  | Y | U | V | Y | U | V |
| ChairliftRide | 8.23 % | 7.75 % | 7.24 % | 18.93 % | 22.57 % | 19.22 % |
| SkateboardInLot | 7.58 % | 11.46 % | 12.84 % | 15.59 % | 26.63 % | 27.83 % |
| Gaslamp | 0.76 % | 0.18 % | 0.76 % | 1.03 % | 0.81 % | 1.36 % |
| Harbor | 0.82 % | 0.51 % | -0.01 % | 1.38 % | 1.88 % | 1.00 % |
| KiteFlite | 1.41 % | 1.68 % | 1.60 % | 2.20 % | 3.43 % | 3.31 % |
| Trolley | 0.40 % | 0.24 % | 0.28 % | 0.49 % | 0.76 % | 0.90 % |
| Overall | 3.20 % | 3.64 % | 3.78 % | 6.60 % | 9.35 % | 8.94 % |

# Conclusion

The proposed method improves the RD performance for the MCTS coded video. It is proposed to test the impact of the tool in a core experiment during the next meeting cycle.

# References

- Hendry, Robert Skupin, Wade Wan, “Description of Core Experiment 12 (CE12): Tile Set Boundary Handling,” JVET-M1032, Marrakech, MA, 9–18 Jan. 2019.

- M. Coban, R. Skupin, “JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding,” JVET-M0870, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).