| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, SW, 19–27 Mar. 2019 | Document: JVET-N0404 |
| --- | --- |

| Title: | CE8: Palette Mode Improvements in HEVC (CE8-2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tsui-Shan Chang, Jian Lou Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz Roman Chernyak, Sergey Ikonin, Jianle Chen | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com yunghsua@qti.qualcomm.com +7-925-008-12-86 chernyak.roman@huawei.com |
| Source: | Alibaba Cloud Computing Ltd. Qualcomm Incorporated Huawei Technologies Co., Ltd. |  |  |

- Abstract

This document proposes the palette mode improvement (CE8-2.2) and reports the results on top of CE8-2.1 when IBC is enabled and disabled. Two improvements are tested on top of CE8-2.1. In CE8-2.2a, the separated palette design for the joint CUs containing both luma and chroma CBs is tested. In CE8-2.2c, the method that combines palette mode and horizontal/vertical intra prediction is tested. In CE8-2.2e, combination of CE8-2.2a and CE8-2.2c is tested.

Both QP range specified in common test condition and low QP range are tested. The CE results show that for CTC QP range, when IBC is enabled,

CE8-2.2a provides 6.85% (AI), 3.18% (RA), and 1.05% (LDB) luma BD-rate gains for class TGM and 2.38% (AI), 1.44% (RA), and 0.99% (LDB) luma BD-rate gains for class F;

CE8-2.2c provides 8.09% (AI), 3.87% (RA), and 1.24% (LDB) luma BD-rate gains for class TGM and 2.63% (AI), 1.79% (RA), and 1.02% (LDB) luma BD-rate gains for class F;

CE8-2.2e provides 8.09% (AI), 3.68% (RA), and 1.26% (LDB) luma BD-rate gains for class TGM and 2.63% (AI), 1.75% (RA), and 1.12% (LDB) luma BD-rate gains for class F.

Additional low QP results of CE8-2.2c are also provided in this proposal. When IBC is enabled, the CE8-2.2c provides 11.86% (AI), 7.57% (RA), and 5.65% (LDB) luma BD-rate gains for class TGM and 1.29% (AI), 1.01% (RA), and 0.52% (LDB) luma BD-rate gains for class F.

Proposal

The decoder syntax in this CE test on top of CE8-2.1. Two palette improvements are tested:

(1) CE8-2.2a: separate palette coding for luma and chroma

In test CE8-2.1, when dual tree is enabled in the configuration (SPS), palette coding is applied separately on luma tree and chroma tree in intra slices and jointly on luma/chroma in inter slices. When dual tree is disabled in the configuration, palette mode is applied jointly for both luma and chroma in all slice types.

In test CE8-2.2a, separated palette coding for luma/chroma components are investigated based on the same palette coding functions in anchor software:  palette coding is applied separately on luma and chroma components in both intra and inter slices

(2) CE8-2.2c: palette mode and horizontal/vertical intra prediction combination

In test CE8-2.2c, the method combining palette mode and intra prediction is tested. The decoder first decoded a flag indicating whether horizontal or vertical intra prediction in used. Then, the decoder decodes a palette and an index map. A specific index (0) is reserved to indicate the sample is reconstructed from intra prediction. When decoder reconstruct the palette coded block, samples with the index, 0, are reconstructed by the horizontal or vertical nearest samples across CU boundary, and samples with other indices are reconstructed by the colors in the palette. Figure 1 shows an example.

(a) vertical intra prediction combination 			 (b) horizontal intra prediction combination

Figure 1. An example of palette mode and horizontal/vertical intra prediction combination.

In test CE8-2.2e, the combination of CE8-2.2a and CE8-2.2c is tested.

Simulation results

The proposed method is evaluated according to the CE8 description [1] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations with the VTM4.0 software. Besides the QP range specified for CTC, additional low QP results are also provided.

CE8-2.2a

The proposed method is tested with/without IBC. Table 1 and 2 shows the results of CE8-2.2a compared with VTM4.0 when IBC is turned on and off in the setting for CTC QP range.

Table 1:Results of the CE8-2.2a for QP range in CTC (IBC is turned on)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.02% | 101% | 103% |
| Class A2 | 0.06% | 0.02% | 0.01% | 99% | 99% |
| Class B | 0.05% | 0.05% | 0.03% | 101% | 99% |
| Class C | 0.06% | 0.03% | 0.01% | 101% | 94% |
| Class E | 0.07% | -0.02% | 0.18% | 101% | 97% |
| Overall | 0.06% | 0.02% | 0.05% | 101% | 98% |
| Class D | 0.09% | 0.06% | 0.05% | 102% | 97% |
| Class F (optional) | -2.38% | -2.20% | -2.28% | 98% | 96% |
| TGM | -6.85% | -5.66% | -5.29% | 90% | 93% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.23% | 0.04% | 101% | 100% |
| Class A2 | 0.04% | 0.05% | 0.14% | 102% | 101% |
| Class B | 0.06% | -0.11% | -0.32% | 102% | 101% |
| Class C | 0.04% | 0.35% | 0.27% | 101% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.02% | 0.00% | 102% | 99% |
| Class D | 0.06% | 0.28% | -0.14% | 102% | 96% |
| Class F (optional) | -1.44% | -1.97% | -1.89% | 100% | 98% |
| TGM | -3.18% | -2.80% | -2.52% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.34% | -0.05% | 101% | 99% |
| Class C | -0.03% | -0.11% | -0.12% | 103% | 104% |
| Class E | -0.03% | 0.14% | 0.27% | 100% | 98% |
| Overall | -0.01% | 0.14% | 0.00% | 101% | 100% |
| Class D | -0.04% | 0.72% | -0.05% | 103% | 102% |
| Class F (optional) | -0.99% | -1.50% | -1.83% | 105% | 100% |
| TGM | -1.05% | -0.75% | -0.55% | 97% | 99% |

Table 2:Results of the CE8-2.2a for QP range in CTC (IBC is turned off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.06% | 102% | 102% |
| Class A2 | 0.05% | 0.13% | 0.03% | 102% | 102% |
| Class B | 0.05% | 0.04% | 0.04% | 102% | 104% |
| Class C | 0.08% | 0.03% | 0.03% | 102% | 105% |
| Class E | 0.09% | 0.07% | 0.06% | 102% | 103% |
| Overall | 0.06% | 0.06% | 0.04% | 102% | 103% |
| Class D | 0.08% | -0.07% | -0.08% | 103% | 103% |
| Class F (optional) | -8.58% | -7.71% | -7.86% | 94% | 96% |
| TGM | -27.12% | -25.34% | -25.16% | 75% | 81% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | 0.06% | 102% | 101% |
| Class A2 | 0.01% | -0.08% | -0.02% | 102% | 101% |
| Class B | 0.05% | -0.03% | -0.15% | 102% | 102% |
| Class C | 0.08% | -0.06% | 0.06% | 104% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.04% | -0.03% | 102% | 102% |
| Class D | 0.12% | -0.15% | -0.33% | 105% | 104% |
| Class F (optional) | -6.72% | -7.52% | -7.22% | 103% | 101% |
| TGM | -13.19% | -12.80% | -12.27% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.41% | -0.16% | 102% | 102% |
| Class C | 0.05% | 0.41% | 0.11% | 102% | 101% |
| Class E | 0.12% | 0.93% | -0.58% | 103% | 99% |
| Overall | 0.05% | 0.20% | -0.17% | 102% | 101% |
| Class D | 0.02% | 0.21% | -0.79% | 103% | 102% |
| Class F (optional) | -3.38% | -3.28% | -2.98% | 103% | 100% |
| TGM | -3.55% | -3.33% | -2.97% | 101% | 100% |

CE8-2.2c

The proposed method is tested with/without IBC. Table 3 and 4 shows the results of CE8-2.2c compared with VTM4.0 when IBC is turned on and off in the setting for CTC QP range.

Table 3:Results of the CE8-2.2c for QP range in CTC (IBC is turned on)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.02% | 103% | 103% |
| Class A2 | 0.06% | 0.04% | 0.03% | 104% | 99% |
| Class B | 0.06% | 0.06% | 0.03% | 105% | 101% |
| Class C | 0.06% | 0.02% | -0.02% | 107% | 107% |
| Class E | 0.07% | -0.03% | 0.18% | 105% | 103% |
| Overall | 0.06% | 0.02% | 0.04% | 105% | 103% |
| Class D | 0.08% | 0.07% | 0.07% | 107% | 107% |
| Class F (optional) | -2.63% | -2.73% | -2.87% | 106% | 100% |
| TGM | -8.09% | -6.55% | -6.16% | 99% | 94% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.18% | -0.03% | 99% | 96% |
| Class A2 | 0.05% | -0.08% | 0.00% | 100% | 98% |
| Class B | 0.06% | -0.22% | -0.21% | 104% | 102% |
| Class C | 0.03% | 0.16% | 0.15% | 105% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.08% | -0.03% | 102% | 100% |
| Class D | 0.05% | 0.03% | -0.09% | 105% | 100% |
| Class F (optional) | -1.79% | -2.61% | -2.61% | 105% | 101% |
| TGM | -3.87% | -3.62% | -3.48% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.21% | 0.07% | 102% | 98% |
| Class C | 0.04% | 0.07% | 0.16% | 105% | 99% |
| Class E | 0.00% | -0.52% | 0.66% | 102% | 99% |
| Overall | 0.02% | -0.02% | 0.24% | 103% | 99% |
| Class D | -0.01% | 0.60% | 0.36% | 103% | 101% |
| Class F (optional) | -1.02% | -2.28% | -2.32% | 107% | 100% |
| TGM | -1.24% | -0.89% | -0.75% | 101% | 99% |

Table 4:Results of the CE8-2.2c for QP range in CTC (IBC is turned off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.06% | 103% | 101% |
| Class A2 | 0.05% | 0.13% | 0.02% | 102% | 102% |
| Class B | 0.05% | 0.04% | 0.04% | 102% | 105% |
| Class C | 0.08% | 0.03% | 0.03% | 101% | 113% |
| Class E | 0.09% | 0.07% | 0.06% | 102% | 111% |
| Overall | 0.06% | 0.06% | 0.04% | 102% | 107% |
| Class D | 0.08% | -0.09% | -0.07% | 104% | 110% |
| Class F (optional) | -8.79% | -7.85% | -8.08% | 103% | 101% |
| TGM | -29.14% | -26.79% | -26.63% | 87% | 83% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.09% | 0.03% | 103% | 100% |
| Class A2 | 0.02% | 0.03% | 0.02% | 106% | 105% |
| Class B | 0.04% | -0.02% | 0.09% | 105% | 104% |
| Class C | 0.05% | -0.06% | -0.06% | 105% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.02% | 105% | 105% |
| Class D | 0.05% | -0.25% | 0.04% | 105% | 107% |
| Class F (optional) | -7.22% | -8.13% | -7.97% | 105% | 103% |
| TGM | -14.51% | -14.35% | -14.25% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.32% | 0.01% | 100% | 97% |
| Class C | 0.04% | 0.38% | -0.15% | 102% | 101% |
| Class E | 0.03% | -0.11% | -0.80% | 101% | 98% |
| Overall | 0.03% | -0.04% | -0.25% | 101% | 99% |
| Class D | 0.05% | -0.38% | -0.28% | 104% | 104% |
| Class F (optional) | -3.45% | -4.53% | -4.63% | 103% | 102% |
| TGM | -3.96% | -3.48% | -3.34% | 104% | 102% |

CE8-2.2e

The proposed method is tested with/without IBC. Table 5 and 6 shows the results of CE8-2.2e compared with VTM4.0 when IBC is turned on and off in the setting for CTC QP range.

Table 5:Results of the CE8-2.2e for QP range in CTC (IBC is turned on)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.00% | 0.02% | 105% | 106% |
| Class A2 | 0.06% | 0.04% | 0.03% | 106% | 100% |
| Class B | 0.06% | 0.06% | 0.03% | 106% | 102% |
| Class C | 0.06% | 0.02% | -0.02% | 107% | 101% |
| Class E | 0.07% | -0.03% | 0.18% | 106% | 101% |
| Overall | 0.06% | 0.02% | 0.04% | 106% | 102% |
| Class D | 0.08% | 0.07% | 0.07% | 107% | 105% |
| Class F (optional) | -2.63% | -2.73% | -2.87% | 106% | 96% |
| TGM | -8.09% | -6.55% | -6.16% | 99% | 91% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.19% | -0.03% | 100% | 99% |
| Class A2 | 0.03% | -0.02% | -0.01% | 101% | 99% |
| Class B | 0.04% | -0.18% | -0.21% | 101% | 98% |
| Class C | 0.04% | 0.23% | 0.21% | 104% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.04% | -0.02% | 101% | 98% |
| Class D | 0.04% | 0.09% | -0.04% | 105% | 100% |
| Class F (optional) | -1.75% | -2.44% | -2.30% | 105% | 101% |
| TGM | -3.68% | -3.36% | -3.01% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.26% | 0.28% | 103% | 100% |
| Class C | 0.04% | -0.13% | -0.01% | 105% | 101% |
| Class E | 0.00% | -0.52% | 0.66% | 101% | 96% |
| Overall | 0.03% | -0.06% | 0.28% | 103% | 99% |
| Class D | -0.02% | 0.51% | 0.39% | 104% | 98% |
| Class F (optional) | -1.12% | -1.73% | -1.72% | 108% | 102% |
| TGM | -1.26% | -1.03% | -0.87% | 99% | 98% |

Table 6:Results of the CE8-2.2e for QP range in CTC (IBC is turned off)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.05% | 0.06% | 102% | 101% |
| Class A2 | 0.05% | 0.13% | 0.02% | 102% | 101% |
| Class B | 0.05% | 0.04% | 0.04% | 102% | 104% |
| Class C | 0.08% | 0.03% | 0.03% | 102% | 109% |
| Class E | 0.09% | 0.07% | 0.06% | 103% | 107% |
| Overall | 0.06% | 0.06% | 0.04% | 102% | 105% |
| Class D | 0.08% | -0.09% | -0.07% | 104% | 108% |
| Class F (optional) | -8.79% | -7.85% | -8.08% | 103% | 98% |
| TGM | -29.14% | -26.79% | -26.63% | 87% | 81% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.10% | 0.04% | 102% | 100% |
| Class A2 | 0.02% | 0.03% | 0.02% | 103% | 102% |
| Class B | 0.04% | -0.02% | 0.09% | 103% | 102% |
| Class C | 0.05% | -0.06% | -0.06% | 104% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | 0.02% | 103% | 102% |
| Class D | 0.05% | -0.25% | 0.08% | 106% | 107% |
| Class F (optional) | -7.21% | -7.96% | -7.75% | 105% | 101% |
| TGM | -14.55% | -13.85% | -13.47% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.33% | 0.01% | 102% | 100% |
| Class C | 0.04% | 0.38% | -0.15% | 101% | 96% |
| Class E | 0.03% | -0.11% | -0.80% | 101% | 95% |
| Overall | 0.03% | -0.04% | -0.25% | 101% | 98% |
| Class D | 0.05% | -0.38% | -0.28% | 103% | 98% |
| Class F (optional) | -3.44% | -3.73% | -3.07% | 102% | 97% |
| TGM | -4.16% | -3.74% | -3.29% | 103% | 98% |

CE8-2.2c (Low QP range)

Table 7 shows the results of CE8-2.2c compared with VTM4.0 when IBC is turned on in the setting for low QP range

Table 7: Results of the CE8.2.1 for low QP (IBC is turned on)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | -0.05% | 105% | 105% |
| Class A2 | 0.01% | 0.01% | 0.01% | 102% | 104% |
| Class B | -0.18% | -0.13% | -0.13% | 109% | 102% |
| Class C | -0.11% | -0.15% | -0.11% | 110% | 105% |
| Class E | -1.00% | -1.82% | -2.13% | 105% | 95% |
| Overall | -0.24% | -0.37% | -0.42% | 107% | 102% |
| Class D | -0.02% | -0.19% | -0.16% | 109% | 104% |
| Class F (optional) | -1.29% | -1.39% | -1.48% | 109% | 99% |
| TGM | -11.86% | -9.79% | -9.62% | 102% | 88% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | -0.03% | 103% | 103% |
| Class A2 | 0.02% | 0.01% | 0.02% | 101% | 102% |
| Class B | -0.03% | -0.03% | 0.00% | 103% | 101% |
| Class C | -0.05% | -0.08% | -0.07% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.03% | -0.02% | 103% | 101% |
| Class D | -0.02% | -0.15% | -0.04% | 105% | 102% |
| Class F (optional) | -1.01% | -1.06% | -1.07% | 106% | 100% |
| TGM | -7.57% | -5.26% | -5.18% | 101% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vcgit.hhi.fraunhofer.de][jvet][VVCSoftware_BMS.git][599ab8f][VTM] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F (optional) | -0.52% | -0.37% | -0.38% | 108% | 102% |
| TGM | -5.65% | -3.61% | -3.49% | 102% | 100% |

Reference

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding Tools”, JVET-M1028, 13th meeting, Marrakech, MA, January 2019.

- Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz, Y.-C. Sun, J. An, J. Lou, “CE15-2: Palette mode of HEVC SCC”, JVET-L0336, 12th meeting, Macao, CN, October 2018.

Patent rights declaration(s)

Alibaba Cloud Computing Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).