| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0406-v2 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0042-r1 |
| --- | --- | --- | --- |

| Title: | CE1: LIC applied after bi-prediction (CE1-5.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay, Yuwen He 9276 Scranton Road, San Diego, CA 92121 | Tel: Email: | +1 858 210 4819 saurav.bandyopadhyay@interdigital.com, yuwen.he@interdigital.com. |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This proposal reports the test results of the LIC simplification as in CE1 Test CE1-5.3 whereby, LIC is applied after bi-prediction. In this simplified method, the bi-prediction reference samples of the template are first generated using bi-predictive weighted average (BPWA) based on the motion vectors of the current block. Then the LIC parameters are derived by minimizing the difference between the template reference samples and the template samples. The results are reported for 2 sub-tests of CE1 Test CE1-5.3 namely, Test 1.5.3a: All reconstructed samples are used, Test 1.5.3b: Samples of intra, CIIP and IBC coded block are not used. i.e. only inter reconstructed samples are used.

The sub-test 1.5.3.a achieves {-0.56%, -0.25%, -0.25%} BD rate in RA configuration over VTM4.0 and {-0.59%, -0.49%, -0.60%} over VTM4.0 in LDB configuration.

The sub-test 1.5.3b achieves {-0.47%, -0.40%, -0.46%} BD rate in RA configuration over VTM4.0 and {-0.54%, -0.38%, -0.57%} over VTM4.0 in LDB configuration.

# Introduction

Local Illumination Compensation (LIC) is an inter coding tool that is currently being studied in the Core Experiment 1 (CE1) to address the issue of local illumination changes that exist in temporal neighboring pictures. The LIC is based on a linear model where a scaling factor and an offset are applied to the reference samples to obtain the prediction samples of the current block. The proposal to apply LIC after the bi-prediction is integrated to the test model according to the description of sub-tests CE1.5.3a and CE1.5.3b [2]. Further details have been described in [3].

# Proposed simplifications

## Simplified LIC process for bi-prediction CUs

The traditional approach performs bi-predictive weighted average (BPWA) on top of LIC. The proposed approach first performs the bi-prediction of the template samples from two prediction lists to produce BPWA based bi-prediction template reference samples. The LIC parameters are estimated using the BPWA based bi-prediction template reference samples and the template samples of the current CU [3].

## Disabling LIC for DMVR

In order to increase the accuracy of the MVs of the merge mode, a bilateral matching based decoder side motion vector refinement (DMVR) is a new inter coding tool included in VTM4.0 [5]. However, the maximum unit size for DMVR searching process is limited to 16x16. Therefore, when the width and height of a CU are larger than 16 luma samples, it will be further divided into sub-blocks with and height equal to 16 luma samples. Disabling LIC for the DMVR will reduce the computational complexity. It is proposed to disable LIC for the DMVR.

# Results

The following experiments were conducted using the VTM-4.0 reference software according to the CE1 description in [2] and the JVET common test conditions [4] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations.

There are two tests CE1-5.3a and CE1-5.3b.

CE1-5.3a: all reconstructed samples except IBC are used.

CE1-5.3b: The neighboring samples of intra, CIIP and IBC coded blocks are excluded.

## Comparison with VTM-4.0 anchor

### CE1-5.3a

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.92% | 0.15% | 0.02% | 129% | 100% |  |  |  |  |  |
| Class A2 | -0.37% | -0.34% | -0.40% | 117% | 102% |  |  |  |  |  |
| Class B | -0.60% | -0.28% | -0.23% | 121% | 103% | -0.84% | -0.79% | -0.66% | 125% | 100% |
| Class C | -0.39% | -0.44% | -0.38% | 124% | 100% | -0.72% | -0.47% | -0.83% | 128% | 100% |
| Class E |  |  |  |  |  | 0.02% | -0.02% | -0.19% | 100% | 98% |
| Overall | -0.56% | -0.25% | -0.25% | 122% | 101% | -0.59% | -0.49% | -0.60% | 119% | 100% |
| Class D | -0.16% | -0.09% | -0.28% | 122% | 101% | -0.22% | 0.03% | -0.69% | 123% | 101% |
| Class F (optional) | -1.05% | -0.65% | -0.67% | 118% | 101% | -3.29% | -2.54% | -3.22% | 120% | 100% |

Table 1.

### CE1-5.3b

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.46% | -0.69% | 132% | 101% |  |  |  |  |  |
| Class A2 | -0.31% | -0.43% | -0.35% | 118% | 102% |  |  |  |  |  |
| Class B | -0.46% | -0.37% | -0.36% | 122% | 101% | -0.75% | -0.57% | -0.53% | 126% | 100% |
| Class C | -0.36% | -0.38% | -0.50% | 127% | 102% | -0.66% | -0.58% | -0.79% | 132% | 102% |
| Class E |  |  |  |  |  | -0.04% | 0.19% | -0.34% | 100% | 100% |
| Overall | -0.47% | -0.40% | -0.46% | 125% | 101% | -0.54% | -0.38% | -0.57% | 121% | 101% |
| Class D | -0.15% | -0.25% | -0.16% | 123% | 101% | -0.23% | -0.07% | -0.64% | 126% | 103% |
| Class F (optional) | -0.92% | -0.61% | -0.65% | 120% | 101% | -2.92% | -2.47% | -3.00% | 122% | 100% |

Table 2.

## Comparison with CE1-5.1 test

### CE1-5.1a

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.16% | -0.30% | 111% | 99% |  |  |  |  |  |
| Class A2 | -0.05% | 0.06% | 0.00% | 107% | 101% |  |  |  |  |  |
| Class B | -0.15% | -0.04% | -0.08% | 108% | 102% | -0.26% | -0.55% | -0.20% | 109% | 100% |
| Class C | -0.05% | -0.17% | -0.03% | 109% | 98% | -0.20% | -0.63% | -0.40% | 111% | 100% |
| Class E |  |  |  |  |  | 0.02% | -0.04% | -0.11% | 97% | 95% |
| Overall | -0.13% | -0.08% | -0.09% | 108% | 100% | -0.17% | -0.45% | -0.24% | 106% | 99% |
| Class D | -0.02% | -0.04% | -0.06% | 108% | 100% | -0.05% | 0.39% | 0.06% | 108% | 100% |
| Class F (optional) | -0.22% | -0.22% | -0.20% | 109% | 100% | -0.75% | -0.91% | -1.28% | 108% | 99% |

Table 3.

### CE1-5.1b

|  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | -0.28% | -0.32% | 111% | 100% |  |  |  |  |  |
| Class A2 | -0.07% | -0.15% | -0.06% | 106% | 100% |  |  |  |  |  |
| Class B | -0.13% | -0.15% | -0.20% | 108% | 100% | -0.24% | -0.28% | -0.16% | 108% | 100% |
| Class C | -0.07% | 0.05% | -0.09% | 110% | 100% | -0.20% | -0.49% | -0.45% | 111% | 101% |
| Class E |  |  |  |  |  | -0.08% | 0.02% | -0.22% | 97% | 96% |
| Overall | -0.12% | -0.12% | -0.17% | 109% | 100% | -0.19% | -0.27% | -0.27% | 106% | 99% |
| Class D | 0.00% | 0.12% | 0.04% | 108% | 99% | -0.06% | 0.19% | 0.18% | 109% | 101% |
| Class F (optional) | -0.18% | -0.12% | -0.17% | 109% | 101% | -0.79% | -0.85% | -0.66% | 107% | 99% |

Table 4.

# Conclusion

The proposed LIC simplifications improves the performance over the base LIC version in CE1-5.1 by generating the bi-prediction reference samples of the template samples for the current block and estimate the LIC parameters only once. By this way, only one single LIC operation (i.e., one LIC parameter estimation and one LIC-based sample adjustment) needs to be performed for generating the prediction signal of the current block. The LIC for DMVR is proposed to be disabled.

# References

- K. Abe, T. Toma and J. Li, “CE-4 related: Low pipeline latency LIC”, JVET-L0120, Macao, CN, October 2018.

- J. Ström, S. Ikonin, V. Seregin, “Description of Core Experiment 1 (CE1): Post-prediction and post-reconstruction filtering”, JVET-M1021, Marrakech, MA, 9–18 Jan. 2019.

- S. Bandyopadhyay, X. Xiu, Y. He, “CE10-related: Local illumination compensation simplifications”, JVET-M0224, Marrakech, MA, January 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, Marrakech, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).