| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0408-v1 |
| --- | --- |

| Title: | Non-CE9: Early Termination Techniques for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal evaluates the coding performance of the early termination techniques (i.e. 2-stage six-point search and SAD-based early termination) currently used in VTM-4.0 software. Besides, a MVD-based early termination technique and other search patterns (i.e. 25-point full search, 2-stage square) are also tested for comparison. Experiments were conducted on top of the VTM-4.0 with Random-Access testing condition. Performance results are summarized for Class A1/A2/C sequences as follows.

[Test 1] Full search                         : (A1/A2/C) -0.07%/-0.08%/0.01%,   (Enc.) 100%, (Dec.) 102%;

[Test 2] Full search + SAD-based  : (A1/A2/C) -0.07%/-0.08%/0.00%,   (Enc.) 100%, (Dec.) 101%;

[Test 3] Full search + MVD-based : (A1/A2/C) -0.08%/-0.04%/-0.01%, (Enc.) 100%, (Dec.) 101%;

[Test 4] 2-stage six-point                : (A1/A2/C) 0.00%/0.00%/0.00%,     (Enc.) 100%, (Dec.) 100%;

[Test 5] 2-stage square                    : (A1/A2/C) -0.05%/-0.07%/-0.03%, (Enc.) 101%, (Dec.) 101%.

# Introduction

During the evolution of DMVR, there are fast algorithm and early termination methods are proposed and evaluated to reach an acceptable balance between coding performance and decoding complexity or to find a compromise between hardware decoder and software decoder. This proposal will introduce these methods, as follows, in the next section. Performance results will be presented in Section 3.

# Technical Description

This section presents the early termination methods for DMVR. It is noted that the search range of DMVR in VTM-4.0 is ±2 pixels vertically and horizontally, where the center search point is located at where the original MV points to. In total, the maximum number of search points is 25 or 5x5.

## SAD-based Early Termination

In sub-block level, DMVR checks whether the SAD (sum of absolute difference) value between two prediction blocks (i.e. L0 and L1) pointed to by the respective original MVs falls within a pre-defined threshold, that is, 4 * sub-block width * sub-block height / 2). If the condition check holds true, then DMVR does not perform for this sub-block.

## MVD-based Early Termination

An additional MVD-based condition is imposed on DMVR to bypass the MV refinement process. Specifically, the MV difference between the selected candidate and any of the previous candidates in the same merge list is not less than a pre-defined threshold (that is, ½- and 1-pixel-wide intervals for blocks with less than 256 and more than 256 pixels, respectively).

## Full Search

The search process starts from the top-left point within the 5x5 search points in raster-scanning order.

## Two-stage 6-point Search

This search pattern starts from the center of the 5x5 search points for these 6 search points {(0,0), (±1,0), (0,±1), (x,y)} in the first search iteration, where x can be either 1 or -1 depending on which one of (1,0) and (-1,0) reaches a lower bilateral matching cost and y can also be either 1 or -1 depending on which one of (1,0) and (-1,0) reaches a lower bilateral matching cost. Then at the second iteration, the same 6-point search is applied, but the center of this search pattern is shifted toward the best search point found in the first iteration.

## Two-stage Square Search

This search process is the same as the 2-stage 6-point search, except that the 6 search points are replaced by 9, covering {(0,0), (±1,±1)}.

# Experimental Results

The experiments for DMVR with the proposed methods were conducted on top of the VTM-4.0 software with the common test conditions [1] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-4.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

Five tests were conducted. The respective performance results are summarized in the table below.

(Test 1) Full search;

(Test 2) Full search + SAD-based early termination;

(Test 3) Full search + MVD-based early termination;

(Test 4) 2-stage six-point;

(Test 5) 2-stage square.

- Performance results.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [Test 1] Full search |  |  |  |  | [Test 2] Full search + SAD-based early termination |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.09% | -0.05% | 100% | 100% | -0.07% | -0.03% | -0.09% | 100% | 100% |
| Class A2 | -0.08% | -0.09% | -0.06% | 101% | 104% | -0.08% | -0.05% | -0.09% | 101% | 102% |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.01% | -0.06% | -0.11% | 100% | 102% | 0.00% | 0.00% | -0.06% | 101% | 101% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.02% | -0.06% | -0.04% | 101% | 104% | -0.02% | -0.05% | -0.08% | 101% | 103% |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | [Test 3) Full search + MVD-based early termination |  |  |  |  | [Test 4] 2-stage six-point search |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.01% | -0.07% | 100% | 100% | 0.00% | 0.03% | -0.01% | 99% | 99% |
| Class A2 | -0.04% | -0.05% | 0.00% | 101% | 103% | 0.00% | 0.05% | 0.02% | 99% | 101% |
| Class B |  |  |  |  |  | 0.01% | 0.00% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.04% | -0.11% | 100% | 101% | 0.00% | -0.05% | -0.05% | 100% | 100% |
| Overall |  |  |  |  |  | 0.00% | 0.00% | -0.02% | 100% | 100% |
| Class D | 0.01% | 0.10% | -0.01% | 101% | 102% | 0.00% | 0.00% | 0.01% | 100% | 101% |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | [Test 5] 2-stage square search |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.06% | -0.04% | 101% | 102% |
| Class A2 | -0.07% | -0.03% | -0.05% | 101% | 103% |
| Class B |  |  |  |  |  |
| Class C | -0.03% | -0.05% | -0.04% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.11% | -0.09% | 100% | 101% |
| Class F |  |  |  |  |  |

# References

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).