| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0409-v1 |
| --- | --- |

| Title: | CE9-related: Horizontal and Vertical Boundary Padding Process of DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen1, Wei-Jung Chien2, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 1chunchic@qti.qualcomm.com, 2wchien@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the coding performance of Decoder-Side Motion Vector Derivation (DMVR) with either horizontal or vertical boundary padding process removed on top of CE9-1.1a (i.e. removing 4xN/8x8 CUs). Unlike CE9-1.1b that the boundary padding process is disabled along horizontal and vertical directions, this contribution disables the boundary padding process only for either one of the 2 directions. The first test removes the horizontal one only that is used to extend the size of reference blocks with 2 extra pixels along horizontal direction. Instead, it loads corresponding pixels from reference pictures directly to fill out the extended part of the reference blocks. Similarly, the second test removes the vertical boundary padding process while keeping the horizontal one as-is. Experiments were conducted on top of the VTM-4.0 with Random-Access testing condition. The performance results are summarized as follows.

[Test 1] Hor. padding off: (Y) xx%, (U) xx%, (V) xx%, (Enc.)   xx%, (Dec.) xx%.

[Test 2] Both padding off: (Y) 0.04%, (U) 0.04%, (V) 0.05%, (Enc.)   99%, (Dec.) 99%.

# Introduction

In CE9.1.1[1], Decoder-side Motion Vector Refinement (DMVR) was studied with (a) 4xN/8x8 CUs disabled and (b) both vertical/horizontal boundary padding process disabled. Based CE9.1.1 software, this contribution studies the same aspect of CE9.1.1b but only disables either one of vertical and horizontal boundary padding processes. Technical details will be presented in the next section. Performance results and complexity analysis (i.e. memory bandwidth) will be presented in Section 3.

# Technical Description

## Removal of Horizontal Boundary Padding Process

This coding tool removes the horizontal boundary padding process that is used in VTM-4.0 to extend the size of reference blocks with 2 extra pixels at each horizontal side. As instead, it loads corresponding pixels at the extended area from reference pictures directly to fill out the extended area at horizontal direction only, resulting in a wider reference block (Figure 1 (b)) than the original one (Figure 1 (a)). As illustrated in Figure 1 (a) and (b), the DMVR in VTM-4.0 only loads the reference block (i.e. (W+7)x(H+7)) pointed to by the original MV and pad out missing pixels at extended area using the boundary pixels of the original reference block, while the proposed method loads a wider range of reference pixels (i.e. (W+7+4)*(H+7)) and leaves the vertical padding process as-is to pad out missing pixels at vertical directions.

| (a) | (b) | (c) |
| --- | --- | --- |

- Comparison of (a) the original reference block, i.e. blue circles, pointed to by the original MV and the extended area for padded pixels, i.e. white circles, (b) the wider reference block with 2 extra loaded pixels at horizontal directions and (c) the taller reference block with 2 extra loaded pixels at vertical directions.

## Removal of Vertical Boundary Padding Process

On the contrary, this coding tool removes the vertical boundary padding process and leave horizontal boundary padding process unchanged. As Figure 1 (c) depicts, a taller reference block of size (W+7)*(H+7+4) is loaded from reference picture and the horizontal padding process is left unchanged to pad out missing pixels at each horizontal side.

# Experimental Results

The experiments for DMVR with the proposed methods were conducted on top of the VTM-4.0 software with the common test conditions [3] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-4.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and tests.

## Coding Performance

Two tests were conducted. The respective performance results are summarized in Table 1. In addition, the performance results of CE9-1.1b [1] (i.e. removing both hor. and ver. boundary padding processes) are also provided in Table 2 for comparison.

[Test 1] CE9-1.1.a + Removing horizontal boundary padding process from DMVR.

[Test 2] CE9-1.1.a + Removing vertical boundary padding process from DMVR.

- Performance results of Test 1 and Test 2 relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test 1 vs. VTM-4.0 |  |  |  |  | Test 2 vs. VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.08% | 0.01% | 0.01% | 100% | 99% |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.12% | 0.09% | 0.08% | 100% | 98% |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

- Performance results of CE9-1.1b relative to VTM anchor.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | CE9-1.1b vs. VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | 0.02% | 99% | 100% |
| Class A2 | 0.03% | 0.07% | 0.12% | 98% | 98% |
| Class B | 0.03% | 0.03% | 0.02% | 99% | 99% |
| Class C | 0.07% | 0.00% | 0.06% | 100% | 99% |
| Overall | 0.04% | 0.04% | 0.05% | 99% | 99% |
| Class D | 0.11% | 0.04% | 0.04% | 100% | 100% |
| Class F | 0.02% | 0.02% | 0.00% | 100% | 99% |

## Memory Bandwidth

The memory bandwidth of DMVR is measured by using the same methodology as introduced in previous CE9 summary report (JVET-M0029) to calculate the number of required reference samples for MC, as follows:

F(W, H, Taps, SR) = ((W + 2 * SR) + Taps - 1 ) * ((H + 2 * SR) + Taps - 1 ) * 2,

where W and H represents the width and height of a given CU, respectively, Taps is the interpolation filter length of MV, SR denotes the search range (i.e. 2) of DMVR and the trailing 2 is for bi-prediction. To understand the full impact on bandwidth when introducing a new coding tool to DMVR, this contribution takes the memory bandwidth of Y/U/V channels into account. Besides, in order to compare the memory bandwidth among different CU sizes, this contribution normalizes the memory bandwidth numbers by the number of pixels of a given CU, as follows:

Fyuv(W, H) = ( F(W, H, 8, 2) + F(W>>1, H>>1, 4, 1) + F(W>>1, H>>1, 4, 1) ) / (W * H).

- Memory bandwidth analysis.

|  | (VTM-4.0) 4x8/8x4 Bi | (VTM-4.0) 4x16 DMVR | (CE9-1.1b) 8x16/16x8 DMVR | (Test 1) 8x16/16x8 DMVR | (Test 2) 8x16/16x8 DMVR |
| --- | --- | --- | --- | --- | --- |
| Bandwidth (Fyuv) | Fyuv(4,8) = 14.7 Fyuv(8,4) = 14.7 | Fyuv(4,16) = 11.3 | Fyuv(8+4,16+4) = 11.7 Fyuv(16+4,8+4) = 11.7 | Fyuv(8+4,16) = 9.9 Fyuv(16+4,8) = 9.2 | Fyuv(8,16+4) = 9.2 Fyuv(16,8+4) = 9.9 |

This proposal additionally evaluates memory bandwidth with various memory access pattern (e.g. 4x2, 4x4, 8x4, 8x8). This can be formulated as

P(M, N, m, n)=ceiling( ( m-1+M+L-1 ) / m ) * ceiling( ( n-1+N+L-1 ) / n ),

where m and n represent a m-by-n memory access pattern, M and N denote the block width and height and L is the length of interpolation filter. This P value is normalized by 8x16 blocks in order to compare the 4x16 DMVR in VTM-4.0 and the 8x16/16x8 DMVR of this proposal.

P4x16_VTM(M, N, m, n) = 2 * P(M, N, m, n),

P8x16_CE9.1.1(M, N, m, n) = 1 * P(M+4, N+4, m, n),

P8x16_Test1(M, N, m, n) = 1 * P(M+4, N, m, n),

P8x16_Test2(M, N, m, n) = 1 * P(M, N+4, m, n).

- Memory bandwidth analysis with various memory access pattern.

| m x n | (VTM-4.0) 4x16 DMVR | (CE9-1.1b) 8x16/16x8 DMVR | (Test 1) 8x16 DMVR | (Test 2) 8x16 DMVR |
| --- | --- | --- | --- | --- |
| 8x2 | 144 | 112 / 100 | 96 / 80 | 84 / 80 |
| 8x4 | 84 | 64 / 60 | 56 / 50 | 48 / 48 |
| 4x2 | 192 | 168 / 160 | 144 / 128 | 140 / 140 |
| 4x4 | 112 | 96 / 96 | 84 / 80 | 80 / 84 |
| 8x8 | 48 | 40 / 40 | 32 / 30 | 30 / 32 |
| 16x16 | 24 | 18 / 18 | 18 / 12 | 12 / 18 |

# References

- C.-C. Chen, W.-J. Chien and M. Karczewicz, “CE9: Removal of 4xN/8x8 CUs and Boundary Padding Process from DMVR (Test 9.1.1),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0407, Mar. 2019.

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 4),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, Jan. 2019.

- F. Bossen, J. Boyce and et. al., “JVET Common Test Conditions and Software Reference Configurations for SDR Video,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1010, Jan. 2019.

- W.-J. Chien, J. Boyce and et. al., “JVET AHG Report: Tool Reporting Procedure (AHG13),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0013, Mar. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).