| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0411 |
| --- | --- |

| Title: | AHG12: Layout signalling of independent coded regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2596 DA The Hague The Netherlands | Tel: Email: | +31 (0)88 866 42 61 emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution addresses the issue of signalling the spatial relationship of independent coded regions. There are several contributions for supporting the decoding of independent coded regions, however this contribution is orthogonal to the actual solution for supporting these independent coded regions.

Two solutions are discussed, the signalling of absolute position and size of the independent coded regions and the signalling of relative position of neighbouring independent coded regions.

The relative spatial relationship between different independent coded regions is expressed by using the concept of hooks. Hooks permit describing the relative position, hence not absolute, between independent coded regions without a coordinate system. A pair of matching hook identifiers expresses that two of independent coded regions are neighbours along a certain border. No matching hook identifiers expresses the absence of spatial relationship between two independent coded regions.

The contribution further compares both absolute and relative signalling in terms:

- Bit efficiency

- Number of rewriting operations for sub bitstream extraction and merging

- Issue of double allocation of sample position in the output picture

- Issue of inconsistency of different aspect ratio between the signalled size and the size of the decoded region in luma samples

- Issue of inconsistency of different values between the signalled size and the size of the decoded region in luma samples

It is asserted that signalling the layout of independent coded regions in the output picture using relative position information as several advantages over absolute position information based on 2D cartesian coordinates.

# Introduction

This contribution addresses the issue of signalling the spatial relationship of independent coded regions. There are several contributions for supporting the decoding of independent coded regions, however this contribution is orthogonal to the actual solution for supporting these independent coded regions. Therefore, this contribution relates to both AHG12 and AHG19 where the possible solutions for these independent coded regions are discussed.

Two solutions are discussed, the signalling of absolute position and size of the independent coded regions and the signalling of relative position of neighbouring independent coded regions.

The relative spatial relationship between different independent coded regions is expressed by using the concept of hooks. Hooks permit describing the relative position, hence not absolute, between independent coded regions without a coordinate system. A pair of matching hook identifiers expresses that two of independent coded regions are neighbours along a certain border. No matching hook identifiers expresses the absence of spatial relationship between two independent coded regions.

The contribution further shows that relative position signalling has greater advantages in terms:

- Bit efficiency

- Smaller number of rewriting operations (even null sometimes) for sub bitstream extraction and merging

- No risk of double allocation of sample position in the output picture

- No risk of inconsistency of different aspect ratio between the signalled size and the size of the decoded region in luma samples

- No risk of inconsistency of different values between the signalled size and the size of the decoded region in luma samples

It is asserted that signalling the layout of independent coded regions in the output picture using relative position information as several advantages over absolute position information based on 2D cartesian coordinates.

# Background - Navigation in panoramic and omnidirectional content

There are a several applications where the spatial subdivision of a large video content can be useful. Examples include applications when the user may want to only watch one area of the full video and/or pan. In this case, streaming and/or decoding the whole frame only to display a sub part of it is inneficient. It is desirable to be able to access only a region of the video.

VR 360 applications do not render the whole video but only a small part of it corresponding to the user's viewport. Figure 1 shows an example of how a cube map video tiled in 24 independent coded regions. Figure 2 shows a system that save bandwidth when streaming: when a user is looking in a certain direction then only the coded regions needed to render the user's viewport are decoded. In this case, this would allow for approximately 50% saving in transmitted bitrate compared to sending the whole bitstream. Note that this tilling of cube map video is merely an example and any other tiling scheme could be used to describe the VR application case.

Figure 1 – 360 video partitioned into 24 independent coded regions (red)

Figure 2 - Viewport-based selection of coded regions

# Problem Statement

## From the content producer perspective

At the production side, each independent coded region has a certain position which conveys a certain meaning. For instance, the position of a coded region in an omnidirectional projected frame determines the region on the sphere it represents. Therefore, there is a need to signal the relationship between all the independent coded regions when producing the content.

## From the content receiver perspective

Applications as presented in the background section may select a subset of the available coded regions. As a result, the spatial relationship of coded regions may be dynamically changed by the application in such a way that the output picture can be further processed by the application in an efficient manner.

# Possible solutions

## Option A: Absolute coordinate system for signalling position and size of each independent coded regions

A first solution is to signal for every independent coded region its position and size in a common layout using 2D cartesian coordinate system. The parameters comprise the position of the top-left corner of the region as well as its size.

Let’s assume the following examples of an arrangement of 16 independent coded regions in a grid fashion.

Figure 3 - Example of absolute signaling of independent coded region layout

## Option B: Relative position information for signalling adjacent independent coded regions

A second solution is to use the concept of neighbours to describe the spatial relationship between independent coded regions. Hooks serve as descriptors of points of contact between different independent coded regions. An independent coded region has four hooks, north, south, west and east. If the identifier of two hooks, either north/south or east/west, from different independent coded regions match then the independent coded regions are adjacent to each other at this common edge. Figure 3 shows an example of matching and non-matching hooks.

Figure 4 – Example of hooks and identifier numbers

The way a decoder can reassemble the independent coded regions into a single output picture is described below with the example from Figure 3.

- Independent coded regions vertically adjacent – region 1 and 2 (bottom hook of 2 and top hook of 1 have same ID – 76 in the example)

  - The decoder will place region 1’s top left pixel below region 2’s bottom left pixel

- Independent coded regions horizontally adjacent – region 3 and 1 (right hook of 3 and left hook of 1 have same ID – 1 in the example)

  - The decoder will place region 1’s top left pixel to the right of region 3’s top right pixel

Figure 5 - Hooks dictate how independent coded regions are arranged in the output picture

Let’s now assume the same example 16 independent coded regions in a grid fashion.

Figure 6 – Example of relative signaling of of independent coded layout

# Proposal

It is argued that option B provides more advantages over option A.

In particular, the relative position information yields the following benefits:

- A higher Bit efficiency

- A smaller (sometimes null) number of rewriting operations for sub bitstream extraction or merging

- Prevents the case of double allocation of sample position in the output picture

- Prevents the case of inconsistency of aspect ratios between the signalled size and the size of the decoded region

- Prevents the case of samples position interpolation when the signalled size does not match the size of the decoded region

## Advantage #1: Bit efficiency

Parameters to be signalled in case of absolute position signalling:

- Layout width

- Layout height

- For each region

  - Top left horizontal position

  - Top Left vertical position

  - Region width

  - Region height

Parameters to be signalled in case of relative position signalling:

- For each region

  - 1 to 3 boundary identifiers

In the example of Figure 2 and Figure 5:

|  | Absolute position signalling | Relative position signalling |
| --- | --- | --- |
| Total number of parameters | 66 | 30 |
| Range of values used for each parameter | 13 bits | 4 bits |
| Maximum size of signalling | 858 bits | 120 bits |

This example shows that the relative position signalling inherently consumes less bits than the absolute signalling.

## Advantage #2: Minimize the number of rewriting operations for sub bitstream extraction or merging

### Example for extraction

Figure 7 – Bitstream extraction rewriting operation with absolute position information

Figure 8 - Bitstream extraction with relative position information

As can be shown in the example in Figure 6, extracting 12 regions leads to rewrite 13 parameters in case of absolute position information. In Figure 7, the neighbouring independent coded regions remain the same, therefore there is no need to rewrite any parameter in case of relative position information.

|  | Absolute position signalling | Relative position signalling |
| --- | --- | --- |
| Parameters to rewrite | 13 | 0 |

### Example for merging

Similar to the above example, if we consider the reverse operation of merging the coded regions 1, 5, 9 and 13 with the extracted one, this will result in the following rewriting operations:

|  | Absolute position signalling | Relative position signalling |
| --- | --- | --- |
| Parameters to rewrite | 13 | 2 (assuming the hooks between region 1 and 2 were not matching before merging) |

### Example for rearranging

Similar to the above example of coded region layout, if we consider the appending the coded regions 1, 5, 9 and 13 to write of the layout, this will result in the following rewriting operations.

For the relative position signalling the following modification are performed: removing west of region 2, east of region 1 and adding matching hooks west of region 1 with east of region 4.

|  | Absolute position signalling | Relative position signalling |
| --- | --- | --- |
| Parameters to rewrite | 20 | 4 |

## Advantage #3: Avoid double allocation of sample position in the output picture

Omitting for now the case of sample value interpolation, every sample can be associated with at most one sample of one coded region.

However, the signalling of the layout with absolute position may express overlap of the independent coded region in the output picture. As opposed to the relative position signalling that do not allow by design overlap of coded regions in the output picture.

## Advantage #4: Avoid inconsistency of different aspect ratio between the signalled size and the size of the decoded region

The independent coded regions have their width and height expressed in luma samples in the bitstream for decoding purposes, regardless of which solution will be chosen. As a result, there is a risk that the aspect ratio of the signalling of the size in the coordinate system of the layout differ from the aspect ratio signalled of the coded region in the bitstream.

As opposed to the relative position signalling that refrains from expressing any size at all which prevents any risk of data inconsistency for aspect ratio.

## Advantage #5: Avoid the need for interpolation of samples values when the signalled size does not match the size do the decoded region

The independent coded regions have their width and height expressed in luma samples in the bitstream for decoding purposes, regardless of which solution will be chosen. As a result, there is a risk that the width or the height of the signalling of the size in the coordinate system of the layout differ from the width or the height signalled of the coded region in the bitstream.

As opposed to the relative position signalling that refrains from expressing any size at all which prevents any risk of data inconsistency for width and height.

## Generic syntax for signalling relative position information of independent coded regions

Irrespective of the actual solution for supporting independent coded regions, the following is the proposed syntax for signalling relative position information between independent coded regions.

| appropriate_syntax_element( ) { | Descriptor |
| --- | --- |
| … |  |
| boundary_id_length_minus1 | ue(v) |
| for( i = 0; i <= NumberIndependentCodedRegions; i++ ) |  |
| north_boundary_id[ i ] | u(v) |
| east_boundary_id[ i ] | u(v) |
| south_boundary_id[ i ] | u(v) |
| west_boundary_id[ i ] | u(v) |
| } |  |
| … |  |
| } |  |

Notes:

- NumberIndependentCodedRegions would be given by the actual solution chosen for supporting the decoding of independent coded regions.

- The boundary identifiers could be also distributed in syntax elements belonging to each independent coded region and not centralised as shown above.

boundary_id_length_minus1 plus 1 specifies the number of bits used to represent the syntax elements north_group_id[ i ], east_group_id[ i ], south_group_id[ i ] and west_group_id[ i ]. The value of boundary_id_length_minus1 shall be in the range of 0 to 15, inclusive.

north_boundary_id[ i ], east_boundary_id[ i ], south_boundary_id[ i ], west_boundary_id[ i ] specifies the identifier respecitvely at the north, east, south and west boundary of the decoded picture of the i-th layer within the output picture. The length of the north_boundary_id[ i ], east_boundary_id[ i ], south_boundary_id[ i ] and west_boundary_id[ i ] syntax elements is boundary_id_length_minus1 + 1 bits.

When both values of the pair north_boundary_id[ i ] and south_boundary_id[ j ] or both values of the pair east_boundary_id[ i ] and west_boundary_id[ j ] are equal the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the output picture and they share a common boundary respectively at the boundary north/south and east/west. Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the output picture.

# Conclusion

The proposal is to adopt the signalling of relative position of independent coded regions as presented in this contribution. The generic syntax provided here would have to updated to be inserted in the actual solution chosen for supporting but the semantic of the parameters would remain the same.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).