| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0413 |
| --- | --- |

| Title: | CE8-related: Quantized residual BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Muhammed Coban | Tel: Email: | +1 858 658 3973 martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes to apply BDPCM in quantized residual domain instead of reconstructed sample domain. BDPCM signalling and prediction directions are same as the methods utilized in sample domain BDPCM. The simulations results show Class F: -3.91% AI, -2.68% RA, X.Xx% LB BD-Rate, and Class SCC: -5.98% AI, -3.57% RA, -2.73% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions for quantized residual BDPCM case. For both proposed methods combined, the simulations results show Class F: -4.04% AI, -2.92% RA, -2.09% LB BD-Rate, and Class SCC: -6.33% AI, -3.83% RA, -2.79% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions.

# Proposal

BDPCM method proposed in [1], uses reconstructed samples to predict the rows or columns line by line. The signalled BDPCM direction indicates whether vertical or horizontal prediction is used. The reference pixels used are unfiltered samples. The prediction error is quantized in the spatial domain. Pixels are resonstructed by adding the dequantized prediction error to the prediction.

As an alternative scheme to BDPCM, quantized residual domain BDPCM is proposed. The signalling and prediction directions used are identical BPCM scheme [1]. The intra prediction is done on the entire block by sample copying in prediction direction (horizontal or vertical prediction) similar to intra prediction. The residual is quantized and the delta between the quantized residual and its predictor (horizontal or vertical) quantized value is coded. This can be described by the following:

For a block of size M (rows) × N (cols),  let ri,j, 0≤i≤M-1, 0≤j≤N-1rij, 0≤i≤M-1, 0≤j≤(N-1) be the prediction residual after performing intra prediction horizontally (copying left neighbor pixel value across the the predicted block line by line) or vertically (copying top neighbor line to each line in the predicted block) using unfiltered samples from above or left block boundary samples. Let Q(ri,j), 0≤i≤M-1, 0≤j≤N-1 denote the quantized version of the residual ri,j, where residual is difference between original block and the predicted block values. Then the block DPCM is applied to the quantized residual samples, resulting in modified M × N array R with elements ri,j. When vertical BDPCM is signalled:

ri,j=Q(ri,j),i=0,  0≤j≤(N-1)Q(ri,j)-Q(r(i-1),j),1≤i≤(M-1),  0≤j≤(N-1).

For horizontal prediction, similar rules apply, and the residual quantized samples are obtained by

ri,j=Q(ri,j), 0≤i≤M-1,  j=0Q(ri,j)-Q(ri,(j-1)),0≤i≤(M-1),  1≤j≤(N-1).

The residual quantized samples ri,j are sent to the decoder.

On the decoder side, the above calculations are reversed to produce Q(ri,j), 0≤i≤M-1, 0≤j≤N-1Q(rij), 0≤i≤M-1, 0≤j≤(N-1). For vertical prediction case,

Q(ri,j)=k=0irk,j,0≤i≤(M-1),  0≤j≤(N-1).

For horizontal case,

Q(ri,j)=k=0jri,k,0≤i≤(M-1),  0≤j≤(N-1).

The inverse quantized quantized residuals, Q-1Qri,j, are added to the intra block prediction values to produce the reconstructed sample values.

The main benefit of this scheme is that the inverse DPCM can be done on the fly during coefficient parsing simply adding the predictor as the coefficients are parsed or it can be performed after parsing. The splitting of 4xN and Nx4 blocks into 2 parallel processed blocks can be eliminated [1].

# Transform skip residual coding modification for BDPCM mode

If the dependency of contexts and Rice parameter derivation template across coefficient groups is eliminated for coding of sig_coeff_flag and abs_remainder_coding. Basically, the samples across the CG boundary is treated as unavailable for sig_coeff_flag coding context derivation and template used for Rice paraemeter derivation. This enables overwriting of parsed residual values with their reconstructed values as they are parsed.

# Results

The proposed scheme is implemented on top of CE8-3.1 software. The following settings are used for simulations [2], i.e. CE8-5.3.1a and CE8-5.3b [1].

Table 1: test configurations

|  | BDPCM residual coding | Transform skip residual coding | CCB limitation |
| --- | --- | --- | --- |
| Proposed BDPCM (1) | JVET-M0464 | JVET-M0464 | 2 |
| Proposed BDPCM + TS residual coding modification (1+2) | JVET-M0464 | JVET-M0464 | 3 |

The simulations were carried out under the JVET common test conditions [5] (IBC + HashME ON for Class F only) and in addition TGM class (IBC+HashME ON).

The test results for proposed BDPCM scheme (1) under JVET common test conditions with TGM class is tabulated in Table 2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.30% | 0.42% | 108% | 110% |
| Class A2 | 0.02% | 0.07% | -0.01% | 106% | 110% |
| Class B | -0.03% | 0.02% | 0.05% | 107% | 100% |
| Class C | -0.33% | -0.30% | -0.25% | 97% | 100% |
| Class E | -0.11% | -0.07% | -0.08% | 93% | 97% |
| Overall | -0.15% | -0.01% | 0.01% | 102% | 103% |
| Class D | -0.30% | -0.26% | -0.31% | 93% | 105% |
| Class F | -3.91% | -2.40% | -2.47% | 92% | 97% |
| Class SCC | -5.98% | -3.37% | -3.38% | 125% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.24% | 0.06% | 103% | 112% |
| Class A2 | 0.01% | 0.10% | 0.03% | 103% | 113% |
| Class B | 0.05% | 0.08% | -0.16% | 87% | 99% |
| Class C | -0.01% | -0.17% | -0.20% | 95% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | -0.09% | 95% | 104% |
| Class D | 0.15% | -0.67% | -0.67% | 90% | 98% |
| Class F | -2.68% | -1.82% | -1.97% | 98% | 99% |
| Class SCC | -3.57% | -1.36% | -1.29% | 123% | 113% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.11% | 0.48% | -0.01% | 98% | 99% |
| Class E | 0.01% | 0.99% | 0.76% | 92% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.34% | 0.24% | -0.04% | 90% | 97% |
| Class F | -1.99% | -1.25% | -1.56% | 94% | 102% |
| Class SCC | -2.73% | -1.64% | -1.59% | 117% | 117% |

Table 2: Average Bjøntegaard delta rate savings of the proposed method (1) (2bin/sample) relative to VTM-4.0.

The test results for proposed BDPCM scheme with TS modification (1 + 2) under JVET common test conditions with TGM class is tabulated in Table 3.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.26% | 0.44% | 109% | 112% |
| Class A2 | 0.03% | 0.15% | -0.02% | 106% | 119% |
| Class B | -0.03% | -0.02% | 0.09% | 89% | 98% |
| Class C | -0.35% | -0.33% | -0.26% | 87% | 98% |
| Class E | -0.12% | 0.03% | -0.03% | 98% | 99% |
| Overall | -0.15% | -0.01% | 0.03% | 96% | 103% |
| Class D | -0.32% | -0.13% | -0.31% | 97% | 102% |
| Class F | -4.04% | -2.46% | -2.62% | 86% | 93% |
| Class SCC | -6.33% | -3.64% | -3.68% | 114% | 111% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.20% | 0.20% | 108% | 120% |
| Class A2 | 0.00% | -0.05% | -0.11% | 107% | 119% |
| Class B | 0.05% | -0.09% | -0.17% | 84% | 98% |
| Class C | 0.00% | -0.07% | -0.22% | 84% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.10% | 93% | 106% |
| Class D | 0.14% | -0.53% | -0.31% | 84% | 96% |
| Class F | -2.92% | -1.97% | -1.96% | 84% | 99% |
| Class SCC | -3.83% | -1.51% | -1.46% | 112% | 114% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.15% | 0.48% | 0.22% | 83% | 98% |
| Class E | 0.09% | -0.17% | 0.94% | 84% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.41% | 0.71% | 0.60% | 81% | 93% |
| Class F | -2.09% | -2.12% | -2.28% | 85% | 100% |
| Class SCC | -2.79% | -1.70% | -1.62% | 113% | 117% |

Table 3: Average Bjøntegaard delta rate savings of the proposed method (1+2) (3bin/sample) relative to VTM-4.0.

Addional results for method 1 for 3bin/sample configuration and method 2 with 2bin/sample configurations are attached.

The measured runtimes reported are not accurate.

# Conclusions

This document presented quantized residual domainBDPCM in quantized residual domain. BDPCM signalling and prediction directions are same as the methods utilized in sample domain BDPCM. In addition a modification to allow independent parsing of coefficient groups for transform skip residual coding for BDPCM was described.

# References

- F. Henry, M. Abdoli, P. Philippe, G. Clare, “CE8: BDPCM with horizontal/vertical predictor and independently decodable areas (test 8.3.1b)”, JVET-M0057, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE 8): Screen content coding,” JVET-M1028, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- https://vcgit.hhi.fraunhofer.de/jvet-m-ce8/VVCSoftware_VTM/tree/CE8-3.1

- “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).