| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0414-v3 |
| --- | --- |

| Title: | Modifications to support the YUV 4:4:4 chroma format |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Yung-Hsuan Chao, Wei-Jung Chien, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents a few changes to VVC coding tools so that YUV 4:4:4 chroma format can be supported. The tool of CCLM mode and affine mode are modified. Simulations are performed on HEVC RExt and SCC test sequences ( RExt natural sequences and TGM444 sequences). When compared with HEVC RExt (IBC is disabled for VTM), the BD rate reduction of VTM with the proposed changes are -20.29% (AI), -31.05 (RA) and -30.50%(LB), respectively. Comparing with HEVC SCC (IBC is enabled for VTM), the performance of VTM with the proposed changes are 30.37%, 5.51% and -7.28% on TGM 444 sequences, respectively.

It is also proposed to unify the operations defined for chroma components in the case of YUV4:4:4 and YUV4:2:0. The triangular partition mode and intra reference sample smoothing of YUV4:4:4 chroma components are proposed to be aligned with that are defined for YUV4:2:0 chroma components. The performance change introduced by the unification is -0.01% (AI), 0.01% (RA) and 0.02% (LB).

# Introduction

## CCLM mode

CCLM (Cross-Component Linear Model) is a coding tool of VVC draft 4. The mode uses luma reconstructed samples to predict chroma samples by using a linear model:

predCi,j=α⋅recL'i,j+β

where recL'i,j is a luma sample values obtained from the reconstructed luma block. The parameters α and β are derived from the reconstructed luma and chroma samples from the top and left neighbouring blocks. For YUV 4:2:0 format, each chroma sample has 4 collocated luma samples, so both luma samples used to derive α  and β and the samples used for calculating predicted chroma samples are down sampled (Fig. 1).

Fig. 1 Luma and Chroma pixels involved in CCLM mode in YUV4:2:0[2]

## Affine mode

In VVC draft 4, for affine mode, the motion vectors of a chroma 4x4 block is defined based on the average motion vector of 2 luma 4x4 blocks (Fig. 2).

Fig. 2 Motion vector derivation of affine chroma block

# Technical description

## CCLM mode

For CCLM mode in the case of YUV 4:4:4 format, the samples subsampling is removed since luma and chroma CBs have the same size.

Fig. 3 Locations of samples used for CCLM mode for YUV 4:4:4 format

## Affine mode

For Affine mode, in the case of YUV 4:4:4 format, for each chroma 4x4 block, it is proposed to use the motion vector(s) of the collocated luma block to perform affine prediction mode.

Fig. 4 Mv derivation for chroma 4x4 block of YUV 4:4:4 format

## Unification for TPM mode and intra reference sample smoothing.

In the current VVC design, when doing pixel blending for TPM mode and intra smoothing, the operations defined for the luma component in the YUV4:2:0 case is applied for chroma components of YUV4:4:4 input. Meanwhile for other tools chroma handling for YUV4:4:4 and YUV4:2:0 are same.

In this contribution, it is proposed to unify the chroma component operation. More specifically, for TPM pixel blending and intra reference sample smoothing, the operations defined for YUV4:2:0 chroma components are applied also to YUV4:4:4 chroma format.

## Software fixes

In DMVR processing, the calculation of chroma pixel position assumed the chroma format was YUV4:2:0, in the case of YUV4:4:4, wrong chroma pixel positions will be calculated. It is proposed to fix this issue by follow the value of ComponentScaleX and ComponentScaleY when calculating the chroma positions.

In the functions to calculate SSE values such as xGetSSE_WTD, the ‘cShift’ value which used to calculate luma pixel positions and step size of pixel lines also assume the chroma format is 4:2:0. It is proposed also to follow the value of ComponentScaleX and ComponentScaleY in such calculation.

# Simulation results

## Compared with HM16.18 RExt

The anchor is HM-16.18+SCM8.7 ran with ‘rext’ cfg files. On top of the default configurations, internal bit-depth was set to 10 for HM. The results are reported in the Table 1

Table 1 Simulation results when compared with HM-16.18 RExt

## Compared with HM16.18 SCC

The anchor is HM-16.18+SCM8.7 ran with ‘scc’ cfg files. On top of the default configurations, internal bit-depth was set to 10 and the search range of IBC was set to 1x4 CTUs for HM. The results are reported in the Table 2.

Table 2 Simulation results when compared with HM-16.18 SCC

## Unification of chroma component handling

In this test, the anchor is VTM code with the changes described in section 2.1, 2.2 and 2.4. And VTM code with all the changes described in section 2.1~2.4 are tested. The results are reported in Table 3.

Table 3 Simulation results of the unification of chroma component handling

# Conclusion

This contribution presents several changes to VVC coding tools so that YUV 4:4:4 chroma format can be supported. Simulations are performed on HEVC RExt and SCC test sequences (RExt natural sequences and TGM444). When compared with HEVC RExt (IBC is disabled for VTM), the performance of VTM with the proposed changes are -20.29% (AI), -31.05 (RA) and -30.50%(LB), respectively. And when compared with HEVC SCC (IBC is enabled for VTM), the performance of VTM with the proposed changes are 30.37%, 5.51% and -7.28% on TGM 444 sequences, respectively.

It is also proposed to unify the operations defined for chroma components in the case of YUV4:4:4 and YUV4:2:0. The triangular partition mode and intra reference sample smoothing of YUV4:4:4 chroma components are proposed to be aligned with that are defined for YUV4:2:0 chroma components. The performance change introduced by the unification is -0.01% (AI), 0.01% (RA) and 0.02% (LB).

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1001, Jan. 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4)” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M1002, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).