| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0415-v2 |
| --- | --- |

| Title: | CE5: Coding tree block based adaptive loop filter (CE5-4) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Hilmi Enes Egilmez, Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | +1 858-651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, the coding tree block (CTB) based ALF scheme proposed in JVET-K0382, JVET-L0391 and JVET-M0429 was tested in VTM-4.0. Test results reportedly show 0.15%, 0.52% and 0.62% luma gain with similar encoding and decoding time, in AI, RA and LDB configuration respectively over VTM-4.0 anchor.

# Introduction

In this contribution, the coding tree block (CTB) based ALF scheme proposed in JVET-K0382, JVET-L0391 and JVET-M0429 was tested and results are reported.

# Algorithm description

## Slice-level temporal filter

Adaptive parameter set (APS) was adopted in VTM4. Each APS contains one set of signalled ALF filters, up to 32 APSs are supported. In the proposal, slice-level temporal filter is tested. A tile group can re-use the ALF information from an APS to reduce the overhead. The APSs are updated as a first-in-first-out (FIFO) buffer.

## CTB based ALF

For luma component, when ALF is applied to a luma CTB, the choice among 16 fixed, 5 temporal or 1 signaled filter sets is indicated. Only the filter set index is signalled. For one slice, only one new set of 25 filters can be signaled. If a new set is signalled for a slice, all the luma CTBs in the same slice share that set. Fixed filter sets can be used to predict the new slice-level filter set and can be used as candidate filter sets for a luma CTB as well. The number of filters is 64 in total.

For chroma component, when ALF is applied to a chroma CTB, if a new filter is signalled for a slice, the CTB used the new filter, otherwise, the most recent temporal chroma filter satisfying the temporal scalability constrain is applied.

As the slice-level temporal filter, the APSs are updated as a first-in-first-out (FIFO) buffer.

# Experimental results

The proposed method was implemented in VTM-4.0 software [1] with common test configuration [2]. In addition, smaller QPs (12 and 17) are used for evaluation under high rate case. Results are summarized in below tables and can be found in detail in the accompanies Excel tables.

## CE5-4.1: Slice-level temporal filter

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | 0.02% | 0.00% | 101% | 99% |
| Class A2 | -0.12% | -0.06% | -0.30% | 99% | 100% |
| Class B | -0.15% | -0.08% | -0.11% | 100% | 100% |
| Class C | -0.23% | -0.38% | -0.28% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.14% | -0.17% | 100% | 100% |
| Class D | -0.46% | -0.54% | -0.49% | 101% | 100% |
| Class F | -0.13% | -0.10% | -0.14% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.22% | -0.08% | -0.06% | 100% | 100% |
| Class C | -0.41% | -0.61% | -0.39% | 101% | 100% |
| Class E | -0.60% | 0.18% | 0.36% | 100% | 99% |
| Overall | -0.38% | -0.19% | -0.07% | 101% | 100% |
| Class D | -0.61% | -0.75% | -1.53% | 99% | 101% |
| Class F | -0.42% | -0.23% | -1.26% | 100% | 101% |

## CE5-4.2: CTB based ALF (Fixed filters + temporal filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.02% | 99% | 101% |
| Class A2 | -0.08% | 0.02% | 0.02% | 100% | 101% |
| Class B | -0.12% | -0.02% | -0.02% | 100% | 100% |
| Class C | -0.18% | -0.11% | -0.11% | 100% | 99% |
| Class E | -0.29% | -0.12% | -0.13% | 100% | 99% |
| Overall | -0.15% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.23% | -0.20% | -0.20% | 101% | 100% |
| Class F | -0.13% | -0.07% | -0.07% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.37% | 0.07% | 0.02% | 100% | 100% |
| Class A2 | -0.57% | 0.06% | -0.17% | 101% | 101% |
| Class B | -0.54% | -0.22% | -0.20% | 99% | 101% |
| Class C | -0.59% | -0.35% | -0.42% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.52% | -0.14% | -0.21% | 100% | 101% |
| Class D | -0.61% | -0.76% | -0.84% | 101% | 100% |
| Class F | -0.26% | -0.11% | -0.14% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.51% | -0.08% | -0.37% | 100% | 99% |
| Class C | -0.77% | -0.26% | -0.09% | 100% | 100% |
| Class E | -0.59% | -0.60% | -0.52% | 101% | 100% |
| Overall | -0.62% | -0.27% | -0.32% | 100% | 100% |
| Class D | -0.74% | -0.16% | -0.68% | 101% | 100% |
| Class F | -0.64% | -0.20% | -0.63% | 100% | 101% |

## CE5-4.3: CTB based ALF (New filters + fixed filters + temporal filters) with QP=12, 17, 22 and 27

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.01% | 101% | 100% |
| Class A2 | -0.03% | 0.00% | 0.00% | 99% | 99% |
| Class B | -0.02% | 0.00% | 0.00% | 99% | 100% |
| Class C | -0.02% | -0.01% | -0.02% | 100% | 98% |
| Class E | -0.02% | -0.01% | -0.01% | 99% | 99% |
| Overall | -0.02% | 0.00% | 0.00% | 99% | 99% |
| Class D | -0.04% | -0.03% | -0.03% | 100% | 100% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.05% | -0.04% | 100% | 100% |
| Class A2 | -0.26% | -0.04% | -0.11% | 99% | 99% |
| Class B | -0.27% | -0.16% | -0.29% | 100% | 101% |
| Class C | -0.28% | -0.23% | -0.33% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.11% | -0.22% | 99% | 100% |
| Class D | -0.30% | -0.60% | -0.57% | 101% | 100% |
| Class F | -0.14% | -0.13% | -0.12% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.24% | -0.12% | -0.27% | 101% | 100% |
| Class C | -0.27% | -0.29% | -0.32% | 100% | 99% |
| Class E | -0.38% | -0.41% | -0.64% | 100% | 100% |
| Overall | -0.29% | -0.25% | -0.38% | 100% | 100% |
| Class D | -0.30% | -0.33% | -0.52% | 99% | 99% |
| Class F | -0.28% | -0.11% | -0.16% | 100% | 100% |

## CE5-4.4: CTB based ALF (New filters + temporal filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.03% | 0.07% | 100% | 101% |
| Class A2 | -0.36% | -0.05% | -0.23% | 101% | 101% |
| Class B | -0.35% | -0.23% | -0.17% | 100% | 100% |
| Class C | -0.38% | -0.44% | -0.36% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.20% | -0.19% | 100% | 100% |
| Class D | -0.48% | -0.91% | -0.91% | 101% | 100% |
| Class F | -0.18% | -0.16% | -0.18% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.40% | -0.16% | 101% | 99% |
| Class C | -0.63% | -0.36% | -0.39% | 99% | 99% |
| Class E | -0.59% | -0.98% | -0.23% | 98% | 101% |
| Overall | -0.53% | -0.53% | -0.26% | 100% | 99% |
| Class D | -0.66% | -0.16% | -1.69% | 100% | 100% |
| Class F | -0.62% | -0.28% | -0.55% | 100% | 100% |

## CE5-4.5: CTB based ALF (New filters + fixed filters)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.02% | 100% | 100% |
| Class A2 | -0.08% | 0.02% | 0.02% | 98% | 99% |
| Class B | -0.12% | -0.02% | -0.02% | 99% | 101% |
| Class C | -0.18% | -0.11% | -0.11% | 101% | 100% |
| Class E | -0.29% | -0.12% | -0.13% | 100% | 99% |
| Overall | -0.15% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.23% | -0.20% | -0.20% | 100% | 101% |
| Class F | -0.13% | -0.07% | -0.07% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.25% | 0.18% | 0.13% | 100% | 99% |
| Class A2 | -0.39% | 0.06% | 0.05% | 100% | 100% |
| Class B | -0.35% | 0.03% | 0.04% | 99% | 99% |
| Class C | -0.39% | -0.08% | -0.15% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | 0.04% | 0.01% | 100% | 100% |
| Class D | -0.18% | -0.12% | -0.26% | 99% | 100% |
| Class F | -0.16% | 0.01% | -0.04% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.18% | 0.11% | 101% | 101% |
| Class C | -0.48% | 0.14% | 0.26% | 99% | 101% |
| Class E | -0.57% | -0.53% | -0.66% | 101% | 100% |
| Overall | -0.43% | -0.16% | -0.03% | 100% | 100% |
| Class D | -0.37% | -0.31% | -0.44% | 101% | 98% |
| Class F | -0.40% | -0.30% | -0.57% | 100% | 100% |

## CE5-4.6: CTB based ALF (New filters + fixed filters) with QP=12, 17, 22 and 27

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.01% | 100% | 99% |
| Class A2 | -0.03% | 0.00% | 0.00% | 100% | 99% |
| Class B | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.02% | -0.01% | -0.02% | 98% | 99% |
| Class E | -0.02% | -0.01% | -0.01% | 99% | 100% |
| Overall | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Class D | -0.04% | -0.03% | -0.03% | 101% | 101% |
| Class F | -0.02% | -0.01% | -0.01% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.07% | 0.07% | 99% | 98% |
| Class A2 | -0.12% | 0.06% | 0.04% | 99% | 100% |
| Class B | -0.16% | 0.01% | -0.04% | 100% | 100% |
| Class C | -0.12% | 0.00% | -0.08% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.03% | -0.02% | 99% | 99% |
| Class D | -0.09% | -0.22% | -0.13% | 101% | 100% |
| Class F | -0.06% | 0.01% | 0.00% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | -0.02% | -0.05% | 100% | 100% |
| Class C | -0.11% | -0.09% | -0.03% | 98% | 101% |
| Class E | -0.27% | -0.39% | -0.46% | 100% | 100% |
| Overall | -0.15% | -0.14% | -0.15% | 99% | 100% |
| Class D | -0.06% | 0.06% | 0.05% | 100% | 101% |
| Class F | -0.22% | -0.26% | -0.33% | 100% | 100% |

# References

[1] VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring “JVET-M1010: JVET common test conditions and software reference configurations for SDR video”

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Working draft

## CTB-based filter index scheme

The following text is modified based on JVET-K1001-v6 with fixed filter, temporal filters and CTB-based filter index

7.3.3.2 Adaptive loop filter data syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_chroma_idc | tu(v) |
| alf_signal_new_filter_luma | u(1) |
| if( alf_signal_new_filter_luma > 0 ) { |  |
| alf_luma_use_fixed_filter_flag | u(1) |
| if(alf_luma_use_fixed_filter_flag){ |  |
| alf_luma_fixed_filter_set_index | tb(v) |
| alf_luma_fixed_filter_usage_pattern | u(1) |
| if (alf_luma_fixed_filter_usage_pattern > 0) |  |
| for (i = 0; i < NumAlfFilters; i++) |  |
| alf_luma_fixed_filter_usage[i] | u(1) |
| alf_num_available_temporal_filter_sets_luma | tb(1) |
| alf_luma_num_filters_signalled_minus1 | tb(v) |
| alf_luma_type_flag | u(1) |
| if( alf_luma_num_filters_signalled_minus1 > 0 ) { |  |
| for( filtIdx = 0; filtIdx < NumAlfFilters; filtIdx++ ) |  |
| alf_luma_coeff_delta_idx[ filtIdx ] | tu(v) |
| } |  |
| alf_luma_coeff_delta_flag | u(1) |
| if ( !alf_luma_coeff_delta_flag && alf_luma_num_filters_signalled_minus1 > 0 ) |  |
| alf_luma_coeff_delta_prediction_flag | u(1) |
| alf_luma_min_eg_order_minus1 | tu(v) |
| for( i = 0; i < ( alf_luma_type_flag = = 1 ) ? 2 : 3; i++ ) |  |
| alf_luma_eg_order_increase_flag[ i ] | u(1) |
| if ( alf_luma_coeff_delta_flag ) { |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++ ) |  |
| alf_luma_coeff_flag[ sigFiltIdx ] | u(1) |
| } |  |
| for( sigFiltIdx = 0; sigFiltIdx <= alf_luma_num_filters_signalled_minus1; sigFiltIdx++){ |  |
| if ( alf_luma_coeff_flag[ sigFiltIdx ] ) { |  |
| for ( j = 0; j < ( alf_luma_type_flag = = 1 ) ? 6 : 12; j++ ) { |  |
| alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] | uek(v) |
| if( alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] ) |  |
| alf_luma_coeff_delta_sign[ sigFiltIdx ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| if ( alf_chroma_idc > 0 ) { |  |
| alf_signal_new_filter_chroma | u(1) |
| if (alf_signal_new_filter_chroma){ |  |
| alf_chroma_min_eg_order_minus1 | tu(v) |
| for( i = 0; i < 2; i++ ) |  |
| alf_chroma_eg_order_increase_flag[ i ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ j ] | uek(v) |
| if( alf_chroma_coeff_abs[ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.4.2 Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_alf_enable_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if (alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] { |  |
| coding_alf_ctu_filter_set_index(CtbAddrInRs ) |  |
| } |  |
| if( alf_chroma_idc = = 1 ¦ ¦ alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 1 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if( alf_chroma_idc = = 2 ¦ ¦ alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 2 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) { |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbLog2SizeY, 0 ) |  |
| else |  |
| coding_quadtree( xCtb, yCtb, CtbLog2SizeY, 0, SINGLE_TREE ) |  |
| } |  |

| coding_alf_ctu_filter_set_index(CtbAddrInRs, slice_type ){ | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| alf_use_new_filter | ae(1) |
| if (alf_use_new_filter == 0){ |  |
| alf_use_fixed_filter | ae(1) |
| } |  |
| if (alf_use_new_filter){ |  |
| alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] = 16 |  |
| } |  |
| else if (alf_use_fixed_filter){ |  |
| alf_fixed_filter_index | tb(v) |
| alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] = alf_fixed_filter_index |  |
| } |  |
| else{ |  |
| alf_temporal_index | tb(v) |
| alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] = 16 + alf_temporal_index |  |
| } |  |

7.4.4.2 Adaptive loop filter data semantics

alf_signal_new_filter_luma equal to 1 specifies a new luma filter set is signalled. alf_signal_new_filter_luma equal to 0 specifies a new luma filter set is not signalled. It is 0 when not present.

alf_luma_use_fixed_filter_flag equal to 1 specifies that fixed filter set is used to signal adaptive loop filter. alf_luma_use_fixed_filter_flag equal to 0 specifies that fixed filter set is not used to signal adaptive loop filter.

alf_luma_fixed_filter_set_index specifies the fixed filter set index. It can be 0…15.

alf_luma_fixed_filter_usage_pattern equal to 0 specifies all new filters use fixed filter. alf_luma_fixed_filter_usage_pattern equal to 1 specifies some of new filters use fixed filter and others don’t.

alf_luma_fixed_filter_usage[i] equal to 1 specifies ith filter uses fixed filter. alf_luma_fixed_filter_usage[i] equal to 0 specifies ith filter does not use fixed filter. When it is not present, it is inferred to 1.

alf_signal_new_filter_chroma equal to 1 specifies a new chroma filter is signalled. alf_signal_new_filter_chroma equal to 0 specifies a new chroma filter is not signalled.

alf_num_available_temporal_filter_sets_luma specifies the number of available temporal filter sets that can be used for current slice, it can be from 0..5. It is 0 when not present.

The variable alf_num_available_filter_sets is derived as 16 + alf_signal_new_filter_luma + alf_num_available_temporal_filter_sets_luma.

If alf_signal_new_filter_luma is 1, the following process

The variable filterCoefficients[ sigFiltIdx ][ j ] with sigFiltIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filterCoefficients[ sigFiltIdx ][ j ] = alf_luma_coeff_delta_abs[ sigFiltIdx ][ j ] * 	(7-50)
	( 1 − 2 * alf_luma_coeff_delta_sign[ sigFiltIdx ][ j ] )

When alf_luma_coeff_delta_prediction_flag is equal 1, filterCoefficients[ sigFiltIdx ][ j ] with sigFiltIdx = 1..alf_luma_num_filters_signalled_minus1 and j = 0..11 are modified as follows:

filterCoefficients[ sigFiltIdx ][ j ] += filterCoefficients[ sigFiltIdx − 1 ][ j ] 	(7-51)

The luma filter coefficients AlfCoeffL with elements AlfCoeffL[ filtIdx ][ j ], with filtIdx = 0..NumAlfFilters − 1 and j = 0..11 are derived as follows

AlfCoeffL[ filtIdx ][ j ] = filterCoefficients[ alf_luma_coeff_delta_idx[ filtIdx ] ][ j ]	(7-52)

if alf_luma_use_fixed_filter_flag is 1 and alf_luma_fixed_filter_usage[ filtidx ] is 1 , the following applies:

AlfCoeffL[filtIdx ][ j ] = AlfCoeffL[filtIdx ][ j ] +   AlfFixedFilterCoeff[ AlfClassToFilterMapping[ alf_luma_fixed_filter_index ][ filtidx ] ][ j ]

The last filter coefficients AlfCoeffL[ filtIdx ][ 12 ] for filtIdx = 0..NumAlfFilters − 1 are derived as follows:

AlfCoeffL[ filtIdx ][ 12 ] = 128 − k ( AlfCoeffL[ filtIdx ][ k ]  <<  1 ), with k = 0..11	(7-53)

It is a requirement of bitstream conformance that the values of AlfCoeffL[ filtIdx ][ j ] with filtIdx = 0..NumAlfFilters − 1, j = 0..11 shall be in the range of −27 to 27 − 1, inclusive and that the values of AlfCoeffL[ filtIdx ][ 12 ] shall be in the range of 0 to 28 − 1, inclusive.

The luma filter coefficients AlfCoeffLumaAll with elements AlfCoeffLumaAll[ filtSetIdx ][ filtIdx ][ j ], with filtSetIdx = 0..15, filtSetIdx = 0..NumAlfFilters – 1 and j = 0..12 are derived as follows

AlfCoeffLumaAll[ filtSetIdx ][ filtIdx ][ j ] = AlfFixedFilterCoeff[ AlfClassToFilterMapping[ filtSetIdx ][ filtidx ] ][ j ]

The luma filter coefficients AlfCoeffLumaAll with elements AlfCoeffLumaAll[ filtSetIdx ][ filtIdx ][ j ], with filtSetIdx = 16, filtSetIdx = 0..NumAlfFilters – 1 and j = 0..12 are derived as follows

Variable closest_temporal_index is initialized as -1. Tid is the temporal layer index of current slice.

if alf_signal_new_filter_luma is 1

AlfCoeffLumaAll[ 16 ][ filtIdx ][ j ] = AlfCoeffL[ filtIdx ][ j ]

otherwise, the following process is invoked

for (i = Tid; i >= 0; i--)

{

for (k = 0; k < temp_size_L; k++)

{

if (tempTid_L[ k ] == i)

{

closest_temporal_index is set as k;

break;

}

}

}

AlfCoeffLumaAll[ 16 ][ filtIdx ][ j ] = TempL[ closest_temporal_index ][ filtIdx ][ j ]

The luma filter coefficients AlfCoeffLumaAll with elements AlfCoeffLumaAll[ filtSetIdx ][ filtIdx ][ j ], with filtSetIdx = 17.. alf_num_available_filter_sets-1, filtSetIdx = 0..NumAlfFilters – 1 and j = 0..12 are derived as follows

i = 17;

for (k = 0; k < temp_size_L and i < alf_num_available_filter_sets; j++)

{

if (tempTid_L[ k ] <= Tid and k is not equal to closest_temporal_index)

{

AlfCoeffLumaAll[ i ][ filtIdx ][ j ] = TempL[ k ][ filtIdx ][ j ];

i++;

}

}

AlfFixedFilterCoeff [64][13] =

{

{ 0,   0,   2,  -3,   1,  -4,   1,   7,  -1,   1,  -1,   5, 112 },

{ 0,   0,   0,   0,   0,  -1,   0,   1,   0,   0,  -1,   2, 126 },

{ 0,   0,   0,   0,   0,   0,   0,   1,   0,   0,   0,   0, 126 },

{ 0,   0,   0,   0,   0,   0,   0,   0,   0,   0,  -1,   1, 128 },

{ 2,   2,  -7,  -3,   0,  -5,  13,  22,  12,  -3,  -3,  17,  34 },

{-1,   0,   6,  -8,   1,  -5,   1,  23,   0,   2,  -5,  10,  80 },

{ 0,   0,  -1,  -1,   0,  -1,   2,   1,   0,   0,  -1,   4, 122 },

{ 0,   0,   3, -11,   1,   0,  -1,  35,   5,   2,  -9,   9,  60 },

{ 0,   0,   8,  -8,  -2,  -7,   4,   4,   2,   1,  -1,  25,  76 },

{ 0,   0,   1,  -1,   0,  -3,   1,   3,  -1,   1,  -1,   3, 122 },

{ 0,   0,   3,  -3,   0,  -6,   5,  -1,   2,   1,  -4,  21,  92 },

{-7,   1,   5,   4,  -3,   5,  11,  13,  12,  -8,  11,  12,  16 },

{-5,  -3,   6,  -2,  -3,   8,  14,  15,   2,  -7,  11,  16,  24 },

{ 2,  -1,  -6,  -5,  -2,  -2,  20,  14,  -4,   0,  -3,  25,  52 },

{ 3,   1,  -8,  -4,   0,  -8,  22,   5,  -3,   2, -10,  29,  70 },

{ 2,   1,  -7,  -1,   2, -11,  23,  -5,   0,   2, -10,  29,  78 },

{-6,  -3,   8,   9,  -4,   8,   9,   7,  14,  -2,   8,   9,  14 },

{ 2,   1,  -4,  -7,   0,  -8,  17,  22,   1,  -1,  -4,  23,  44 },

{ 3,   0,  -5,  -7,   0,  -7,  15,  18,  -5,   0,  -5,  27,  60 },

{ 2,   0,   0,  -7,   1, -10,  13,  13,  -4,   2,  -7,  24,  74 },

{ 3,   3, -13,   4,  -2,  -5,   9,  21,  25,  -2,  -3,  12,  24 },

{-5,  -2,   7,  -3,  -7,   9,   8,   9,  16,  -2,  15,  12,  14 },

{ 0,  -1,   0,  -7,  -5,   4,  11,  11,   8,  -6,  12,  21,  32 },

{ 3,  -2,  -3,  -8,  -4,  -1,  16,  15,  -2,  -3,   3,  26,  48 },

{ 2,   1,  -5,  -4,  -1,  -8,  16,   4,  -2,   1,  -7,  33,  68 },

{ 2,   1,  -4,  -2,   1, -10,  17,  -2,   0,   2, -11,  33,  74 },

{ 1,  -2,   7, -15, -16,  10,   8,   8,  20,  11,  14,  11,  14 },

{ 2,   2,   3, -13, -13,   4,   8,  12,   2,  -3,  16,  24,  40 },

{ 1,   4,   0,  -7,  -8,  -4,   9,   9,  -2,  -2,   8,  29,  54 },

{ 1,   1,   2,  -4,  -1,  -6,   6,   3,  -1,  -1,  -3,  30,  74 },

{-7,   3,   2,  10,  -2,   3,   7,  11,  19,  -7,   8,  10,  14 },

{ 0,  -2,  -5,  -3,  -2,   4,  20,  15,  -1,  -3,  -1,  22,  40 },

{ 3,  -1,  -8,  -4,  -1,  -4,  22,   8,  -4,   2,  -8,  28,  62 },

{ 0,   3, -14,   3,   0,   1,  19,  17,   8,  -3,  -7,  20,  34 },

{ 0,   2,  -1,  -8,   3,  -6,   5,  21,   1,   1,  -9,  13,  84 },

{-4,  -2,   8,  20,  -2,   2,   3,   5,  21,   4,   6,   1,   4 },

{ 2,  -2,  -3,  -9,  -4,   2,  14,  16,   3,  -6,   8,  24,  38 },

{ 2,   1,   5, -16,  -7,   2,   3,  11,  15,  -3,  11,  22,  36 },

{ 1,   2,   3, -11,  -2,  -5,   4,   8,   9,  -3,  -2,  26,  68 },

{ 0,  -1,  10,  -9,  -1,  -8,   2,   3,   4,   0,   0,  29,  70 },

{ 1,   2,   0,  -5,   1,  -9,   9,   3,   0,   1,  -7,  20,  96 },

{-2,   8,  -6,  -4,   3,  -9,  -8,  45,  14,   2, -13,   7,  54 },

{ 1,  -1,  16, -19,  -8,  -4,  -3,   2,  19,   0,   4,  30,  54 },

{ 1,   1,  -3,   0,   2, -11,  15,  -5,   1,   2,  -9,  24,  92 },

{ 0,   1,  -2,   0,   1,  -4,   4,   0,   0,   1,  -4,   7, 120 },

{ 0,   1,   2,  -5,   1,  -6,   4,  10,  -2,   1,  -4,  10, 104 },

{ 3,   0,  -3,  -6,  -2,  -6,  14,   8,  -1,  -1,  -3,  31,  60 },

{ 0,   1,   0,  -2,   1,  -6,   5,   1,   0,   1,  -5,  13, 110 },

{ 3,   1,   9, -19, -21,   9,   7,   6,  13,   5,  15,  21,  30 },

{ 2,   4,   3, -12, -13,   1,   7,   8,   3,   0,  12,  26,  46 },

{ 3,   1,  -8,  -2,   0,  -6,  18,   2,  -2,   3, -10,  23,  84 },

{ 1,   1,  -4,  -1,   1,  -5,   8,   1,  -1,   2,  -5,  10, 112 },

{ 0,   1,  -1,   0,   0,  -2,   2,   0,   0,   1,  -2,   3, 124 },

{ 1,   1,  -2,  -7,   1,  -7,  14,  18,   0,   0,  -7,  21,  62 },

{ 0,   1,   0,  -2,   0,  -7,   8,   1,  -2,   0,  -3,  24,  88 },

{ 0,   1,   1,  -2,   2, -10,  10,   0,  -2,   1,  -7,  23,  94 },

{ 0,   2,   2, -11,   2,  -4,  -3,  39,   7,   1, -10,   9,  60 },

{ 1,   0,  13, -16,  -5,  -6,  -1,   8,   6,   0,   6,  29,  58 },

{ 1,   3,   1,  -6,  -4,  -7,   9,   6,  -3,  -2,   3,  33,  60 },

{ 4,   0, -17,  -1,  -1,   5,  26,   8,  -2,   3, -15,  30,  48 },

{ 0,   1,  -2,   0,   2,  -8,  12,  -6,   1,   1,  -6,  16, 106 },

{ 0,   0,   0,  -1,   1,  -4,   4,   0,   0,   0,  -3,  11, 112 },

{ 0,   1,   2,  -8,   2,  -6,   5,  15,   0,   2,  -7,   9,  98 },

{ 1,  -1,  12, -15,  -7,  -2,   3,   6,   6,  -1,   7,  30,  50 },

};

AlfClassToFilterMapping [ 16 ][ 25 ] =

{

{ 8,   2,   2,   2,   3,   4,  53,   9,   9,  52,   4,   4,   5,   9,   2,   8,  10,   9,   1,   3,  39,  39,  10,   9,  52 },

{ 11,  12,  13,  14,  15,  30,  11,  17,  18,  19,  16,  20,  20,   4,  53,  21,  22,  23,  14,  25,  26,  26,  27,  28,  10 },

{ 16,  12,  31,  32,  14,  16,  30,  33,  53,  34,  35,  16,  20,   4,   7,  16,  21,  36,  18,  19,  21,  26,  37,  38,  39 },

{ 35,  11,  13,  14,  43,  35,  16,   4,  34,  62,  35,  35,  30,  56,   7,  35,  21,  38,  24,  40,  16,  21,  48,  57,  39 },

{ 11,  31,  32,  43,  44,  16,   4,  17,  34,  45,  30,  20,  20,   7,   5,  21,  22,  46,  40,  47,  26,  48,  63,  58,  10 },

{ 12,  13,  50,  51,  52,  11,  17,  53,  45,   9,  30,   4,  53,  19,   0,  22,  23,  25,  43,  44,  37,  27,  28,  10,  55 },

{ 30,  33,  62,  51,  44,  20,  41,  56,  34,  45,  20,  41,  41,  56,   5,  30,  56,  38,  40,  47,  11,  37,  42,  57,   8 },

{ 35,  11,  23,  32,  14,  35,  20,   4,  17,  18,  21,  20,  20,  20,   4,  16,  21,  36,  46,  25,  41,  26,  48,  49,  58 },

{ 12,  31,  59,  59,   3,  33,  33,  59,  59,  52,   4,  33,  17,  59,  55,  22,  36,  59,  59,  60,  22,  36,  59,  25,  55 },

{ 31,  25,  15,  60,  60,  22,  17,  19,  55,  55,  20,  20,  53,  19,  55,  22,  46,  25,  43,  60,  37,  28,  10,  55,  52 },

{ 12,  31,  32,  50,  51,  11,  33,  53,  19,  45,  16,   4,   4,  53,   5,  22,  36,  18,  25,  43,  26,  27,  27,  28,  10 },

{ 5,   2,  44,  52,   3,   4,  53,  45,   9,   3,   4,  56,   5,   0,   2,   5,  10,  47,  52,   3,  63,  39,  10,   9,  52 },

{ 12,  34,  44,  44,   3,  56,  56,  62,  45,   9,  56,  56,   7,   5,   0,  22,  38,  40,  47,  52,  48,  57,  39,  10,   9 },

{ 35,  11,  23,  14,  51,  35,  20,  41,  56,  62,  16,  20,  41,  56,   7,  16,  21,  38,  24,  40,  26,  26,  42,  57,  39 },

{ 33,  34,  51,  51,  52,  41,  41,  34,  62,   0,  41,  41,  56,   7,   5,  56,  38,  38,  40,  44,  37,  42,  57,  39,  10 },

{ 16,  31,  32,  15,  60,  30,   4,  17,  19,  25,  22,  20,   4,  53,  19,  21,  22,  46,  25,  55,  26,  48,  63,  58,  55 }, };

if alf_signal_new_filter_chroma is 1, the following process

The chroma filter coefficients AlfCoeffC[ j ], with j = 0..5 are derived as follows:

AlfCoeffC[ j ] = alf_chroma_coeff_abs[ j ] * ( 1 − 2 * alf_chroma_coeff_sign[ j ] )	(7-57)

The last filter coefficient for j = 6 is derived as follows:

AlfCoeffC[ 6 ] = 128 − k ( AlfCoeffC[ k ]  <<  1 ), with k = 0..5	(7-58)

It is a requirement of bitstream conformance that the values of AlfCoeffC[ j ] with j = 0..5 shall be in the range of −27 − 1 to 27 − 1, inclusive and that the values of AlfCoeffC[ 6 ] shall be in the range of 0 to 28 − 1, inclusive.

otherwise (alf_signal_new_filter_chroma is 0), the following is invoked

for (i = Tid; i >= 0; i--)

{

for (k = 0; k < temp_size_C; k++)

{

if (tempTid_C[ k ] == i)

{

closest_temporal_index is set as k;

break;

}

}

}

The chroma filter coefficients AlfCoeffC[ j ], with j = 0..6 are derived as follows:

AlfCoeffC[ j ] = TempC[ closest_temporal_index ][ j ]

7.4.5.2 Coding tree unit semantics

alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] specifies the filter set index of luma CTB at location ( xCtb, yCtb ).

alf_use_new_filter is equal to 1 specifies that alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is 16. alf_use_new_filter is equal to 0 specifies that alf_luma_ctb_filter_set_index[xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is not 16.

alf_use_fixed_filter is equal to 1 specifies that one of the fixed filter sets is used. alf_use_fixed_filter is equal to 0 specifies that current luma CTB doesn’t use any fixed filter set.

alf_fixed_filter_index specifies the fixed filter set index, which can be from 0 to 15.

alf_temporal_index specifies the temporal filter set index, which can be from 0 to alf_num_available_temporal_filter_sets_luma - 1.

8.5.1 General

- When sps_alf_enabled_flag is equal to 1, the following applies:

–	The temporal filter refresh process as specified in clause 8.5.4.5 is invoked.

–	The adaptive loop filter process as specified in clause 8.5.4.1 is invoked with the reconstructed picture sample arrays SL, SCb and SCr as inputs, and the modified reconstructed picture sample arrays S′L, S′Cb and S′Cr after sample adaptive offset as outputs.

–	The arrays S′L, S′Cb and S′Cr are assigned to the arrays SL, SCb and SCr (which represent the decoded picture), respectively.

–	The temporal filter update process as specified in clause 8.5.4.6 is invoked.

8.5.4.2 Coding tree block filtering process for luma samples

  - The array of luma filter coefficients f[ j ] corresponding to the filter specified by filtIdx[ x ][ y ] is derived as follows with j = 0..12:

f[ j ] = AlfCoeffLumaAll [ alf_luma_ctb_filter_set_index[ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] ] ][ filtIdx[ x ][ y ] ][ j ]		(8-732)

8.5.4.5 Temporal filter refresh

If any of the below condition is true,

–	Current picture is IDR picture

–	Current picture is BLA picture

–	In decoding order, current picture is the first picture with POC larger than the POC of the last decoded IRAP picture, i.e. after leading pictures and before trailing pictures.

then temp_size_L and temp_size_C are set as 0.

8.5.4.6 Temporal filter update

If slice_alf_enabled_flag is 1 and alf_signal_new_filter_luma is 1, the following applies.

If luma temporal filter buffer size temp_size_L < 5, temp_size_L = temp_size_L + 1.

TempL[ i ][ j ][ k ] with i = temp_size_L – 1 … 1, j = 0… NumAlfFilters-1 and k = 0…12 is updated as,

TempL[ i ][ j ][ k ] = TempL[ i-1 ][ j ][ k ]

TempL[ 0 ][ j ][ k ] with j = 0… NumAlfFilters-1 and k = 0..12 is updated as

TempL[ 0 ][ j ][ k ] = AlfCoeffL[ j ][ k ]

TempTid_L[ i ] with i = temp_size_L – 1…1 is updated as

TempTid_L[ i ] = TempTid_L[ i-1 ]

TempTid_L[ 0 ] is set as current slice’s temporal layer index Tid

if alf_chroma_idx is not 0 and alf_signal_new_filter_chroma is 1, the following applies

Tempc[ i ][ j ] with i = temp_size_c – 1 … 1 and j = 0…6 is updated as,

Tempc[ i ][ j ] = Tempc[ i-1 ][ j ]

Tempc[ 0 ][ j ] with j = 0…6 is updated as

Tempc[ 0 ][ j ] = AlfCoeffC [ j ]

TempTid_C[ i ] with i = temp_size_C – 1…1 is updated as

TempTid_C[ i ] = TempTid_C[ i-1 ]

TempTid_C[ 0 ] is set as current slice’s Tid

Table 9-4 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
|  | alf_use_new_filter_set | FL | cMax = 1 |
|  | alf_used_fixed_filter | FL | cMax = 1 |
|  | alf_fixed_filter_index | TB | cMax = 15 |
|  | alf_temporal_index | TB | cMax = alf_num_available_temporal_filter_sets_luma - 1 |

Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| alf_use_new_filter_set | 0 | na | na | na | na | na |
| alf_used_fixed_filter | 0 | na | na | na | na | na |