| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0418-v1 |
| --- | --- |

| Title: | CE4: Using regular merge candidate list for triangular PU mode (CE4-4.9). |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Vadim Seregin, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | 1-858-658-4535 hongtaow@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE4-4.9. In which, it is proposed to reuse regular merge list construction to derive TPM merge list. After deriving merge candidates based on the regular merge list, a uni-directional MV will used as it is when adding it to the final list, and each bi-MV is split into uni-L0-MV and uni-L1-MV, which are added to the final TPM merge list. Full pruning or no pruning is performed when final uni-directional TPM list is derived. Depending on how regular merge list is used and whether pruning is performed in the second step, there are 4 tests:

- Derive TPM merge list based on the regular merge list, after that, uni-MV is used as it is and bi-MV is split into uni-MVs and pruning is applied.

- Test a) with no pruning applied to uni-directional MVs in the final TPM merge list.

- Merge list is derived based on the spatial and temporal MV candidates from the regular merge list, bi-MV candidate is split into uni-MVs and pruning is applied.

- Test c) with no pruning applied to uni-directional MVs in the final TPM merge list.

Simulation results reportedly show UPDATE_WITH_FULL_RA_RESULTS and -0.03%, 0.31%, 0.13% and 0.34% (LB) respectively, in the for tests, respectively.

# Introduction

The triangular partition mode [1] was adopted in 12th JVET meeting. In this method, a CU is split into 2 triangular partitions by its diagonal or anti-diagonal line. Each partition uses a uni-directional motion vector for motion compensation. The triangular partition mode is only applied to skip and merge modes and the motion vector used by this mode is represented as an index of a candidate set. Adaptive pixel blending based on the motion information on both sides are performed. Transform and quantization are performed on the whole CU.

# Technical description

In CE4-4.9, it is proposed that regular merge candidate list is re-used as the first step of the TPM merge candidate list generation. In the second step, the candidates in regular merge candidate list are converted into uni-directional motion vectors and are added into the final TPM merge candidate list until the list reaches the predefined maximum size (5). Among them, test 4.9.a and test 4.9.b use the complete set of regular merge list, which includes motion vectors candidates of spatial and temporal neighbors, history-based candidates and pairwise based candidate and zero MVs. While test 4.9.c and test 4.9.d use candidates from spatial and temporal neighbors only.

Meanwhile, in the test 4.9.a and test 4.9.c, pruning is performed as it is in VTM-4.0 and in the test 4.9.b and 4.9.d, pruning in the 2nd step of TPM merge candidate list generation is removed.

## Complexity analysis

|  | Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE4-4.9.a | 5 | 13 | 9+38 | 2+0 | 1 | 0 | 12 |  |
| CE4-4.9.b | 5 | 13 | 9+0 | 2+0 | 1 | 0 | 12 |  |
| CE4-4.9.c | 5 | 7 | 5+38 | 2+0 | 1 | 0 | 7 |  |
| CE4-4.9.d | 5 | 7 | 5+0 | 2+0 | 1 | 0 | 7 |  |

# Simulation results

## CE4-4.9.a

The performance of using complete regular merge list in the 1st step and performing pruning in the 2nd step is reported in the Table 1.

Table 1 Performance of CE4-4.9.a over VTM-4.0

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.08% | -0.16% | -0.16% | 100% | 100% |
| Class B | -0.04% | -0.11% | -0.09% | 99% | 100% |
| Class C | -0.04% | -0.19% | -0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.18% | -0.04% | 100% | 100% |
| Class F | -0.02% | -0.06% | -0.13% | 99% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.17% | 0.17% | 100% | 100% |
| Class C | -0.01% | 0.20% | -0.16% | 100% | 102% |
| Class E | -0.08% | 0.41% | -0.11% | 100% | 100% |
| Overall | -0.03% | 0.10% | -0.01% | 100% | 101% |
| Class D | 0.05% | -0.27% | -0.23% | 99% | 99% |
| Class F | -0.02% | -0.33% | -0.35% | 100% | 99% |

## CE4-4.9.b

The performance of using complete regular merge list in the 1st step and avoid pruning in the 2nd step is reported in the Table 2.

Table 2 Performance of CE4-4.9.b over VTM-4.0

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.08% | -0.15% | -0.10% | 100% | 100% |
| Class B | -0.01% | -0.10% | -0.04% | 99% | 100% |
| Class C | 0.02% | -0.04% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | -0.16% | -0.01% | 100% | 101% |
| Class F | 0.03% | 0.03% | 0.00% | 100% | 99% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.06% | 0.01% | 100% | 103% |
| Class C | 0.33% | 0.67% | 0.55% | 100% | 100% |
| Class E | 0.52% | 0.92% | -0.51% | 100% | 102% |
| Overall | 0.31% | 0.48% | 0.06% | 100% | 102% |
| Class D | 0.30% | 0.42% | -0.36% | 99% | 102% |
| Class F | 0.10% | 0.46% | 0.78% | 99% | 100% |

## CE4-4.9.c

The performance of using spatial and temporal merge candidates only in the 1st step and performing pruning in the 2nd step is reported in the Table 3.

Table 3 Performance of CE4-4.9.c over VTM-4.0

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.06% | -0.06% | -0.07% | 100% | 99% |
| Class B | -0.01% | -0.09% | -0.01% | 99% | 100% |
| Class C | 0.00% | -0.01% | -0.11% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.26% | -0.13% | 100% | 100% |
| Class F | -0.01% | -0.03% | -0.05% | 99% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | -0.22% | -0.13% | 100% | 99% |
| Class C | 0.15% | 0.49% | 0.37% | 101% | 101% |
| Class E | 0.17% | 0.47% | 0.04% | 100% | 102% |
| Overall | 0.13% | 0.19% | 0.08% | 100% | 101% |
| Class D | 0.10% | 0.13% | -0.35% | 98% | 100% |
| Class F | -0.08% | 0.00% | -0.28% | 100% | 101% |

## CE4-4.9.d

The performance of using complete regular merge list in the 1st step and avoid pruning in the 2nd step is reported in the Table 4.

Table 4 Performance of CE4-4.9.d over VTM-4.0

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | -0.04% | -0.14% | -0.04% | 100% | 100% |
| Class B | 0.01% | -0.08% | -0.11% | 100% | 101% |
| Class C | 0.03% | -0.05% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.22% | 0.00% | 100% | 100% |
| Class F | 0.01% | 0.01% | -0.08% | 99% | 100% |
|  |  |  | Low Delay B |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | 0.22% | 0.16% | 100% | 101% |
| Class C | 0.33% | 0.65% | 0.52% | 101% | 102% |
| Class E | 0.60% | 0.57% | 0.25% | 101% | 101% |
| Overall | 0.34% | 0.45% | 0.30% | 100% | 101% |
| Class D | 0.31% | 0.00% | -0.19% | 99% | 99% |
| Class F | 0.08% | 0.32% | 0.62% | 101% | 100% |

# Conclusion

This contribution reports the test results of CE4-4.9. In which, it is proposed to reuse regular merge list construction to derive TPM merge list. After deriving merge candidates based on the regular merge list, a uni-directional MV candidate will used as it is when adding it to the final list, and each bi-MV candidate is split into uni-L0-MV and uni-L1-MV, which are added to the final TPM merge list. Full pruning or no pruning is performed when final uni-directional TPM list is derived. Four tests are performed on different ways of using regular merge candidate list and different pruning methods. Simulation results reportedly show UPDATE_WITH_FULL_RESULTS and -0.03%, 0.31%, 0.13% and 0.34% (LB) in the for tests, respectively.

# Reference

- R.-L. Liao and C. S. Lim, “CE10: Triangular prediction unit mode”, JVET-K0144, Ljubljana, July 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).