| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0419-v1 |
| --- | --- |

| Title: | Non-CE6: A Simplification of Implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Amir Said, Nan Hu, Chao-Hsiung Hung, Vadim Seregin and Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution document proposes a simplification that removes the block-shape dependency of the implicit transform selection in VTM-4.0. The proposed method provides minor coding gains over the implicit MTS of VTM-4.0 for all-intra (AI) and random access (RA). On average, 0.11% AI, 0.02% RA and -0.xx LDB luma coding gains are achieved.

# Introduction

In VTM-4.0, multiple transform selection (MTS) uses a high-level flag to determine whether the transform is (i) explicitly signaled to select among multiple candidates or (ii) implicitly derived based on block-shape. In the latter case, combinations of DST-7 and DCT-2 as horizontal or vertical transforms up to size 16. Specifically, for intra-coded luma blocks, the following block-shape dependent conditions define the implicit MTS in VTM-4.0:

- If the width and height of a block are equal and both are less than or equal to 16, DST-7 is used in both horizontal and vertical directions.

- If the width of a block is smaller than its height and it is less than or equal to 16, DST-7 is in horizontal and DCT-2 is used in vertical direction.

- If the height of a block is smaller than its width and it is less than or equal to 16, DST-7 is in vertical and DCT-2 is used in horizontal direction.

- Otherwise, DCT-2 is used in both directions.

In the VVC specification draft, JVET-M1001-v6, the above conditions are satisfied in the following two equations:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1030)

where “0” and “1” denote DCT-2 and DST-7 respectively.

This contribution introduces a simplification to the implicit MTS design by removing the block-shape dependency.

# Proposed method

The proposed implicit MTS is also applied to intra-coded luma blocks as in VTM-4.0, yet it is defined based on the following simpler condition:

- For any row or column with less than or equal to 16 (luma) samples, DST-7 is used.

- Otherwise, DCT-2 is used to transform any row or column with number of (luma) samples greater than 16.

The proposed change can be reflected in JVET-M1001-v6 as:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1030)

where the changes are colored in red and the removed syntax elements are crossed out as red.

# Experimental Results

The proposed method is tested over VTM-4.0 where the implicit MTS is enabled with the following changes on top of CTC configurations [1]: MTS=0 and ImplicitMTS=1 for both reference (anchor) and test. The results are presented in Table 1.

Table 1: Results of the proposed method over VTM-4.0 with MTS=0 and ImplicitMTS=1.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.40% | -0.33% | 101% | 96% |
| Class A2 | -0.12% | -0.33% | -0.42% | 101% | 98% |
| Class B | -0.12% | -0.40% | -0.36% | 101% | 96% |
| Class C | 0.05% | -0.37% | -0.16% | 101% | 95% |
| Class E | -0.18% | -0.43% | -0.53% | 101% | 99% |
| Overall | -0.11% | -0.39% | -0.35% | 101% | 96% |
| Class D | 0.04% | -0.20% | -0.12% | 101% | 94% |
| Class F | -0.02% | -0.30% | -0.10% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.31% | -0.21% | 98% | 97% |
| Class A2 | 0.03% | -0.04% | -0.06% | 99% | 100% |
| Class B | -0.06% | -0.18% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.24% | -0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.07% | 100% | 99% |
| Class D | -0.05% | 0.05% | -0.26% | 100% | 99% |
| Class F | 0.00% | -0.16% | -0.35% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 with MTS=0 and ImplicitMTS=1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Proposed text for specification document

The proposed syntax changes for JVET-M1001 version 6 are as follows, where the changes are colored in red and the removed syntax elements are crossed out as red.

….

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16  &&  nTbW <= nTbH ) ? 1 : 0	(8-1029)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16  &&  nTbH <= nTbW ) ? 1 : 0	(8-1030)

….

# Conclusions

In this contribution document, a simplification for the implicit MTS in VTM-4.0 is proposed. The proposed simplification removes block-shape dependency and provides about 0.1% all-intra coding gain. With the JVET-N0420, proposing the same implicit transform selection for intra-subblock partitioning (ISP), this proposal can be viewed as a unification of both implicit transform selection scheme.

It is recommended to adopt the proposed method in VVC.

# References

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, Jan. 2019.

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- X. Zhao and H. E. Egilmez, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-M1026, 13th meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).