| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0422 |
| --- | --- |

| Title: | AHG19: Support of adaptive resolution change (ARC) in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang Nan Hu Muhammed Coban W.-J. Chien V. Seregin M. Karczewicz | Tel: Email: | +1-858-658-4535 hongtaow@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this document a basic adaptive resolution change scheme allowing seamless switching between different resolutions without requiring IDR pictures at switching points is described. Picture prediction across different resolutions is supported by normative upsampling with filtering and simple subsampling of reference pictures along with scaling of motion information across different resolutions. Proposed method supports 1:2 and 2:1 resolution change in horizontal and vertical dimensions. The software for proposed scheme is implemented on top of VTM-4.0 SW.

# Introduction

TBD

# Technical description

## Signalling

To support adaptive resolution change (ARC), it is proposed to include one flag (adaptive_resolution_change_flag) in SPS to indicate ARC support in the sequence.

If adaptive_resolution_change_flag is set, then change in resolution is signalled by referring to a PPS with different resolution or indicating the new picture resolution in tile group header by a relative picture size indicator w.r.t. to the highest supported resolution.

## DPB management

In the proposed ARC scheme, DPB pictures are stored at the full resolution. The reconstructed pictures with smaller resolution are up-sampled to full resolution. In addition, temporal motion information is scaled to the full resolution. The method used for up-sampling the reconstructed picture is described in section 2.3.

For pictures with resolutions smaller than full resolution, when fetching the TMVP and storing MV into DPB buffer, MV scaling is performed to make sure the precision of MVs fetched and stored is consistent across picture sizes. At the same time it is proposed to perform MV clipping when fetching a TMVP to make sure TMVP values are suitable for the current picture size.

## Up-sampling filter

The up-sampling filter described in [1] is used to up-sample the pictures for DPB if needed. The filter first inserts zeros at odd sample positions and then use the following filter to obtain the up-sampled picture.

( -3 0 19 32 19 0 -3)/32

## Motion compensation

Since the reference pictures are stored at full resolution, the motion compensation of full resolution pictures can be performed without requiring any change.

For down-sampled pictures, the motion compensation with downsampling isis performed on the fly as follows:

- If the motion vector (MV) is an integer pixel MV, the MV is scaled to a MV value for the full resolution reference picture that the MV points to the block which is the closest to the collocated block in a reference picture with down-sampled size. And the corresponding block is used for motion compensation.

- If the MV has fractional pixel components. A block of integer pixels is fetched in the same manner as integer MV and the fractional pixels are interpolated based on the integer pixel blocks.

- The picture boarder extension of the reference picture is modified so that the pixels that are closest to the collocated positions of the down-sampled pictures (if down-sampled) are used to extend the picture boarders.

Figure 1 MV scaling and motion compensation for down-sampled pictures

## Software implementation

Regarding to the software implementation, it is proposed to follow the following aspects:

- When a picture is switched to lower resolution by ARC, the picture to be encoded is obtained by down-sampling the full resolution input picture. And the reconstructed lower resolution picture is filled into the top left corner of a full resolution picture to be written into reconstruction file.

- The PSNRs of lower resolution pictures are calculated against the down-sampled input picture.

- The encoder accepts 2 parameters to determine the start/end of ARC resolution change.

# References

- T. Davies, “Resolution switching for coding efficiency and resilience”, JCTVC-F158, Turin, IT, 14-22 July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).