| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0426-v4 |
| --- | --- |

| Title: | Non-CE3: History-based intra MPM default angular modes derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, P. Wennersten, R. Yu, J. Ström, R. Sjöberg | Tel: Email: | +46761499428 zhi.a.zhang@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents a History-based intra MPM list default angular modes derivation method for intra coding. An intra modes buffer is applied to store the intra prediction mode for decoded blocks. The intra modes buffer is updated following FIFO rule. Only angular intra prediction mode is stored in the intra modes buffer. Only unique mode is stored in the intra modes buffer. The intra modes buffer is updated through entire slice. The proposed method has a luma BDR impact for AI/RA/LDB of -0.05%/-0.06%/-0.03% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/100%/100% compared to VTM4.0.

# Introduction

An extended 6 MPM list is adopted in Versatile Video Coding (VVC). To reduce the complexity, only left and above intra prediction modes are used for MPM list generation. The left and above intra prediction modes are inserted to MPM list. The rest of the modes in MPM list are populated by four different ordering methods. [ref JVET-L0165].

If both the left and above intra prediction modes are non-angular intra prediction modes, the rest modes in MPM are populated by a pre-defined angular mode ordering:

- When the extended reference line index is larger than 0, the pre-defined angular mode ordering is [VER_IDX, HOR_IDX, 2, DIA_IDX, VDIA_IDX, 26]

- When intra sub-partition split type is horizontal split, the pre-defined angular mode ordering is [HOR_IDX, 25, 10, 65, VER_IDX]

- When intra sub-partition split type is vertical split, the pre-defined angular mode ordering is [VER_IDX, 43, 60, 3, HOR_IDX]

- When extended reference line index equals to 0 and intra sub-partition split type is no split, the pre-defined angular mode ordering is [VER_IDX, HOR_IDX, VER_IDX – 4, VER_IDX + 4]

The pre-defined angular mode ordering is derived bases on an optimized experimental result over all test sequences and all frames of each test sequences. It is noticed that the intra prediction modes are spatially correlated, which means that some intra prediction modes are selected more frequently than others in a spatial neighborhood. The pre-defined angular mode ordering can not capture the spatially correlation in a local area.

# Proposed method

This contribution presents a history-based intra MPM default angular modes derivation method to improve the intra MPM derivation process when both the left and above intra prediction modes are non-angular intra prediction modes.

The intra prediction information of a previously coded block is stored in an intra modes buffer. The intra modes buffer table is maintained during the encode / decode process. The intra modes buffer is composed by 6 unique angular intra prediction modes, wherein 3 unique modes have a vertical likely intra prediction mode ≥ 34 and 3 unique modes have a horizontal likely intra prediction mode < 34. Herein, the intra modes buffer is represented by two tables, wherein one vertical likely modes table and one horizontal likely modes table.

When a new slice is encountered, the 3 vertical likely unique angular modes are reset to a pre-defined mode ordering [VER_IDX, VER_IDX – 4, DIA_IDX], the 3 horizontal likely unique angular modes are reset to a pre-defined mode ordering [HOR_IDX, 2, 26].

Whenever there is a block is coded with angular intra prediction mode, the horizontal likely table or vertical likely table is selected to update depends on the coded mode is vertical likely mode or horizontal likely mode. The coded mode is added to the last entry of the selected table and the first entry of the selected table is removed from the selected table if no duplicated mode is found, otherwise the duplicated mode is removed from the selected table. The coded mode will be stored in the intra prediction buffer as the latest coded mode. The latest coded mode will be applied for intra MPM default angular modes derivation.

Given a block is coded with intra prediction mode X, figure 1 shows an example of the update process of intra modes buffer table:

Figure 1, An example of intra modes buffer table update

When start encoding / decoding a CTU, the intra modes buffer table inherits from the last block in CTU at the left of current CTU. When the current CTU is at the picture left boundary, the intra modes buffer table inherits from the last block in CTU at the above of current CTU. Figure 2 shows the intra modes buffer table initialization and update in CTUs in one slice:

Figure 2, Intra modes buffer table update in one slice

## Test 1: the left and above intra prediction modes are initiated to planar (as VTM4.0)

In the test 1, the left and above intra prediction modes are initiated to planar mode as the current VTM4.0. The left intra prediction mode (L) is updated when the left neighbor block exists and has an intra prediction mode. The above intra prediction mode (A) is updated when the above neighbor block exists and has an intra prediction mode. When both L and A are non-angular modes (L < 2 && A < 2), the intra modes buffer is applied to derive the angular modes in MPM list as follows:

- When the extended reference line index is larger than 0:

|  | latest coded mode ≥ 34 | 1 < latest coded mode < 34 |
| --- | --- | --- |
| MPM list | V3, H3, V2, H2, V1, H1 | H3, V3, H2, V2, H1, V1 |

- When intra sub-partition split type is horizontal split or vertical split:

|  | Vertical split | Horizontal split |
| --- | --- | --- |
| MPM list | Planar, V3, V2, V1, H3, H2 | Planar, H3, H2, H1, V3, V2 |

- When extended reference line index equals to 0 and intra sub-partition split type is no split:

|  | latest coded mode ≥ 34 | 1 < latest coded mode < 34 |
| --- | --- | --- |
| MPM list (L == planar mode) | Planar, dc, V3, H3, V2, H2 | Planar, dc, H3, V3, H2, V2 |
| MPM list (L == dc mode) | Dc, planar, V3, H3, V2, H2 | Dc, planar, H3, V3, H2, V2 |

Wherein, the V3 is the latest vertical likely mode inserted into the buffer table, the H3 is the latest horizontal likely mode inserted into the buffer table.

The simulation result of Test 1 has a luma BDR impact for AI/RA/LDB of -0.05%/-0.06%/-0.03% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/100%/100% compared to VTM4.0.

## Test 2: the left and above intra prediction modes are initiated to -1

In the test 2, the left and above intra prediction modes are initiated to -1. The left intra prediction mode (L) is updated when the left neighbor block exists and has an intra prediction mode. The above intra prediction mode (A) is updated when the above neighbor block exists and has an intra prediction mode.

With the change of the initiate value for the intra prediction mode of the left and above neighbor blocks. The number of candidates to be inserted to the encoder full-RD search candidate list increases from 1 to 2 in the follow cases:

- left neighbor block does not exist or not have an intra prediction mode and above neighbor block has an intra prediction mode equals to planar mode.

- above neighbor block does not exist or not have an intra prediction mode and left neighbor block has an intra prediction mode equals to planar mode.

When both the L and A are non-angular intra prediction modes, the intra MPM list default angular modes derivation method is same as presented in the Test 1.

The simulation result of Test 2 has a luma BDR impact for AI/RA/LDB of -0.07%/-0.05%/-0.01% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/99%/99% compared to VTM4.0.

## Test B encoder full RD search for all modes in MPM list

The test B is defined in CE3: Summary Report on Intra Prediction and Mode Coding JVET-N0023 [2] test, wherein the encoder will do full RD search for all modes in MPM list. The encoder search update is same as CE3.3.1.1 and CE3.3.1.2.

Both test 1 and test 2 in section 2.1 and 2.1 are tested with the encoder full RD search for all modes in MPM list. The simulation results for test B use the anchor VTM4.0 with full RD search for all modes in MPM list.

The test B for test 1 has a luma BDR impact for AI of -0.07% compared to the modified anchor.

The test B for test 2 has a luma BDR impact for AI of -0.07% compared to the modified anchor.

# Complexity analysis

The complexity compared to VTM4.0 has increased due to update process of the buffer table. The worst case for the MPM list derivation process does not change. Complexity analysis compared to VTM4.0:

- A buffer table with 7 unsigned integers to store the coded angular intra prediction modes.

- Maximum of 4 comparison to update the buffer table.

- No complexity increases for MPM list derivation process.

The buffer table update process has been skipped when the coded block size is 4 × 4. The restriction does not increase the complexity on 4 × 4 intra block compared to VTM4.0.

The buffer table update process can be within one CTU. Whenever start encoding / decoding a CTU, the buffer table is reset to a pre-defined mode ordering. Test 3 simulation results for buffer table update process within one CTU has a luma BDR impact for AI/RA/LDB of -0.04%%/-0.05%/-0.03% compared to VTM4.0 and decoding time impact for AI/RA/LDB of 100%/99%/99% compared to VTM4.0. The detail simulation result reports in VTM-4.0_psnr_he10_RunSimVTM4.0_TEST3_ctuBaseBuf_psnr.xlsm.

# Simulation result

## Test 1: the left and above intra prediction modes are initiated to planar (as VTM4.0)

The detail simulation result reports in

VTM-4.0_psnr_he10_RunSimVTM4.0_TEST1_sliceBaseBuf_psnr.xlsm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -0,06% | -0,08% | 92% | 100% |
| Class A2 | -0,06% | 0,02% | -0,13% | 100% | 102% |
| Class B | -0,07% | -0,07% | 0,00% | 91% | 99% |
| Class C | -0,02% | -0,07% | -0,04% | 71% | 102% |
| Class E | 0,01% | 0,15% | 0,07% | 83% | 100% |
| Overall | -0,05% | -0,02% | -0,03% | 86% | 100% |
| Class D | 0,02% | -0,09% | -0,17% | 77% | 100% |
| Class F | 0,04% | 0,02% | -0,07% | 83% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,06% | -0,14% | -0,03% | 101% | 100% |
| Class A2 | -0,07% | 0,11% | -0,03% | 97% | 99% |
| Class B | -0,05% | 0,06% | -0,19% | 97% | 100% |
| Class C | -0,04% | -0,15% | -0,29% | 77% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,06% | -0,02% | -0,16% | 92% | 100% |
| Class D | 0,01% | -0,17% | -0,10% | 92% | 102% |
| Class F | 0,01% | -0,15% | -0,24% | 90% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,14% | -0,38% | 99% | 99% |
| Class C | -0,05% | 0,28% | -0,19% | 88% | 100% |
| Class E | 0,03% | 1,18% | 0,55% | 79% | 100% |
| Overall | -0,03% | 0,33% | -0,09% | 90% | 100% |
| Class D | -0,04% | 0,10% | 0,05% | 69% | 99% |
| Class F | -0,11% | -0,16% | 0,32% | 88% | 102% |

## Test 2: the left and above intra prediction modes are initiated to -1

The detail simulation result reports in

VTM-4.0_psnr_he10_RunSimVTM4.0_TEST2_sliceBaseBuf_newInit_psnr.xlsm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,09% | -0,01% | -0,10% | 80% | 100% |
| Class A2 | -0,08% | 0,03% | -0,09% | 85% | 101% |
| Class B | -0,10% | -0,07% | -0,02% | 79% | 99% |
| Class C | -0,05% | -0,14% | -0,05% | 78% | 100% |
| Class E | -0,01% | -0,01% | -0,01% | 79% | 100% |
| Overall | -0,07% | -0,05% | -0,05% | 80% | 100% |
| Class D | -0,04% | 0,04% | -0,06% | 74% | 101% |
| Class F | 0,03% | -0,03% | 0,00% | 85% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,07% | -0,15% | -0,14% | 97% | 99% |
| Class A2 | -0,10% | -0,06% | -0,03% | 95% | 100% |
| Class B | -0,03% | 0,02% | -0,18% | 86% | 98% |
| Class C | -0,03% | -0,19% | -0,11% | 84% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,05% | -0,09% | -0,12% | 89% | 99% |
| Class D | -0,05% | -0,25% | -0,10% | 84% | 100% |
| Class F | 0,03% | 0,02% | -0,14% | 89% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | -0,13% | -0,23% | 96% | 99% |
| Class C | -0,01% | 0,15% | 0,31% | 85% | 100% |
| Class E | -0,01% | 0,59% | -0,19% | 79% | 99% |
| Overall | -0,01% | 0,14% | -0,04% | 88% | 99% |
| Class D | -0,04% | 0,31% | 0,11% | 72% | 101% |
| Class F | -0,13% | 0,19% | -0,65% | 82% | 102% |

## Test B: encoder full RD search for all modes in MPM list

### Test 1_B: the left and above intra prediction modes are initiated to planar (as VTM4.0)

The detail simulation result reports in

VTM-4.0_psnr_he10_RunSimVTM4.0_TEST1_sliceBaseBuf_testB_psnr.xlsm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (encoder full RD search for modes in MPM list) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,10% | -0,11% | -0,08% | 77% | 76% |
| Class A2 | -0,08% | -0,14% | -0,09% | 67% | 66% |
| Class B | -0,10% | -0,16% | -0,08% | 60% | 59% |
| Class C | -0,07% | -0,07% | -0,01% | 59% | 59% |
| Class E | 0,03% | 0,03% | 0,06% | 59% | 58% |
| Overall | -0,07% | -0,10% | -0,04% | 63% | 63% |
| Class D | -0,01% | -0,04% | -0,01% | 59% | 59% |
| Class F | 0,04% | -0,11% | 0,06% | 58% | 58% |

### Test 2_B: the left and above intra prediction modes are initiated to -1

The detail simulation result reports in

VTM-4.0_psnr_he10_RunSimVTM4.0_TEST2_sliceBaseBuf_newInit_testB_psnr.xlsm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (encoder full RD search for modes in MPM list) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,11% | -0,12% | -0,06% | 60% | 59% |
| Class A2 | -0,09% | -0,15% | -0,12% | 62% | 60% |
| Class B | -0,10% | -0,13% | -0,11% | 61% | 58% |
| Class C | -0,07% | -0,10% | -0,05% | 60% | 59% |
| Class E | 0,02% | -0,03% | 0,04% | 60% | 57% |
| Overall | -0,07% | -0,11% | -0,07% | 60% | 59% |
| Class D | -0,01% | 0,09% | 0,09% | 60% | 58% |
| Class F | 0,05% | -0,03% | 0,01% | 58% | 58% |

# Conclusions

We propose to adopt history-based intra MPM list default angular modes derivation method for intra mode coding to VVC and VTM to improve coding efficiency performance.

# References

[1]. JVET-L0165: CE3-6.2.1: Extended MPM list

[2]. JVET-N0023: CE3: Summary Report on Intra Prediction and Mode Coding

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).