| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0432 |
| --- | --- |

| Title: | AHG16/Non-CE3: On 1xN and 2xN subpartitions in intra subpartition coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adarsh K. Ramasubramonian Geert Van der Auwera Ted Hsieh Vadim Seregin Luong Pham Van Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Email: | aramasub@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes changes to intra-subpartition (ISP) coding with respect to the handling of 1xN and 2xN subpartitions. For coding blocks of size 4xN that are ISP-coded with vertical partitions, only the vertical intra prediction mode is allowed. For coding blocks of size 8xN that are ISP-coded with vertical partitions, prediction is performed in two 4xN regions of samples. It is asserted that these changes alleviate the implementation issues with respect to 1xN and 2xN subpartitions, with negligible performance loss. The BD-rate performance of the proposed modifications are as follows: (0.04%, 0.09%, 0.06%) for (Y, Cb, Cr) with runtimes of 101% (Enc) and 102% (Dec) for the AI configuration.

# Introduction

In the 13th JVET meeting, intra subpartition coding (ISP) [1] was adopted. Under ISP, an intra-coded luma coding block may be split into two or four subpartitions, and an indication in the coding unit determines whether the subpartitions are obtained by horizontal or vertical split. Each subpartition of a coding block is reconstructed successively. A subpartition in a coding block may depend on the reconstructed samples of preceding subpartitions in the coding block. An additional restriction that any subpartition consist of at least 16 samples is imposed to match the delay of the 4x4 luma coding blocks that are not ISP-coded. ISP is only applied to luma coding blocks.

Concerns regarding handling of subpartitions of sizes 1xN and 2xN were raised during the 13th JVET meeting and during subsequent discussions in the JVET reflector. It was remarked that subpartitions of these sizes are difficult to handle by hardware decoders, and in some cases encoders. Some of the concerns include dependency of 1xN subpartitions on previously reconstructed 1xN subpartitions and overhead in reading/writing these subpartitions from buffer/local memory.

A CE test [1] was also performed disabling the 1xN and 2xN subpartitions; the performance loss of disabling such subpartitions was considerable. In this proposal, two changes to ISP coding are proposed that alleviate the implementation concerns related to 1xN and 2xN subpartitions.

# Proposal

An analysis of ISP reveals that the following ISP-coded blocks result in subpartitions of sizes 1xN and 2xN: (a) 4xN coding blocks with N greater than 4 that are split vertically and (b) 8xN coding blocks with N greater than 4 that are split vertically (ISP is disabled for 4x4 coding blocks; for coding blocks of size 8x4 that are split vertically result in subpartitions of size 4x4.).

Currently for each subpartition, the first subpartition in a coding block is reconstructed from the residual and predicted sample blocks, and then the next subpartition is reconstructed and so on. The critical path of prediction of a 1xN subpartition thus depends on the reconstruction of the previous subpartitions of size 1xN. Similar argument holds for 2xN subpartitions.

For 4xN coding blocks split vertically with ISP where N > 4, it is proposed to only allow the vertical mode for intra prediction (a similar idea was proposed in [2] for 4xN coding blocks only). This ensures that the prediction of any of the subpartitions (1xN subpartitions in case of 4xN coding block with N > 8 and 2xN subpartitions in case of 4x8 coding block) would be independent of the other subpartitions in the coding block, and hence may be performed in parallel or by considering all the subpartitions as a 4xN block for prediction. The intra mode is not signalled for such coding blocks. Figure 1 shows an example of how the vertically split subpartitions of a 4xN coding block only use vertical intra mode for prediction. The inverse transform operation is still performed at the 1xN subpartition level.

Figure 1: Only vertical intra mode prediction is allowed for 4xN coding blocks that are coded with ISP and vertical split

For 8xN coding blocks split vertically with ISP where N > 4, it is proposed that the prediction of samples is performed on regions of four samples in width. For example, when the coding block size is 8xN (N > 4), two prediction regions each of size 4xN are chosen. Figure 2 shows an example of how an 8xN subpartition is split into two prediction regions (region 1 and region 2) of size 4xN each, and four transforms of size 2xN each. The prediction of the region 2 is dependent on the reconstruction of the region 1. Thus, the prediction of any sample within the coding block is only dependent on reconstructed samples from neighbouring coding blocks and preceding 4xN prediction regions.

Figure 2: An 8xN (N > 4) coding block coded with ISP and split vertically; prediction is done on 4xN prediction regions and the transform block (TB) size is 2xN

In Figure 1, the four line residuals of size 1xN and similarly in Figure 2, within one prediction region (region 1 or 2), the two residual blocks of 2xN can be generated in parallel or together in one step, i.e., the vertical inverse transform operation of the two 2xN subpartitions within each prediction region may be performed by considering the region as a 4xN block because the vertical transform for all the subpartitions in ISP is identical. The horizontal transform may also be done for the 2xN subpartitions in parallel – either by separate processing, or by considering the two subpartitions as a single 4xN block and the horizontal inverse transform operation as a combination of the two smaller inverse transform operations in the horizontal direction.

The following table summarizes the proposed partition sizes for prediction and transform sizes; for other block sizes, the prediction and transform sizes are as currently defined for ISP.

| Coding block sizes | Partition sizes for prediction | Transform sizes |
| --- | --- | --- |
| 4x8 | 4x8 (vertical mode only) | 2xN |
| 4xN (N > 8) | 4xN (vertical mode only) | 1xN |
| 8xN (N > 4) | 4xN | 2xN |

# Simulation results

The proposed method was tested on top of VTM-4.0 for AI and RA configurations. The simulation results show that the proposed method has a negligible loss of (0.04%, 0.07%, 0.01%) for (Y, Cb, Cr) with runtimes of 100% (Enc) and 101% (Dec) for the AI configuration.

Table 1: Simulations results obtained for the proposed changes compared to VTM-4.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.05% | 101% | 102% |
| Class A2 | 0.02% | 0.20% | 0.00% | 102% | 102% |
| Class B | 0.02% | 0.08% | 0.03% | 101% | 103% |
| Class C | 0.07% | 0.04% | 0.12% | 101% | 102% |
| Class E | 0.09% | 0.15% | 0.10% | 101% | 101% |
| Overall | 0.04% | 0.09% | 0.06% | 101% | 102% |
| Class D | 0.07% | 0.19% | -0.04% | 100% | 100% |
| Class F | 0.04% | 0.00% | -0.12% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.06% | -0.04% | -0.09% |  |  |
| Class F |  |  |  |  |  |

# References

- S. De Luxán Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI), “CE3: Intra Sub-Partitions Coding Mode,” JVET-M0102.

- S. De Luxán Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI), “CE3-related: Improvement on the Intra Sub-Partitions Coding Mode,” JVET-M0426.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).