| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0436 |
| --- | --- |

| Title: | CE3-3.4.1, CE3-3.4.2, CE3-3.4.3: Results for unified MPM list construction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Bossen1, K. Misra2 | Email: | fbossen@sharplabs.com misrak@sharplabs.com |
| Source: | 1Sharp Electronics of Canada, 2Sharp Labs of America |  |  |

_____________________________

# Abstract

This contribution provides results for the following CE3 experiments: CE3-3.4.1, CE3-3.4.2, and CE3-3.4.3. The experiments evaluate performance of a modified luma intra Most Probable Modes (MPM) list comprising solely angular modes, as proposed in JVET-M0528. A flag is used to indicate if the intra mode is angular or non-angular. For the non-angular modes, a further flag is received to indicate Planar or DC mode. It is asserted that the proposed list construction is a simplification over the current VVC design that features four different MPM lists: a list for zero reference line, a list for non-zero reference line, a list for intra sub-partitions and a list for combined intra-inter prediction.

Test CE3-3.4.1 evaluates the proposed list construction with 6 MPM and shows a coding benefit of 0.11% (all-intra) with no impact on encoder runtime. Test CE3-3.4.2 makes use of the same list construction as in CE3-3.4.1 but also tests N lowest bit-cost modes at the encoder. This results in a larger coding gain of 0.29% with a 3% encoder runtime increase. It is also reported that for class A1 sequences in all-intra configuration the proposed test provides a coding gain of 0.52% with no impact on encoder runtime. In CE3-3.4.3, the MPM list construction process is truncated to a size of 4. This results in a slightly lower coding gain of 0.05%. As requested in the CE3-3 description, all three tests were carried out using a further encoder modification (i.e. ALL_MPM) that evaluates all the candidates in the MPM list (for reference line 0) during the R-D mode decision. The performance of the ALL_MPM tests was measured versus anchors that contain the same encoder change. Tests CE3-3.3.4.1, CE3-3.4.2, CE3-4.3 were also evaluated with that a modification that reduces the number of contexts used for coding angular mode flag. The context reduction removes checking of left and above syntax elements and is enabled by setting the macro REDUCE_CTXT to 1 in the software released as part of CE3 testing. Listed below is the summary of test results:

| CE3 Tests | All−Intra |  |  |  |  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.4.1 | −0.11% | −0.08% | −0.09% | 99% | 101% | −0.02% | −0.02% | −0.13% | 100% | 100% |
| 3.4.2 | −0.29% | −0.2% | −0.17% | 103% | 100% | −0.08% | −0.01% | −0.07% | 100% | 100% |
| 3.4.3 | −0.05% | −0.05% | −0.02% | 97% | 100% | 0.00% | −0.01% | −0.04% | 99% | 99% |
| 3.4.1+ALL_MPM | −0.13% | −0.15% | −0.08% | 99% | 101% | −0.04% | 0.10% | −0.04% | 100% | 100% |
| 3.4.2+ALL_MPM | −0.13% | −0.14% | −0.08% | 100% | 101% | −0.05% | 0.07% | −0.04% | 99% | 99% |
| 3.4.3+ALL_MPM | −0.07% | −0.09% | −0.05% | 99% | 100% | −0.02% | 0.08% | 0.02% | 99% | 98% |
| 3.4.1+REDUCE_CTXT | −0.11% | −0.07% | −0.08% | 98% | 100% | −0.03% | −0.05% | −0.07% | 99% | 100% |
| 3.4.2+REDUCE_CTXT | −0.29% | −0.18% | −0.18% | 103% | 101% | −0.09% | 0.01% | −0.18% | 99% | 100% |
| 3.4.3+REDUCE_CTXT | −0.05% | −0.04% | −0.06% | 96% | 100% | 0.00% | 0.02% | −0.10% | 99% | 100% |

A complexity analysis is also provided, including efficient software implementation details.

# Introduction

When intra sub partitions (ISP) are not used, the proposed MPM list construction process makes use of the same angular most probable mode (MPM) list independently of the reference line. This is the same as the approach used in JVET-M0528 [1]. When ISP is used the list includes the 5 angular modes derived in VTM4 [2] as well as an additional angular mode for 6 MPMs. Since the list no longer contains non-angular modes (INTRA_PLANAR, INTRA_DC modes) the luma intra mode signaling is modified to first signal a flag indicating if the selected mode is angular. For non-angular mode a further flag is received indicating if the mode is INTRA_PLANAR or INTRA_DC. ISP does not support INTRA_DC so for ISP mode, when non-angular mode is signaled INTRA_PLANAR is inferred. For Combined Inter/Intra Prediction (CIIP) the only angular modes allowed are: vertical and horizontal modes. Therefore, a single flag needs to be signaled to distinguish between the two angular modes (no list construction is needed).

The detailed Working Draft (WD) changes for 6 MPM case are listed in document attached to this contribution. For the 4 MPM case the list construction process is to be truncated and the syntax binarizations need to be appropriately modified.

# Experimental Results

The proposed change is implemented in VTM-4.0 [2] and tests CE3-3.4.1, CE3-3.4.2, CE3-3.4.3 are evaluated under common test conditions [3]. The tests were also evaluated using a further encoder modification (i.e. ALL_MPM) that evaluates all the candidates in the MPM list (for reference line 0) during the R-D mode decision. The performance of the ALL_MPM tests was measured versus anchors that contain the same encoder change. The summary results of the tests is listed below. The detailed results can be found in attached excel documents.

Results are also provided that reduces the number of contexts used for coding angular mode flag. In CE3-3.4.x, the angular mode flag’s is code using a context from one of 10 contexts. It is selected using the following equation:

ctxInc = ( condL  &&  availableL ) + ( condLB  &&  availableLB ) + ( condA  &&  availableA && sameCtuA ) + ( condAR  &&  availableAR && sameCtuAR ) + (intra_subpartitions_mode_flag[ x0 ][ y0 ] != 0 ? 5 : 0)

The context reduction tests removes checking of left and above syntax elements in equation above resulting in a context reduction of 4 contexts:

ctxInc = ( condLB  &&  availableLB ) + ( condAR  &&  availableAR && sameCtuAR ) + (intra_subpartitions_mode_flag[ x0 ][ y0 ] != 0 ? 3 : 0)

As can be seen in results below, the simplification has very little impact on coding efficiency (0.00% to 0.01%).

## Results for CE3-3.4.1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.31% | -0.25% | 97% | 101% |
| Class A2 | -0.08% | 0.03% | -0.08% | 99% | 102% |
| Class B | -0.11% | -0.11% | -0.03% | 100% | 102% |
| Class C | 0.00% | -0.08% | -0.04% | 98% | 100% |
| Class E | -0.04% | 0.10% | -0.11% | 99% | 100% |
| Overall | -0.11% | -0.08% | -0.09% | 99% | 101% |
| Class D | 0.02% | -0.03% | -0.16% | 100% | 101% |
| Class F | 0.08% | -0.01% | 0.00% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.05% | -0.10% | 99% | 100% |
| Class A2 | 0.00% | -0.06% | -0.03% | 99% | 100% |
| Class B | 0.01% | 0.02% | -0.08% | 100% | 100% |
| Class C | 0.01% | -0.02% | -0.31% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.13% | 100% | 100% |
| Class D | 0.03% | -0.35% | 0.02% | 100% | 100% |
| Class F | 0.00% | -0.12% | -0.22% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.18% | -0.37% | 101% | 101% |
| Class C | 0.00% | 0.58% | -0.03% | 100% | 102% |
| Class E | -0.07% | 0.83% | 1.12% | 99% | 100% |
| Overall | -0.02% | 0.32% | 0.12% | 100% | 101% |
| Class D | -0.02% | 0.10% | 0.15% | 101% | 102% |
| Class F | -0.15% | 0.13% | 0.10% | 100% | 101% |

## Results for CE3-3.4.2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.50% | -0.32% | 100% | 100% |
| Class A2 | -0.23% | 0.00% | -0.19% | 103% | 101% |
| Class B | -0.28% | -0.17% | -0.17% | 103% | 101% |
| Class C | -0.20% | -0.18% | -0.02% | 106% | 99% |
| Class E | -0.25% | -0.16% | -0.18% | 104% | 99% |
| Overall | -0.29% | -0.20% | -0.17% | 103% | 100% |
| Class D | -0.17% | -0.04% | -0.20% | 107% | 100% |
| Class F | -0.03% | 0.01% | 0.02% | 102% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.16% | -0.04% | 100% | 100% |
| Class A2 | -0.06% | 0.15% | -0.01% | 100% | 99% |
| Class B | -0.09% | -0.03% | -0.08% | 100% | 101% |
| Class C | -0.01% | 0.02% | -0.12% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.08% | -0.01% | -0.07% | 100% | 100% |
| Class D | -0.05% | 0.08% | -0.06% | 100% | 101% |
| Class F | -0.05% | -0.21% | -0.13% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.09% | -0.09% | 100% | 101% |
| Class C | -0.04% | 0.25% | 0.06% | 100% | 101% |
| Class E | 0.02% | 1.62% | 0.76% | 99% | 100% |
| Overall | -0.01% | 0.45% | 0.17% | 100% | 101% |
| Class D | -0.01% | 0.17% | -0.50% | 100% | 102% |
| Class F | -0.19% | -0.18% | -0.13% | 100% | 101% |

## Results for CE3-3.4.3

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.32% | -0.20% | 95% | 97% |
| Class A2 | -0.03% | 0.07% | -0.07% | 99% | 99% |
| Class B | -0.07% | -0.07% | 0.01% | 97% | 100% |
| Class C | 0.08% | -0.03% | 0.08% | 98% | 102% |
| Class E | 0.06% | 0.09% | 0.00% | 97% | 100% |
| Overall | -0.05% | -0.05% | -0.02% | 97% | 100% |
| Class D | 0.05% | 0.00% | -0.01% | 98% | 101% |
| Class F | 0.18% | 0.08% | -0.01% | 100% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.13% | -0.09% | 98% | 98% |
| Class A2 | 0.03% | 0.04% | 0.05% | 98% | 98% |
| Class B | -0.01% | 0.00% | -0.07% | 99% | 99% |
| Class C | 0.03% | 0.05% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | -0.04% | 99% | 99% |
| Class D | 0.02% | 0.19% | -0.03% | 99% | 100% |
| Class F | 0.08% | -0.02% | 0.07% | 99% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | 0.02% | 98% | 101% |
| Class C | -0.05% | 0.22% | -0.08% | 98% | 101% |
| Class E | -0.03% | 0.93% | 0.72% | 98% | 100% |
| Overall | -0.03% | 0.29% | 0.16% | 98% | 101% |
| Class D | -0.03% | 1.07% | 0.39% | 96% | 101% |
| Class F | -0.18% | -0.20% | -0.17% | 98% | 101% |

## Results for CE3-3.4.1 + ALL_MPM

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.30% | -0.22% | 97% | 99% |
| Class A2 | -0.09% | -0.15% | -0.06% | 99% | 101% |
| Class B | -0.14% | -0.12% | -0.13% | 100% | 101% |
| Class C | -0.08% | -0.11% | 0.01% | 100% | 100% |
| Class E | -0.07% | -0.14% | -0.03% | 100% | 102% |
| Overall | -0.13% | -0.15% | -0.08% | 99% | 101% |
| Class D | -0.05% | -0.05% | 0.11% | 96% | 91% |
| Class F | 0.03% | -0.06% | 0.13% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.11% | -0.18% | 101% | 101% |
| Class A2 | -0.01% | 0.14% | 0.02% | 100% | 100% |
| Class B | -0.06% | 0.19% | 0.04% | 100% | 100% |
| Class C | -0.02% | -0.05% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.10% | -0.04% | 100% | 100% |
| Class D | 0.01% | -0.11% | -0.63% | 100% | 100% |
| Class F | 0.02% | 0.00% | 0.15% | 100% | 98% |

## Results for CE3-3.4.2 + ALL_MPM

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.28% | -0.33% | -0.24% | 101% | 102% |
| Class A2 | -0.08% | -0.10% | -0.07% | 100% | 100% |
| Class B | -0.14% | -0.10% | -0.07% | 100% | 101% |
| Class C | -0.09% | -0.08% | -0.02% | 100% | 101% |
| Class E | -0.06% | -0.12% | -0.04% | 100% | 100% |
| Overall | -0.13% | -0.14% | -0.08% | 100% | 101% |
| Class D | -0.05% | -0.06% | 0.07% | 98% | 92% |
| Class F | 0.02% | -0.02% | 0.10% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.09% | -0.15% | 100% | 99% |
| Class A2 | -0.01% | 0.16% | 0.01% | 99% | 98% |
| Class B | -0.05% | 0.09% | -0.06% | 100% | 100% |
| Class C | -0.02% | -0.02% | 0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | 0.07% | -0.04% | 99% | 99% |
| Class D | -0.01% | -0.13% | -0.57% | 99% | 99% |
| Class F | -0.01% | -0.08% | 0.05% | 99% | 98% |

## Results for CE3-3.4.3 + ALL_MPM

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.24% | -0.15% | 97% | 99% |
| Class A2 | -0.03% | -0.09% | -0.04% | 99% | 100% |
| Class B | -0.08% | -0.08% | -0.09% | 99% | 99% |
| Class C | -0.02% | -0.05% | 0.00% | 101% | 103% |
| Class E | 0.06% | 0.02% | 0.02% | 99% | 101% |
| Overall | -0.07% | -0.09% | -0.05% | 99% | 100% |
| Class D | 0.00% | 0.07% | 0.10% | 99% | 94% |
| Class F | 0.14% | 0.05% | 0.03% | 98% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0+ALL_MPM=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.13% | -0.08% | 99% | 98% |
| Class A2 | 0.00% | 0.14% | -0.01% | 99% | 98% |
| Class B | -0.04% | 0.02% | -0.02% | 99% | 97% |
| Class C | 0.01% | 0.08% | 0.15% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.08% | 0.02% | 99% | 98% |
| Class D | 0.04% | -0.31% | -0.31% | 99% | 99% |
| Class F | 0.03% | 0.04% | 0.06% | 98% | 96% |

## Results for CE3-3.4.1 with reduced number of contexts for angular mode flag

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | -0.30% | -0.26% | 96% | 100% |
| Class A2 | -0.09% | 0.09% | -0.10% | 99% | 100% |
| Class B | -0.13% | -0.05% | -0.08% | 98% | 100% |
| Class C | -0.01% | -0.11% | 0.00% | 98% | 98% |
| Class E | -0.04% | 0.00% | -0.01% | 99% | 100% |
| Overall | -0.11% | -0.07% | -0.08% | 98% | 100% |
| Class D | 0.03% | 0.03% | -0.12% | 99% | 98% |
| Class F | 0.08% | -0.04% | -0.09% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.28% | -0.08% | 99% | 99% |
| Class A2 | -0.01% | 0.00% | 0.09% | 98% | 99% |
| Class B | -0.03% | 0.04% | -0.22% | 100% | 100% |
| Class C | 0.04% | -0.04% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.05% | -0.07% | 99% | 100% |
| Class D | 0.00% | -0.26% | -0.20% | 98% | 98% |
| Class F | 0.03% | 0.06% | -0.17% | 99% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.32% | 0.03% | 99% | 100% |
| Class C | -0.03% | 0.21% | -0.21% | 99% | 99% |
| Class E | 0.05% | 0.61% | 0.97% | 98% | 99% |
| Overall | 0.00% | 0.09% | 0.18% | 99% | 99% |
| Class D | 0.01% | 0.01% | -0.67% | 99% | 100% |
| Class F | -0.15% | 0.12% | -0.65% | 100% | 100% |

## Results for CE3-3.4.2 with reduced number of contexts for angular mode flag

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.40% | -0.40% | 100% | 100% |
| Class A2 | -0.23% | -0.04% | -0.17% | 102% | 100% |
| Class B | -0.29% | -0.16% | -0.17% | 103% | 101% |
| Class C | -0.19% | -0.12% | -0.06% | 105% | 102% |
| Class E | -0.26% | -0.19% | -0.15% | 104% | 100% |
| Overall | -0.29% | -0.18% | -0.18% | 103% | 101% |
| Class D | -0.17% | -0.06% | -0.25% | 105% | 100% |
| Class F | -0.05% | -0.06% | -0.03% | 102% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.22% | -0.20% | 99% | 99% |
| Class A2 | -0.08% | 0.03% | 0.02% | 99% | 98% |
| Class B | -0.08% | 0.04% | -0.15% | 100% | 100% |
| Class C | -0.05% | 0.11% | -0.35% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.01% | -0.18% | 99% | 100% |
| Class D | -0.06% | -0.07% | -0.18% | 100% | 99% |
| Class F | -0.08% | -0.02% | -0.16% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.08% | 0.02% | 98% | 99% |
| Class C | -0.03% | 0.44% | -0.08% | 100% | 100% |
| Class E | 0.11% | 0.58% | 0.27% | 98% | 99% |
| Overall | 0.01% | 0.26% | 0.05% | 99% | 99% |
| Class D | -0.03% | -0.26% | -0.77% | 100% | 100% |
| Class F | -0.23% | -0.30% | 0.51% | 99% | 99% |

## Results for CE3-3.4.3 with reduced number of contexts for angular mode flag

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.28% | -0.26% | 93% | 99% |
| Class A2 | -0.03% | 0.02% | -0.06% | 96% | 99% |
| Class B | -0.07% | -0.07% | -0.02% | 96% | 100% |
| Class C | 0.08% | -0.01% | -0.03% | 98% | 101% |
| Class E | 0.04% | 0.13% | 0.03% | 97% | 100% |
| Overall | -0.05% | -0.04% | -0.06% | 96% | 100% |
| Class D | 0.08% | -0.03% | -0.22% | 98% | 101% |
| Class F | 0.16% | 0.08% | 0.05% | 98% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.06% | -0.09% | 98% | 100% |
| Class A2 | 0.03% | 0.16% | 0.10% | 99% | 99% |
| Class B | -0.01% | 0.11% | -0.17% | 99% | 100% |
| Class C | 0.03% | -0.13% | -0.18% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | -0.10% | 99% | 100% |
| Class D | 0.03% | 0.01% | -0.19% | 99% | 100% |
| Class F | 0.08% | -0.01% | -0.10% | 99% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.14% | -0.36% | 99% | 99% |
| Class C | -0.01% | 0.13% | -0.31% | 100% | 99% |
| Class E | -0.01% | 0.94% | 0.53% | 98% | 99% |
| Overall | -0.02% | 0.34% | -0.12% | 99% | 99% |
| Class D | -0.02% | 0.07% | 0.09% | 99% | 100% |
| Class F | -0.07% | -0.07% | -0.17% | 99% | 100% |

# Complexity aspects

## CABAC

The number of bins required to represent the intra mode is summarized in the table below, where. ‘a + b’ is defined as: ‘a’ is the maximum number of context-coded bins in the worst case, which includes bins for coding ISP and MRL modes; and ‘b’ is the maximum number of bypass bins in the worst case. Essentially one additional context-coded bin may need parsing with this proposal.

|  | VTM 4.0 | Proposal |
| --- | --- | --- |
| MRL == 0, ISP == 0 | 3 + 6 | 4 + 6 |
| MRL != 0 | 2 + 5 | 2 + 5 2 + 3 (test 3.4.3) |
| ISP != 0 | 3 + 5 | 4 + 5 4 + 3 (test 3.4.3) |

The number of contexts for coding the intra mode is increased from 1 to 12. This increase can be reduced from 1 to 8 without incurring a coding efficiency penalty, as provided in the results section. Further reductions may be possible.

## MPM list construction

The MPM list construction process can be implement in ways that are efficient in software. For example, the following code implements the list contribution process for all cases:

bool leftAndAboveAreSame = leftIntraDir == aboveIntraDir;

int maxDir = std::max(leftIntraDir, aboveIntraDir) - 2;

int minDir = std::min(leftIntraDir, aboveIntraDir) - 2;

uint64_t mpms;

if (!leftAndAboveAreSame && minDir >= 0)

{

int idx = Clip3(0, 3, ((maxDir - minDir + 2) & 63) - 2);

const uint32_t s[4] = { 0x00010001, 0x00010001, 0x01000101, 0x00000101 };

const uint32_t k[4] = { 0x3e023f01, 0x023e013f, 0x3e013f01, 0x013f013f };

uint32_t x = minDir * s[idx] + maxDir * (s[idx] ^ 0x01010101) + k[idx];

mpms = leftIntraDir + (aboveIntraDir << 8) + ((uint64_t) x << 16) - 0x0202;

mpms &= 0x3f3f3f3fffffull;

}

else if (maxDir >= 0)

{

bool maxDirIsVertical = maxDir >= 32;

int idx = leftAndAboveAreSame && (maxDirIsVertical == isHorSplit) ? ispType : 0;

const uint64_t s[3] = { 0x10101010101ull, 0x010101ull, 0x010101ull };

const uint64_t k[3] = { 0x3d023e013f00ull, 0x090310013f00ull, 0x383d30013f00ull };

mpms = s[idx] * maxDir + k[idx];

mpms &= 0x3f3f3f3f3fffull;

}

else

{

const uint64_t k[3] = { 0x184020001030ull, 0x36303f081710ull, 0x1a10015a3930ull };

mpms = k[ispType];

}

It should be noted that in an implementation such as the above, where a single register/variable is used to represent all elements in the list, the list size doesn’t impact the operation count (i.e., the operation count is the same regardless of whether 4 or 6 MPMs are in the list).

The equivalent code spans over 200 lines in VTM 4.0.

At a high level, the code distinguishes between 3 cases:

- Both neighbours use different angular modes

- One neighbour uses an angular or both neighbours use the same angular mode

- Neither neighbour uses an angular mode

For each case an index is computed. This index is then used to address a lookup table to determine the MPM list using simple arithmetic.

## MPM mode reconstruction

Reconstructing the mode from the MPM index is still straightforward using the single register/variable representation:

mode = (uint8_t)(mpms >> (8 * mpmIdx)) + 2

## Non-MPM mode reconstruction

Given that in this proposal DC and planar modes are coded separately, there are only 65 modes that need consideration when reconstructing a non-MPM mode. This reduced number of modes can be beneficial when using bit arithmetic and 64-bit registers.

While the reference software and the text describe the reconstruction process as using a sort on a vector of modes, no such sorting is actually needed. Below is a software implementation that uses the BMI2 instruction set to efficiently reconstruct the mode from a remainder index:

uint64_t mask = 0;

for (int i = 0; i < 6; i++)

{

uint8_t v = mpms >> (8 * i);

if (!(v & 64))

mask |= 1ull << v;

}

ipred_mode = 2 + _tzcnt_u64(_pdep_u64(1ull << ipred_mode, ~mask));

# Notes on Encoder

This section describes the search process for VTM4 and the CE3-3.4.x tests:

## VTM4

In VTM4, for an intra tile group, intra modes are selected for full RD testing as follows:

STEP 1: Generate a list uiRdModeList of modes with the lowest (simplified) RD cost for reference line 0. Copy list uiRdModeList to m_rdModeListWithoutMrl representing a list of candidate modes for ISP.

STEP 2: Add non-zero reference line (NZRL) mode candidates from the MPM list. A list extendRefList, keeps track of the reference line for list uiRdModeList. 
STEP 3: Add numCand candidate modes to uiRdModeList list, if the candidate mode is not already in the list. The value of numCand and the candidate modes are derived based on intra modes of the neighbouring blocks. In VTM4 the added modes essentially correspond to the top-right and bottom-left neighbours.
STEP 4: For ISP, add numCandISP candidate ISP modes to list of modes for ISP, if the candidate ISP mode is not already in the list. numCandISP and the ISP candidate modes are derived based on intra modes of the neighboring blocks
STEP 5: Remove modes from list of modes for ISP that cannot be signaled by ISP MPM list

For non-intra tile groups there exist some encoder speedups operating on the lists derived in STEP 1 to 5 above.

## CE3-3.4.1

In CE3-3.4.1, STEPs 1 to 5 are executed in an identical fashion to VTM4. In particular, the modes added in STEP 3 are the same as in VTM4. No adaptation to the proposed coding method is done. It is asserted that changing the modes added in STEP3 to more closely reflect what is done in other CE3-3 experiments provides an additional coding benefit of about 0.07%. It should also be noted that while the proposed method allows for 7 different modes (6 MPM + planar) to be coded for ISP, the 6th MPM is also removed from the ISP candidate list, if present, such as to not favor the proposed method.

## CE3-3.4.2

In CE3-3.4.2, STEP 3 is modified as follows. The numCand intra modes with the lowest bit counts for mode coding are additionally included in uiRdModeList if the intra mode is not already in the list. Note that only the bit counts already computed in STEP 1 are considered here.

## CE3-3.4.3

In CE3-3.4.2, STEPs 1 to 5 are executed in an identical fashion to VTM4. Note that because the MPM list is shorter, fewer modes may be tested for NZRL and ISP.

# Recommendation

In the current VVC WD there exists four different luma intra MPM list construction process. This contribution proposes the use of a unified list construction process. It is recommended to adopt the proposed luma intra MPM list construction process in the next version of the VVC WD and VTM.

# References

[1] F. Bossen, K. Misra, “Non-CE3: A unified luma intra mode list construction process,” JVET-M0528, 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0 VTM-4.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).