Changes with respect to VVC WD highlighted in yellow

# Syntax Changes

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| … |  |
| } |  |

# Semantical changes

The syntax elements intra_luma_mpm_flag[ x0 ][ y0 ], intra_luma_mpm_idx[ x0 ][ y0 ] and intra_luma_mpm_remainder[ x0 ][ y0 ] specify the intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.4.2.

When intra_luma_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

# Decoding process change

Table 8-15 – Specification of trTypeHor and trTypeVer depending on predModeIntra

| predModeIntra | trTypeHor | trTypeVer |
| --- | --- | --- |
| INTRA_PLANAR, INTRA_ANGULAR31, INTRA_ANGULAR32, INTRA_ANGULAR34, INTRA_ANGULAR36, INTRA_ANGULAR37 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |
| INTRA_ANGULAR33, INTRA_ANGULAR35 INTRA_DC | 0 | 0 |
| INTRA_ANGULAR2, INTRA_ANGULAR4,…,INTRA_ANGULAR28, INTRA_ANGULAR30, INTRA_ANGULAR39, INTRA_ANGULAR41,…,INTRA_ANGULAR63, INTRA_ANGULAR65 | ( nTbW >= 4 && nTbW <= 16 ) ? 1 : 0 | 0 |
| INTRA_ANGULAR3, INTRA_ANGULAR5,…, INTRA_ANGULAR27, INTRA_ANGULAR29, INTRA_ANGULAR38, INTRA_ANGULAR40,…,INTRA_ANGULAR64, INTRA_ANGULAR66 | 0 | ( nTbH >= 4 && nTbH <= 16 ) ? 1 : 0 |

| Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_mpm_flag[ ][ ] | (intra_subpartitions_mode_flag[][] ? 1 : ( IntraLumaRefLineIdx[][] != 0 ) ? 2 : 0 ) | na | na | na | na | na |

…

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.4.4.2.5 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p, and the intra prediction reference line index refIdx as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.4.4.2.6 is invoked with the transform block width nTbW, the transform block height nTbH, and the reference sample array p, and the intra prediction reference line index refIdx as inputs, and the output is the predicted sample array predSamples.

##### 8.4.4.2.5 Specification of INTRA_PLANAR intra prediction mode

Inputs to this process are:

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx,

- a variable refIdx specifying the intra prediction reference line index,

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables nW and nH are derived as follows:

nW = Max( nTbW, 2 )			(8-116)

nH = Max( nTbH, 2 )			(8-117)

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1 and y = 0..nTbH − 1, are derived as follows:

predV[ x ][ y ] = ( ( nH − 1 − y ) * p[ x ][ −1 − refIdx ] + ( y + 1 ) * p[ −1 − refIdx ][ nTbH ] ) << Log2 ( nW ) 	(8-118)

predH[ x ][ y ] = ( ( nW − 1 − x ) * p[ −1 − refIdx ][ y ] + ( x + 1 ) * p[ nTbW ][ −1 − refIdx ] ) << Log2 ( nH ) 	(8-119)

predSamples[ x ][ y ] = ( predV[ x ][ y ] + predH[ x ][ y ] + nW * nH )  >>  (Log2 ( nW ) + Log2 ( nH ) + 1 )	(8-120)

##### 8.4.4.2.6 Specification of INTRA_DC intra prediction mode

Inputs to this process are:

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx,

- a variable refIdx specifying the intra prediction reference line index.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, are derived by the following ordered steps:

- A variable dcVal is derived as follows:

- When nTbW is equal to nTbH:

dcVal =x'=0nTbW-1px'-1 - refIdx+y'=0nTbH-1p-1 - refIdxy'+nTbW≫(Log2nTbW+1) 	(8-121)

- When nTbW is greater than nTbH:

dcVal =x'=0nTbW-1px'-1 - refIdx+nTbW≫1≫Log2nTbW	(8-122)

- When nTbW is less than nTbH:

dcVal =y'=0nTbH-1p-1 - refIdxy'+nTbH≫1≫Log2nTbH	(8-123)

- The prediction samples predSamples[x][y] are derived as follows:

predSamples[ x ][ y ] = dcVal, with x = 0.. nTbW − 1, y = 0.. nTbH − 1	(8-124)