| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0437 |
| --- | --- |

| Title: | Non-CE3: On allowing non-MPM modes for ISP and non-zero reference line |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Bossen1, K. Misra2 | Email: | fbossen@sharplabs.com misrak@sharplabs.com |
| Source: | 1Sharp Electronics of Canada, 2Sharp Labs of America |  |  |

_____________________________

# Abstract

In the current VVC working draft, non-zero reference line (NZRL) prediction and intra sub partitions (ISP) are limited to selecting intra prediction modes from the most probable mode (MPM) list. The angular modes in the MPM list are derived from neighboring intra blocks. As a result, if an angular mode most beneficial to the current block is not in the MPM list, then this mode is not available for sample prediction. This contribution proposes to remove the restriction and allow non-MPM modes for ISP and NZRL. It is asserted that the change has at least three benefits: (i) it makes the intra mode signaling design more consistent, (ii) it reduces the number of checks required for parsing, and (iii) it allows flexibility in the design of future encoders.

# Introduction

In the current VVC working draft (WD) [1], non-zero reference line (NZRL) prediction and intra sub partitions (ISP) are limited to selecting intra prediction modes from the most probable mode (MPM) list. As a result, if an angular mode most beneficial to the current block is not in the MPM list, then this mode is not available for sample prediction. The contribution proposes to remove this restriction and allow non-MPM modes for NZRL and ISP. The modification also results in: (i) more consistent signaling, (ii) a reduction in the number of checks compared to the current signaling in the VVC WD and (iii) flexibility in the design of future encoders. The proposed modification to VVC WD is listed in attached document.

The current design of VVC WD does not define INTRA_PLANAR and INTRA_DC sample prediction for NZRL. The figure below illustrates the reference sample selection for INTRA_PLANAR and INTRA_DC prediction modes when using different reference lines.

# Results

Four sets of results are presented for the All-Intra encoder configuration defined in Common Test Conditions [2]. They are:

- Allow ISP to signal the use of non-MPM modes and allow the encoder to test and select these modes

- Allow NZRL to signal the use of non-MPM modes and allow the encoder to test non-MPM modes in the same way as is done for the zero reference line

- Allow ISP and NZRL to use non-MPM mode (combining experiments 1 and 2 above)

- Allow ISP and NZRL to use non-MPM mode but keep the VTM4 [3] search

The summary results are listed below while the detailed results can be found in the attached excel file.

## Allow non-MPM for ISP with encoder changes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.02% | -0.02% | 108% | 98% |
| Class A2 | -0.03% | 0.07% | -0.05% | 109% | 98% |
| Class B | -0.07% | 0.00% | 0.00% | 110% | 99% |
| Class C | -0.17% | -0.15% | -0.17% | 111% | 99% |
| Class E | -0.16% | -0.08% | -0.11% | 109% | 99% |
| Overall | -0.09% | -0.03% | -0.07% | 110% | 99% |
| Class D | -0.21% | -0.17% | -0.24% | 110% | 95% |
| Class F | -0.21% | -0.11% | -0.15% | 105% | 98% |

## Allow non-MPM for NZRL with encoder changes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.05% | 156% | 97% |
| Class A2 | -0.03% | 0.17% | 0.03% | 156% | 98% |
| Class B | -0.03% | 0.03% | 0.05% | 157% | 98% |
| Class C | -0.06% | -0.07% | -0.03% | 159% | 99% |
| Class E | -0.02% | 0.12% | 0.07% | 159% | 98% |
| Overall | -0.03% | 0.05% | 0.03% | 157% | 98% |
| Class D | -0.05% | 0.09% | 0.00% | 160% | 98% |
| Class F | -0.15% | -0.02% | -0.14% | 131% | 96% |

## Allow non-MPM for ISP and NZRL with encoder changes

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.06% | 0.05% | 167% | 98% |
| Class A2 | -0.07% | 0.13% | 0.02% | 162% | 98% |
| Class B | -0.09% | 0.04% | -0.04% | 164% | 97% |
| Class C | -0.23% | -0.22% | -0.19% | 161% | 97% |
| Class E | -0.17% | 0.04% | -0.07% | 167% | 100% |
| Overall | -0.12% | 0.00% | -0.05% | 164% | 98% |
| Class D | -0.27% | -0.24% | -0.31% | 157% | 88% |
| Class F | -0.34% | -0.18% | -0.36% | 134% | 97% |

## Allow non-MPM for ISP and NZRL with same encoder search as VTM4

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.05% | 0.03% | 99% | 97% |
| Class A2 | 0.01% | 0.08% | 0.01% | 100% | 98% |
| Class B | 0.03% | -0.04% | 0.08% | 100% | 98% |
| Class C | 0.07% | -0.04% | 0.00% | 101% | 99% |
| Class E | 0.06% | 0.11% | 0.18% | 99% | 99% |
| Overall | 0.04% | 0.02% | 0.06% | 100% | 98% |
| Class D | 0.08% | -0.01% | -0.09% | 101% | 99% |
| Class F | 0.10% | 0.01% | 0.04% | 98% | 97% |

# Recommendation

In the current VVC working draft (WD) [1], non-zero reference line (NZRL) prediction and intra sub partitions (ISP) are limited to selecting intra prediction modes from the most probable mode (MPM) list. The contribution proposes to remove this restriction and allow non-MPM modes for NZRL and ISP. It is recommended to adopt the proposed restriction removal in the next version of the VVC WD and VTM. Furthermore, it is not recommended to modify the mode search in VTM at this time.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001, 13th meeting: Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

[3] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0 VTM-4.0

# Patent rights declaration(s)

Sharp Laboratories of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).