| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0438-v1 |
| --- | --- |

| Title: | AHG12: Loop filter disabled across virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Yen Lin, Lin Liu, Jian-Liang Lin, Yung-Chang Chang, Chi-Cheng Ju No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 Philippe Hanhart, Yuwen He 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Jian-Liang Lin +886-3-5670766 ext. 25250 {Sheng-Yen.Lin, Lin.Liu, jl.lin, yc.zhang,cc.ju}@mediatek.com Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |
| Source: | MediaTek Inc. InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The in-loop filters in a conventional video coding system are used to remove undesired artifacts and enhance image quality. For a 360° video, the layout for a specific projection format may have one or more discontinuous edges. Applying the in-loop filters on these discontinuous edges directly may result in poor visual quality and decreased coding efficiency because the accessed pixels (to be referenced and/or filtered) across discontinuous edges are jointly processed (filtered). To address this problem, it is proposed to disable the in-loop filters across vertical and/or horizontal virtual boundaries in the picture.

# Introduction

For projection formats composed of a plurality of faces, no matter what kind of compact frame packing arrangement is used, discontinuities appear between two or more adjacent faces in the frame packed picture. For example, considering the 3×2 frame packing configuration depicted in Fig. 1, the three faces in the top half are continuous in the 3D geometry, the three faces in the bottom half are continuous in the 3D geometry, but the top and bottom halves of the frame packed picture are discontinuous in the 3D geometry. If in-loop filtering operations are performed across this discontinuity, face seam artifacts may become visible in the reconstructed video.

Fig. 1. An image of HEC in 3×2 layout

To alleviate face seam artifacts, in-loop filtering operations may be disabled across discontinuities in the frame-packed picture. This technique was tested in CE13 test 1.1 [1] at Marrakech meeting. Subjective viewing of the CE test results showed that this technique provided a “good improvement” [2]. In JVET-M0892 [3], a syntax is proposed to signal vertical and/or horizontal virtual boundaries across which the in-loop filtering operations are disabled. Compared to using two tiles, one for each set of continuous faces, and to disable in-loop filtering operations across tiles, the proposed solution is more flexible as it does not require the face size to be a multiple of the CTU size. The signaling proposed in JVET-M0892 is designed to be general purpose, and applicable to other non-360° video use cases. The BoG on CE13 and CE13 related 360° video coding recommended to adopt JVET-M0892 for disabling of in-loop filters at vertical and horizontal boundaries signalled in the SPS [2]. However, during the JVET plenary, some concerns were raised by HW experts regarding the 4 luma samples granularity and the maximum number of vertical/horizontal virtual boundaries that could be signaled. In this contribution, the following changes are proposed compared to JVET-M0892 to address these concerns:

- The virtual boundary granularity is increased from 4 luma samples to 8 luma samples. In this way, the virtual boundaries are aligned with the deblocking grid.

- The maximum number of vertical virtual boundaries is reduced from pic_width_in_luma_samples ÷ 4 – 1 to 3.

- The maximum number of horizontal virtual boundaries is reduced from pic_height_in_luma_samples ÷ 4 – 1 to 3.

# Proposed Syntax

An exemplary syntax to disable the in-loop filters across virtual boundaries is shown in Table I.

Table I. The in-loop filters disabled across virtual boundaries syntax in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_loop_filter_across_virtual_boundaries_disabled_flag | u(1) |
| if( sps_loop_filter_across_virtual_boundaries_disabled_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) { |  |
| sps_virtual_boundaries_pos_x[ i ] | u(v) |
| } |  |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) { |  |
| sps_virtual_boundaries_pos_y[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_loop_filter_across_virtual_boundaries_disabled_flag equal to 1 specifies that the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. sps_loop_filter_across_virtual_boundaries_disabled_flag equal to 0 specifies that the in-loop filtering operation may be performed in pictures referring to the SPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of sps_loop_filter_across_virtual_boundaries_disabled_flag is inferred to be equal to 0.

sps_num_ver_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the SPS. The value of sps_num_ver_virtual_boundaries shall be in the range of 0 to 3, inclusive. When sps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

sps_virtual_boundaries_pos_x[ i ] is used to compute the value of SpsVirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The number of bits used to represent sps_virtual_boundaries_pos_x[ i ] is equal to Ceil( Log2( pic_width_in_luma_samples ) − 3 ). sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the vertical virtual boundary SpsVirtualBoundariesPosX[ i ] is derived as follows:

SpsVirtualBoundariesPosX[ i ] = sps_virtual_boundaries_pos_x[ i ] * 8

sps_num_hor_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_y[ i ] syntax elements that are present in the SPS. The value of sps_num_hor_virtual_boundaries shall be in the range of 0 to 3, inclusive. When sps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

sps_virtual_boundaries_pos_y[ i ] is used to compute the value of SpsVirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The number of bits used to represent sps_virtual_boundaries_pos_y[ i ] is equal to Ceil( Log2( pic_height_in_luma_samples ) − 3 ). sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the horizontal virtual boundary SpsVirtualBoundariesPosY[ i ] is derived as follows:

SpsVirtualBoundariesPosY[ i ] = sps_virtual_boundaries_pos_y[ i ] * 8

# Experimental Results

The simulations of the proposed method are conducted on PHEC. The software is implemented on 360Lib-9.0-dev and VTM-4.0 followed by the JVET common test conditions and evaluation procedures for 360° video [4]. The resolution of each face is set to 1280×1280, which yields a frame resolution of 3840×2560. As compared to PHEC, the BD-rate performance, in terms of end-to-end WS-PSNR, codec WS-PSNR and codec PSNR, provided by the proposed method is listed in Table II.

Table II. BD-rate performance for PHEC with in-loop filters disabled across face discontinuities

|  | End-to-end WS-PSNR |  |  | Codec WS-PSNR |  |  | Codec PSNR |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| SkateboardInLot | 0.07% | -0.56% | 0.19% | 0.07% | -0.91% | -0.09% | -0.25% | -2.22% | -1.74% |
| ChairLift | -0.03% | -0.37% | -0.18% | -0.02% | -0.51% | -0.26% | -0.39% | -1.21% | -0.66% |
| KiteFlite | -0.02% | -0.05% | -0.01% | -0.03% | -0.07% | -0.01% | -0.26% | -0.61% | -0.56% |
| Harbor | -0.15% | -0.20% | -0.09% | -0.15% | -0.35% | -0.25% | -0.87% | -2.27% | -3.01% |
| Trolley | -0.04% | -0.09% | -0.07% | -0.04% | -0.14% | -0.11% | -0.22% | -1.25% | -0.72% |
| GasLamp | -0.09% | -0.13% | -0.14% | -0.09% | -0.24% | -0.23% | -0.35% | -1.98% | -1.74% |
| Balboa | 0.05% | -0.32% | -0.36% | 0.01% | -0.42% | -0.44% | -0.43% | -1.25% | -1.16% |
| Broadway | 0.07% | 0.04% | 0.07% | 0.02% | -0.03% | 0.02% | -0.40% | -0.88% | -0.75% |
| Landing2 | -0.04% | -1.39% | -0.90% | -0.07% | -2.24% | -1.73% | -0.32% | -4.19% | -3.45% |
| BranCastle2 | 0.02% | 0.12% | -0.09% | 0.00% | 0.01% | -0.14% | -0.09% | -0.37% | -0.58% |
| Class S1 | -0.04% | -0.23% | -0.05% | -0.04% | -0.37% | -0.16% | -0.39% | -1.59% | -1.41% |
| Class S2 | 0.02% | -0.39% | -0.32% | -0.01% | -0.67% | -0.57% | -0.31% | -1.67% | -1.49% |
| Overall | -0.02% | -0.29% | -0.16% | -0.03% | -0.49% | -0.32% | -0.36% | -1.62% | -1.44% |
| Enc | xxx% |  |  |  |  |  |  |  |  |
| Dec | 99% |  |  |  |  |  |  |  |  |

# References

- S.-Y. Lin, L. Liu, J.-L. Lin, Y-C Chang, C.-C. Ju, P. Hanhart, and Y. He “CE13: In-loop filters disabled across face discontinuities (Test 1.1.a and Test1.1.b),” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0362, Jan. 2019, Marrakech, MA.

- J. Boyce “BoG report on CE13 and CE13 related 360° video coding,” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0874, Jan. 2019, Marrakech, MA.

- S.-Y. Lin, L. Liu, J.-L. Lin, Y-C Chang, C.-C. Ju, P. Hanhart, and Y. He “CE-13 related : Loop filter disabled across virtual boundaries,” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0892, Jan. 2019, Marrakech, MA.

- P. Hanhart, J. Boyce, K. Choi, and J.-L. Lin “JVET common test conditions and evaluation procedures for 360° video” Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1012, Oct. 2018, Macau, CN.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).