| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0440 |
| --- | --- |

| Title: | CE9-related: Disabling DMVR and BDOF when underlying assumptions are false |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbin Liu, Li Zhang, Kai Zhang, Jizheng Xu, Weijia Zhu, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190 Beijing, China | Tel: Email: | +86 13911971613 liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In current VTM [1], decoder-side motion vector refinement (DMVR) and bi-directional optical flow (BDOF) are applied to bi-predicted CUs. There are two common assumptions in DMVR and BDOF: first, prediction samples in the two reference blocks are similar, second, the derived motion vector offsets (MVO) are symmetric in the two prediction directions. However, in some cases, such assumptions may be not true, therefore DMVR and BDOF cannot achieve coding gain there. First, when unequal weighted prediction is applied, prediction samples in the two reference blocks may be not similar. Second, when the two reference pictures are with different picture order count (POC) distances from the current picture, the derived MVO may be asymmetric. This contribution tested disabling DMVR and BDOF in both cases. Simulation results reportedly show 0.01% loss under RA Main10 configurations with minor coding time saving. In addition, it is asserted that there is 0.10% coding gains for class F.

# Introduction

In current VTM, when unequal weighted prediction (UWP) is applied, BDOF is disallowed and DMVR is allowed. On the other hand, when the two reference pictures are with different POC distances from the current picture, DMVR is disallowed and BDOF is allowed. Such design is inconsistent with the underlying assumptions in DMVR and BDOF.

# Proposed method

To address the issues mentioned above, this contribution proposes to disable both DMVR and BDOF in the following cases:

- UWP is applied.

- The two reference pictures are with different POC distances from the current picture.

# Experimental results

The proposed methods were implemented on top of reference software VTM-4.0 [1], and simulations were performed following the common test conditions [2]. The results are shown in Table 1.

Table 1: Coding performance of proposed method.

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | 0.05% | 99% | 100% |
| Class A2 | 0.02% | 0.11% | 0.11% | 99% | 99% |
| Class B | 0.00% | 0.00% | -0.04% | 97% | 98% |
| Class C | 0.01% | -0.01% | -0.03% | 95% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.01% | 97% | 97% |
| Class D | 0.02% | -0.09% | -0.05% | 96% | 90% |
| Class F | -0.10% | -0.07% | -0.11% | 96% | 93% |

# Conclusions

This contribution proposes to disable DMVR and BDOF when the underlying assumptions are not true. It has no impact on compression performance. It is recommended to adopt this proposal into VVC.

# References

- VTM-4.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-4.0

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring, Doc. JVET-M1010, Jan. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).