| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0441 |
| --- | --- |

| Title: | CE4-3.3 : Candidates optimization on MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, hm.jang, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of CE4-3.3, which is about MMVD optimization described in JVET-M0307 [3]. Specifically, instead of 8 motion distance indices of MMVD in VTM4.0, 4 motion distance indices are used, and the actual motion distance from the given motion distance index is further adjusted considering mv resolution of the base motion vector.

In this proposal, following CE tests are reported.

- CE4-3.3a : MMVD distance table is reduced to 4 and is refined based on resolution of the base MV.

- CE4-3.3b : CE4-3.3a + CE4-3.4a

- CE4-3.3c : CE4-3.3a + CE4-1.1

- CE4-3.3d : CE4-3.3a + CE4-3.4a + CE4-1.1

The experimental results from four sub-tests from CE4-3.3 are reported as follows.

- CE4-3.3a RA : (Y 0.01%, Cb 0.07%, Cr 0.05%, EncT : 96%, DecT : 101%)

LB : (Y 0.01%, Cb 0.19%, Cr 0.00%, EncT : 96%, DecT : 101%)

- CE4-3.3b RA : (Y -0.08%, Cb 0.09%, Cr 0.10%, EncT : 97%, DecT : 101%)

LB : (Y -0.05%, Cb 0.25%, Cr 0.21%, EncT : 96%, DecT : 100%)

- CE4-3.3c RA : (Y -0.02%, Cb 0.13%, Cr 0.13%, EncT : 96%, DecT : 101%)

LB : (Y -0.03%, Cb 0.25%, Cr 0.02%, EncT : 97%, DecT : 101%)

- CE4-3.3d RA : (Y -0.11%, Cb 0.11%, Cr 0.17%, EncT : 97%, DecT : 101%)

LB : (Y -0.12%, Cb 0.20%, Cr 0.05%, EncT : 98%, DecT : 101%)

Introduction

MMVD scheme [1] was adopted in VTM3.0. In MMVD, a new motion vector expression is introduced in MERGE/SKIP mode. Three parameters are used to express the motion vector offset information, which is starting point, motion distance, and motion direction and overall number of candidates used for MMVD is 64 and is calculated as (2 starting point * 8 motion distance * 4 motion direction).

Motion vector information for MMVD is introduced as follows.

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| …… |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| …… |  |
| } |  |
| } |  |

In here, mmvd_merge_flag means the starting point for MVD that is derived from merge candidate list. mmvd_distance_index indicates distance from starting point and the distance is pre-defined in a range of 1 ~ 128(1/4pel ~ 32pel) as Table 1. And, mmvd_direction_idx indicates the direction from starting point and the direction is defined as Table 2.

Table 1. Specification of MmvdDistance[ x0 ][ y0 ] based on mmvd_distance_idx[ x0 ][ y0 ]

| mmvd_distance_idx[ x0 ][ y0 ] | MmvdDistance[ x0 ][ y0 ] |
| --- | --- |
| 0 | 1 |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

Table 2. Specification of MmvdSign[ x0 ][ y0 ] based on mmvd_direction_idx[ x0 ][ y0 ]

| mmvd_direction_idx[ x0 ][ y0 ] | MmvdSign[ x0 ][ y0 ][0] | MmvdSign[ x0 ][ y0 ][1] |
| --- | --- | --- |
| 0 | +1 | 0 |
| 1 | -1 | 0 |
| 2 | 0 | +1 |
| 3 | 0 | -1 |

From the MmvdDistance and MmvdSign as shown in Table 1 and Table 2, MmvdOffset is derived as follows.

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][0]

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( MmvdDistance[ x0 ][ y0 ] << 2 ) * MmvdSign[ x0 ][ y0 ][1]

Proposed method

This contribution proposed to reduce the number of distance candidates from 8 to 4 as follow:

Conventional distance candidates (8): {1, 2, 4, 8, 16, 32, 64, 128}

Proposed distance candidates (4): {1, 2, 4, 8}

In VTM4.0, actual motion distance is directly coded by sending the predefined distance candidates. In the proposed method, an index from 4 distance candidates is first coded and actual motion distance is further derived by considering both the index and the motion vector resolution. More details on the derivation process for actual motion distance are described in Table 3.

Table 3. Pseudo-code of the proposed scheme

distance = MmvdDistance[ x0 ][ y0 ] ; if (mvLX[0] != 0 ¦¦ mvLX[1] != 0) { if( mvLX[0] %32 == 0 && mvLX[1] % 32 == 0) distance = MmvdDistance[ x0 ][ y0 ] << 2; else if ( mvLX[0] %16 == 0 && mvLX[1] % 16 == 0) distance = MmvdDistance[ x0 ][ y0 ] << 1; }

In Table 3, if predFlagL0 of the base motion vector is equal to 1, X is set to 0. Otherwise, if predFlagL1 of the base motion vector is equal to 1, X is set to 1. Motion vector resolution of the base motion vector, mvLX with X being 0 or 1 is checked for each x and y component, mvLX[0] and mvLX[1], respectively. Using the distance derived in Table 3, MVD (MmvdOffset) from starting point for MMVD is changed as follows.

MmvdOffset[ x0 ][ y0 ][ 0 ] = ( distance << 2 ) * MmvdSign[ x0 ][ y0 ][0]

MmvdOffset[ x0 ][ y0 ][ 1 ] = ( distance << 2 ) * MmvdSign[ x0 ][ y0 ][1]

Experimental results

The proposed method has been implemented on top of the VTM4.0 with default setting and tested using JVET SDR common test condition [2].

CE4-3.3a experimental results:

The experimental results for CE4-3.3a reportedly show 0.01% (0.01%) BD-rate changes with 96% (96%) encoding complexity and 101% (101%) decoding complexity compared to VTM 4.0 as Anchor in RA (LB), respectively.

Table 4. CE4-3.3a experimental results on VTM4.0 RA / LB

|  | Over VTM 4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.15% | 0.15% | 96% | 100% |  |  |  |  |  |
| Class A2 | -0.01% | 0.11% | 0.09% | 96% | 100% |  |  |  |  |  |
| Class B | 0.01% | 0.13% | 0.01% | 96% | 101% | -0.02% | 0.00% | 0.06% | 96% | 100% |
| Class C | 0.03% | -0.11% | 0.02% | 96% | 101% | -0.01% | 0.46% | 0.32% | 96% | 102% |
| Class E |  |  |  |  |  | 0.09% | 0.16% | -0.52% | 96% | 101% |
| Overall | 0.01% | 0.07% | 0.05% | 96% | 101% | 0.01% | 0.19% | 0.00% | 96% | 101% |
| Class D | 0.00% | -0.10% | -0.04% | 96% | 101% | 0.02% | 0.36% | -0.42% | 96% | 103% |
| Class F | 0.02% | 0.04% | 0.02% | 96% | 100% | 0.10% | 0.11% | -0.17% | 97% | 101% |

CE4-3.3b experimental results:

In CE4-3.3b, the harmonization with CE4-3.4a is tested. In CE4-3.4a, diagonal direction candidates for MMVD was proposed in [4]. The experimental results for CE4-3.3b reportedly show -0.08% (-0.05%) BD-rate changes with 97% (96%) encoding complexity and 101% (100%) decoding complexity compared to VTM 4.0 as Anchor in RA (LB), respectively.

Table 5. CE4-3.3b experimental results on VTM4.0 RA / LB

|  | Over VTM 3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.13% | 0.26% | 97% | 100% |  |  |  |  |  |
| Class A2 | -0.19% | 0.01% | 0.01% | 97% | 100% |  |  |  |  |  |
| Class B | -0.04% | 0.12% | 0.16% | 97% | 100% | -0.07% | 0.08% | 0.35% | 96% | 98% |
| Class C | -0.10% | 0.08% | -0.01% | 97% | 101% | -0.06% | 0.23% | 0.28% | 96% | 101% |
| Class E |  |  |  |  |  | 0.01% | 0.56% | -0.12% | 96% | 100% |
| Overall | -0.08% | 0.09% | 0.10% | 97% | 101% | -0.05% | 0.25% | 0.21% | 96% | 100% |
| Class D | -0.14% | 0.09% | -0.20% | 96% | 101% | -0.13% | 0.02% | -0.13% | 96% | 103% |
| Class F | 0.03% | 0.15% | 0.12% | 97% | 101% | 0.10% | 0.24% | -0.15% | 98% | 100% |

CE4-3.3c experimental results:

In CE4-3.3c, the harmonization with CE4-1.1 is tested. In CE4-1.1, signaling MMVD syntax elements when merge_flag equal to 0 was proposed in [5]. The experimental results for CE4-3.3c reportedly show -0.02% (-0.03%) BD-rate changes with 97% (97%) encoding complexity and 101% (101%) decoding complexity compared to VTM 4.0 as Anchor in RA (LB), respectively.

Table 5. CE4-3.3c experimental results on VTM4.0 RA / LB

|  | Over VTM 3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.18% | 0.24% | 97% | 100% |  |  |  |  |  |
| Class A2 | -0.04% | 0.20% | 0.26% | 97% | 101% |  |  |  |  |  |
| Class B | 0.01% | 0.07% | 0.09% | 96% | 101% | -0.04% | 0.21% | -0.10% | 97% | 100% |
| Class C | -0.06% | 0.09% | 0.00% | 96% | 102% | -0.05% | 0.52% | 0.21% | 97% | 102% |
| Class E |  |  |  |  |  | 0.02% | -0.05% | -0.04% | 98% | 102% |
| Overall | -0.02% | 0.13% | 0.13% | 96% | 101% | -0.03% | 0.25% | 0.02% | 97% | 101% |
| Class D | 0.06% | -0.03% | 0.11% | 96% | 102% | 0.07% | 0.47% | 0.20% | 97% | 103% |
| Class F | -0.08% | 0.02% | 0.00% | 96% | 101% | -0.18% | -0.12% | 0.04% | 98% | 101% |

CE4-3.3d experimental results:

In CE4-3.3d, the harmonization with CE4-3.4a and CE4-1.1 is tested. The experimental results for CE4-3.3d reportedly show -0.11% (-0.12%) BD-rate changes with 97% (98%) encoding complexity and 101% (101%) decoding complexity compared to VTM 4.0 as Anchor in RA (LB), respectively.

Table 5. CE4-3.3d experimental results on VTM4.0 RA / LB

|  | Over VTM 3.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.22% | 0.32% | 97% | 100% |  |  |  |  |  |
| Class A2 | -0.22% | 0.18% | 0.16% | 97% | 101% |  |  |  |  |  |
| Class B | -0.07% | 0.05% | 0.14% | 97% | 101% | -0.15% | 0.03% | 0.15% | 97% | 100% |
| Class C | -0.14% | 0.05% | 0.10% | 97% | 102% | -0.09% | 0.34% | 0.19% | 98% | 102% |
| Class E |  |  |  |  |  | -0.11% | 0.28% | -0.28% | 98% | 102% |
| Overall | -0.11% | 0.11% | 0.17% | 97% | 101% | -0.12% | 0.20% | 0.05% | 98% | 101% |
| Class D | -0.04% | 0.23% | 0.24% | 97% | 102% | -0.02% | 0.15% | -0.22% | 98% | 104% |
| Class F | -0.06% | 0.03% | 0.05% | 97% | 101% | 0.02% | 0.31% | -0.07% | 99% | 101% |

Conclusion

In this contribution, MMVD optimization scheme, which was proposed in JVET-M0307 is tested. Specifically, instead of 8 motion distance indices of MMVD in VTM4.0, 4 motion distance indices are used, and the actual motion distance from the given motion distance index is further adjusted considering mv resolution of the base motion vector. Simulation results show encoder complexity reduction with negligible coding loss. Harmonized methods between the proposed one and CE4-3.4a, CE4-1.1 are also tested in CE4-3.3b, CE4-3.3c and CE4-3.3d. It is recommended to consider this method in the next VTM.

Reference

- Seungsoo Jeong, et al., “CE4 Ultimate motion vector expression (Test 4.5.4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0054, 12th meeting, Macao, CN, 3-10 Oct. 2018.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- Naeri Park, et al., “CE4-related : Candidates optimization on MMVD”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0307, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- Tomonori Hashimoto, et al., “CE4: Enhanced Merge with MVD (Test 4.4.4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0060, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- Eiichi Sasaki, et al., “Non-CE4: Syntax change of MMVD”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0069, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).