| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0442 |
| --- | --- |

| Title: | Non-CE9: Conditions fix for DMVR and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, hm.jang, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM4.0, SAD instead of mean-removed SAD as a cost function was adopted with DMVR module considering the decoding complexity. However, DMVR using SAD is not considered to be a desirable when GBi index is not “DEFAULT” and when weighting flags by the explicit weighted prediction are not zero. In this contribution, conditions for DMVR are fixed considering the GBi index and weighting flag of explicit weighted prediction. Also, same condition is applied for BDOF. The experimental results reportedly show 0.00% BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM4.0 as anchor in RA configurations.

Introduction

DMVR was adopted in VTM4.0 [1]. DMVR process is performed when all of the following conditions are satisfied as described in VVC Draft [2].

Table 1. Current DMVR conditions

- sps_dmvr_enabled_flag is equal to 1 - merge_flag[ xCb ][ yCb ] is equal to 1 - both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1 - mmvd_flag[ xCb ][ yCb ] is equal to 0 - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic ) - cbHeight is greater than or equal to 8 - cbHeight*cbWidth is greater than or equal to 64

Also, BDOF process is performed when all of the following conditions are satisfied as described in VVC Draft [2].

Table 2. Current BDOF conditions

- sps_bdof_enabled_flag is equal to 1. - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1. - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0. - MotionModelIdc[ xCb ][ yCb ] is equal to 0. - merge_subblock_flag[ xCb ][ yCb ] is equal to 0. - GbiIdx[ xCb ][ yCb ] is equal to 0. - cIdx is equal to 0.

Proposed method

Current DMVR, by comparing the SAD of the reference blocks to be weighted averaged later might search the un-matching block. In this proposal, for block whose GBi index is not “DEFAULT”, DMVR is not performed. Because, in this case, two references may have different weighting factors each other. Also, for block whose weighting flags by the explicit weighted prediction are not zero, DMVR is not performed. Table 2 shows the proposed condition which is yellow marked.

Table 3. Proposed DMVR conditions

- sps_dmvr_enabled_flag is equal to 1 - merge_flag[ xCb ][ yCb ] is equal to 1 - both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1 - mmvd_flag[ xCb ][ yCb ] is equal to 0 - GbiIdx[ xCb ][ yCb ] is equal to 0 - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0 - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic ) - cbHeight is greater than or equal to 8 - cbHeight*cbWidth is greater than or equal to 64

For BDOF, GBi is considered but weighting flags of the explicit weighted prediction are not considered. Table 3 shows the proposed condition which is yellow marked.

Table 4. Proposed BDOF conditions

- sps_bdof_enabled_flag is equal to 1. - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1. - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0. - MotionModelIdc[ xCb ][ yCb ] is equal to 0. - merge_subblock_flag[ xCb ][ yCb ] is equal to 0. - GbiIdx[ xCb ][ yCb ] is equal to 0. - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0. - cIdx is equal to 0.

Experimental results

The proposed method has been implemented on top of the VTM4.0 with default setting and tested using JVET SDR common test condition [3]. The experimental results reportedly show 0.00% BD-rate changes without encoding/decoding complexity changes compared to VTM 4.0 as Anchor in RA.

Table 5. Experimental results on VTM4.0 RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.06% | 101% | 100% |
| Class A2 | 0.02% | 0.10% | 0.10% | 101% | 100% |
| Class B | -0.01% | 0.03% | -0.01% | 100% | 100% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.02% | 100% | 100% |
| Class D | 0.02% | -0.09% | -0.08% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.08% | 100% | 100% |

Conclusion

In this contribution, conditions for DMVR and BDOF are fixed considering the GBi index and weighting flag of the explicit weighted prediction. The experimental results reportedly show 0.00% BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM4.0 as anchor in RA configurations. It is recommended to consider this method in the next VTM.

Reference

- Sriram Sethuraman, et al., “CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0147, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- Benjamin Bross, et al., “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).