| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N443 |
| --- | --- |

| Title: | Non-CE9 : BDOF processing considering assumptions of the optical flow |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, hm.jang, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM4.0, BDOF is performed when the current CU has true-bi prediction condition. Considering that the optical flow equation is designed to predict motion of objects, which are moving with the constant speed (i.e. motion amount), the current true bi condition is not optimum to be considered for applying BDOF. In this contribution, conditions for BDOF are fixed considering the distance of the reference pictures. The experimental results reportedly show 0.01% BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM4.0 as anchor in RA configurations.

Introduction

BDOF was designed to enhance the performance of the motion compensation using the optical flow concept. According to BIO introduced in [1], it assumes that objects are moving with constant speed (steady motion), as shown in Figure 1 and luminance of each pixel does not change while the object is moving. These assumptions lead the equation of optical flow as below.

Figure 1. Steady motion

Δ(i, j) = A – B = I(x + δx, y + δy, t + δt) - I(x - δx, y - δy, t – δt)

Proposed method

In VTM4.0, BDOF is performed when the current CU has true-bi prediction condition [2]. However, this condition does not mean that the object moves at a constant speed. In this proposal, BDOF is performed when distance of the reference pictures as a δt in equation is same.

By this proposal, spec text can be changed as follows.

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- Otherwise, bdofFlag is set equal to FALSE.

Experimental results

The proposed method has been implemented on top of the VTM4.0 with default setting and tested using JVET SDR common test condition [3]. The experimental results reportedly show 0.01% BD-rate changes without encoding/decoding complexity changes compared to VTM 4.0 as Anchor in RA.

Table 1. Experimental results on VTM4.0 RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 101% | 100% |
| Class B | 0.02% | -0.05% | -0.03% | 100% | 100% |
| Class C | 0.02% | -0.06% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.03% | -0.02% | 100% | 100% |
| Class D | 0.05% | -0.05% | -0.07% | 100% | 100% |
| Class F | 0.00% | 0.00% | -0.02% | 100% | 100% |

Conclusion

In this contribution, BDOF is performed considering the distance of the reference pictures. The experimental results reportedly show 0.01% BD-rate changes with 100% and 100% encoding and decoding run-time compared to VTM4.0 as anchor in RA configurations. It is recommended to consider this method in the next VTM.

Reference

- A. Alshin, E. Alshina, T. Lee, “BI-DIRECTIONAL OPTICAL FLOWFOR IMPROVING MOTION COMPENSATION”, 28th Picture Coding Symposium, PCS2010, December 8-10, 2010, Nagoya, Japan.

- Xiaoyu Xiu, et al., “CE9-related: Complexity reduction and bit-width control for bi-directional optical flow (BIO)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0256, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).