| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0444 |
| --- | --- |

| Title: | Non-CE9 : Mismatch between text specification and reference software on BDOF and DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, hm.jang, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution provides specification changes aligned with VTM4.0 software on BDOF and DMVR. In this document, block size restriction conditions for BDOF is fixed same as VTM4.0. In Geneva meeting, it is adopted that allowing only uni-prediction when the block size is 4x8 or 8x4 block. Therefore, in this contribution, this modification is reflected on DMVR and BDOF.

Introduction

In current VVC WD4 [1], mismatches between specification text and reference software on BDOF and DMVR are found.

Proposed method

#1. Mismatch on BDOF

According to the VTM4.0 and JVET-L0256, block size restriction is added as follows in 8.5.7.1. Changes are highlighted by yellow color.

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

- Otherwise, bdofFlag is set equal to FALSE.

In Geneva meeting, it is adopted that allowing only uni-prediction when the block size is 4x8 or 8x4 block. Therefore, in this contribution, this modification is reflected on DMVR and BDOF. Considering the modification, following text is changed for BDOF.

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- cbHeight is greater than or equal to 8

- cbHeight is greater than 4

- Otherwise, bdofFlag is set equal to FALSE.

Also, for DMVR, condition is modified as follows.

- sps_dmvr_enabled_flag is equal to 1

- merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- cbHeight is greater than 4

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

- -

Conclusion

In this contribution, mismatches between specification and software are fixed. It is recommended to consider these changes into the next WD.

Reference

- Benjamin Bross, et al., “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).