| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0444 |
| --- | --- |

| Title: | Non-CE9 : Mismatch between text specification and reference software on BDOF and DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, hm.jang, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution provides specification changes aligned with VTM4.0 software on BDOF and DMVR. In this document, block size restriction conditions for BDOF and SAD threshold for DMVR are fixed same as VTM4.0.

Introduction

In current VVC WD4 [1], mismatches between specification text and reference software on BDOF and DMVR are found.

Proposed method

#1. Mismatch on BDOF

According to the VTM4.0 and JVET-L0256, block size restriction is added as follows in 8.5.7.1. Changes are highlighted by yellow color.

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- cIdx is equal to 0.

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 64

- Otherwise, bdofFlag is set equal to FALSE.

#2. Mismatch on DMVR

According to the VTM4.0 and JVET-M0147, SAD threshold is changed as follows in 8.5.3.1. Changes are highlighted by yellow color.

#### 8.5.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock in luma samples,

- a variable sbHeight specifying the height of the current coding subblock in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvL0 and mvL1,

- the selected luma reference picture sample arrays refPicL0L and refPicL1L.

Outputs of this process are:

- delta luma motion vectors dMvL0 and dMvL1.

The variable subPelFlag is set to 0. and the variables srRange, offsetH0, offsetH1, offsetV0, and offsetV1 are all set equal to 2.

Both components of the delta luma motion vectors dMvL0 and dMvL1 are set equal to zero and modified as follows:

- For each X being 0 or 1, the ( sbWidth + 2 * srRange ) x ( sbHeight + 2 * srRange ) array predSamplesLXL of prediction luma sample values is derived by invoking the fractional sample bilinear interpolation process specified in 8.5.3.2.1 with the luma location ( xSb, ySb ), the prediction block width set equal to ( sbWidth + 2 * srRange ), the prediction block height set equal to ( sbHeight + 2 * srRange ), the reference picture sample array refPicLXL, the motion vector mvLX and the refinement search range srRange as inputs.

- The list sadList[ i ] with i = 0..8 is derived by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- When sadList[ 4 ] is greater than or equal to 4 * (sbHeight >> 1) * sbWidth, the following applies:

- The variable bestIdx is derived by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1.

- Otherwise, the following applies:

dX = bestIdx % 3 − 1		(8-436)

dY = bestIdx / 3 − 1		(8-437)

dMvL0[ 0 ] += 16 * dX		(8-438)

dMvL0[ 1 ] += 16 * dY		(8-439)

offsetH0 += dX		(8-440)

offsetV0 += dY		(8-441)

offsetH1 −= dX		(8-442)

offsetV1 −= dY		(8-443)

- The list sadList[ i ] with i = 0..8 is modifed by invoking the sum of absolute differences calculation process specified in 8.5.3.3 with sbWidth, sbHeight, offsetH0, offsetH1, offsetV0, offsetV1, predSamplesL0L and predSamplesL1L as inputs.

- The variable bestIdx is modified by invoking the array entry selection process specified in clause 8.5.3.4 with the list sadList[ i ] with i = 0..8 as input.

- If bestIdx is equal to 4, subPelFlag is set equal to 1

- Otherwise (bestIdx is not equal to 4), the following applies:

dMvL0[ 0 ] += 16 * ( bestIdx % 3 − 1 )		(8-444)

dMvL0[ 1 ] += 16 * ( bestIdx / 3 − 1 )		(8-445)

- When subPelFlag is equal to 1, the parametric motion vector refinement process specified in clause 8.5.3.5 is invoked with the list sadList[ i ] with i = 0..8, and the delta motion vector dMvL0 as inputs and the modified dMvL0 as output.

- The delta motion vector dMvL1 is derived as follows:

dMvL1[ 0 ] = −dMvL0[ 0 ]		(8-446)

dMvL1[ 1 ] = −dMvL0[ 1 ]		(8-447)

Conclusion

In this contribution, mismatches between specification and software are fixed. It is recommended to consider these changes into the next WD.

Reference

- Benjamin Bross, et al., “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).