| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, Switzerland, 19-27 March 2019 | Document: JVET-N0446 |
| --- | --- |

| Title: | New Adaptive Streaming Test Conditions for the VVC development |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mariana Afonso Andrey Norkin Joel Sole Kyle Swanson Anne Aaron Jungsun Kim Krasimir Kolarov David Singer Alexis M. Tourapis Yan Ye Wenfei Jiang | Tel: Email: | mafonso@netflix.com anorkin@netflix.com jsole@netflix.com kyleswanson@netflix.com aaaron@netflix.com jungsun_kim@apple.com kolarov@apple.com singer@apple.com atourapis@apple.com yan.ye@alibaba-inc.com wenfei.jwf@taobao.com |
| Source: | Netflix Inc, Apple Inc, Alibaba Inc |  |  |

_____________________________

Abstract

This document describes an updated proposal for the Adaptive Streaming test conditions for VTM, originally proposed in the document JVET-M0466. These test conditions are intended to simulate a video streaming scenario where streams with different bitrates, qualities, and spatial resolutions can be adaptively selected for the same video content. More specifically, two sets of test conditions, AS-RA and MR-RA are proposed. Experimental results between VTM 4.0, VTM 3.0 and HM 16.19 demonstrate that BD-rate gains when using AS-RA/MR-RA test conditions can be different from the BD-rate gains on RA CTC. In addition, tool off experiments using two adopted tools, ALF and SAO are presented. It is proposed to adopt one of the proposed test conditions as part of the CTC to better cover the adaptive streaming use case.

1. AS-RA

The Adaptive Streaming Random Access (AS-RA) test conditions are composed of 4 rate-points per test sequence, each containing a different ISclice QP value and spatial resolution. These rate-points were obtained using an optimization methodology described in JVET-M0466.

1.1 Updated rate-quality-resolution points for AS-RA

This document updates our previous proposal, outlined in JVET-M0466 [1]. The updated rate-points for AS-RA, using VTM 4.0 are shown in Table 1.1 for class B and Table 1.2 for classes A1 and A2. In order to better approximate the adaptive streaming conditions at lower bitrates, an additional smaller resolution, 960x540, was added to the choices of lower resolutions for the test sequences of class A.

Table 1.1 Specification of AS-RA rate-points for Class B (1080p original resolution)

| Sequence name | Resolution | QP | Bitrate (Kbps) | Sequence name | Resolution | QP | Bitrate (Kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MarketPlace | 1920×1080 | 27 | 5336 | BasketballDrive | 1920×1080 | 27 | 4645 |
|  | 1920×1080 | 32 | 2359 |  | 1920×1080 | 32 | 2185 |
|  | 1280×720 | 34 | 988 |  | 1920×1080 | 40 | 725 |
|  | 960×540 | 37 | 369 |  | 1280×720 | 45 | 217 |
| RitualDance | 1920×1080 | 27 | 4571 | BQTerrace | 1920×1080 | 27 | 4987 |
|  | 1920×1080 | 32 | 2419 |  | 1920×1080 | 32 | 1427 |
|  | 1280×720 | 34 | 1218 |  | 1920×1080 | 37 | 593 |
|  | 960×540 | 38 | 511 |  | 1920×1080 | 42 | 279 |
| Cactus | 1920×1080 | 27 | 3868 |  |  |  |  |
|  | 1920×1080 | 32 | 1774 |  |  |  |  |
|  | 1920×1080 | 39 | 637 |  |  |  |  |
|  | 1280×720 | 42 | 239 |  |  |  |  |

Table 1.2 Specification of AS-RA rate-points for Classes A1 and A2 (2160p original resolution)

| Sequence name | Resolution | QP | Bitrate (Kbps) | Sequence name | Resolution | QP | Bitrate (Kbps) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tango2 | 2560×1440 | 22 | 10745 | DaylightRoad2 | 3840×2160 | 27 | 9280 |
|  | 2560×1440 | 27 | 4486 |  | 3840×2160 | 31 | 4532 |
|  | 1280×720 | 31 | 1279 |  | 2560×1440 | 35 | 1558 |
|  | 960×540 | 37 | 449 |  | 1920×1080 | 39 | 621 |
| ParkRunning3 | 2560×1440 | 29 | 16188 | CatRobot1 | 3840×2160 | 27 | 8075 |
|  | 1920×1080 | 31 | 6994 |  | 3840×2160 | 32 | 3915 |
|  | 1920×1080 | 35 | 3332 |  | 2560×1440 | 35 | 1694 |
|  | 1280×720 | 37 | 1104 |  | 1920×1080 | 38 | 804 |
| FoodMarket4 | 3840×2160 | 25 | 11155 | Campfire | 3840×2160 | 27 | 16448 |
|  | 2560×1440 | 27 | 4727 |  | 2560×1440 | 27 | 7891 |
|  | 1920×1080 | 32 | 1735 |  | 1280×720 | 27 | 3272 |
|  | 960×540 | 37 | 386 |  | 1280×720 | 34 | 1420 |

1.2 Updated simulation parameters and methodology

As argued in the original proposal document, JVET-M0466, real-world adaptive streaming applications typically use different parameters compared to the current Random Access CTC outlined in document JVET-M1010 [2]. In particular, closed GOP is used to better facilitate chunk-based encoding and switching between different resolutions at random access points. In addition, adaptive streaming applications usually use longer intra-picture period than the 1s in the current RA CTC.

For these reasons, in AS-RA, the following testing methodologies or encoding parameters are applied:

- closed-GOP (GOP1: IBB...P / GOP2: IBB...P): this can be realised by the encoding of non-overlapping chunks of the input video and the subsequent stitching of the resulting bitstreams.

- 2-second Intra Periods, i.e. value 48 shall be used for a sequence with a frame rate equal to 24fps, 64 for 30fps, 96 for 50fps, 128 and for 60fps.

We used the Lanczos filter with 3 lobes (alpha = 3) in our simulations for both upscaling and downscaling, although this does not preclude the use of other scaling filters. The implementation in the open-source FFMPEG [3] was used for the simulations. Other alternatives include the implementation in HDRTools [4].

2. MR-RA

The Multi-Resolution Random Access (MR-RA) test conditions are composed of 16 rate-points per test sequence, using a fixed set of QP values: 22, 27, 32 and 37, and 4 different spatial resolutions. The spatial resolutions used depend on the class, A1, A2 or B and correspond to 3840x2160, 2560x1440, 1920x1080 and 1280x720 for classes A1 and A2, and 1920x1080, 1280x720, 960x540 and 640x360 for class B. BD-rates can be computed using the rate-points of each resolution separately or for the rate-points that are part of the convex hull. In this document, we report the MR-RA results on the convex hull points. For more information, please refer to JVET-M0466.

3. Experimental results between VTM versions and HM

3.1 VTM 4.0 vs VTM 3.0

Experiments to assess the performance difference between versions of VTM using AS-RA and MR-RA were conducted and this is compared with the results obtained using the conventional CTC. BD-rate results using the average PSNR value (as defined in the CTC, JVET-M1010) are shown in Table 3.1.

It is clear that savings indicated by AS-RA are different than the RA CTC. For example, for Campfire, BD-PSNR-Y is -1.52 % using AS-RA versus -6.15 % for the RA CTC. Differences are also reflected in different saving for the chroma channels.

Table 3.1 BD-rate between VTM 4.0 and VTM 3.0 using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  | MR-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Seq. | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | -5.95 | 8.82 | 6.48 | -4.79 | 6.25 | 4.11 | -4.86 | 4.23 | 3.59 |
|  | FoodMarket4 | -4.22 | 4.38 | 3.99 | -4.66 | 4.94 | 6.08 | -4.64 | 3.10 | 2.87 |
|  | Campfire | -6.15 | 6.08 | 0.67 | -1.52 | 1.40 | 4.95 | -3.77 | 5.06 | -0.17 |
|  | Average | -5.44 | 6.43 | 3.71 | -3.66 | 4.20 | 5.05 | -4.42 | 4.13 | 2.10 |
| Class A2 | CatRobot1 | -5.23 | 3.96 | 5.11 | -6.12 | 1.24 | 1.71 | -6.12 | 1.36 | 2.79 |
|  | DaylightRoad2 | -6.09 | 6.55 | 6.86 | -6.63 | 0.64 | 1.75 | -6.43 | 4.89 | 7.02 |
|  | ParkRunning3 | -9.24 | 6.16 | 6.74 | -6.06 | 2.47 | 2.64 | -6.66 | 4.87 | 5.49 |
|  | Average | -6.85 | 5.56 | 6.24 | -6.27 | 1.45 | 2.03 | -6.40 | 3.70 | 5.10 |
| Class B | MarketPlace | -7.33 | 3.58 | 1.94 | -3.71 | 16.74 | 16.56 | -1.78 | 10.99 | 8.51 |
|  | RitualDance | -4.76 | 4.04 | 4.90 | -5.28 | 10.17 | 12.15 | -3.09 | 6.19 | 8.36 |
|  | Cactus | -3.70 | 4.85 | 4.42 | -3.92 | 1.52 | 1.91 | -3.68 | 1.94 | 2.63 |
|  | BasketballDrive | -4.90 | 6.93 | 7.57 | -5.79 | 8.70 | 10.47 | -3.69 | 9.93 | 9.79 |
|  | BQTerrace | -4.60 | 10.99 | 8.22 | -4.39 | 7.67 | 4.84 | -5.40 | 5.01 | 6.65 |
|  | Average | -5.06 | 6.08 | 5.41 | -4.62 | 8.96 | 9.18 | -3.53 | 6.81 | 7.19 |
| Overall Average |  | -5.65 | 6.03 | 5.17 | -4.81 | 5.61 | 6.11 | -4.56 | 5.23 | 5.23 |

3.2 VTM 4.0 vs HM 16.19

We also report the compression efficiency comparison between VTM 4.0 and HM 16.19 using multiple test conditions. This is presented in Table 3.2. Differences in BD-rate savings can be observed. For instance, for class A1, an average BD-rate of -27.46 % is obtained for AS-RA compared to -32.65 % reported using the JVET RA CTC.

Table 3.2 BD-rate between VTM 4.0 and HM 16.19 using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  | MR-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Seq. | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | -34.57 | -49.75 | -47.42 | -29.18 | -43.36 | -37.71 | -28.85 | -48.96 | -43.85 |
|  | FoodMarket4 | -34.05 | -32.94 | -35.92 | -31.05 | -31.39 | -30.98 | -30.14 | -35.07 | -35.31 |
|  | Campfire | -29.34 | -34.61 | -42.86 | -22.14 | -18.47 | -30.87 | -24.32 | -32.28 | -40.95 |
|  | Average | -32.65 | -39.10 | -42.07 | -27.46 | -31.07 | -33.19 | -27.77 | -38.77 | -40.03 |
| Class A2 | CatRobot1 | -40.26 | -46.15 | -39.99 | -40.38 | -45.56 | -40.64 | -39.68 | -45.03 | -39.94 |
|  | DaylightRoad2 | -40.51 | -41.81 | -33.47 | -39.96 | -46.72 | -36.52 | -39.91 | -44.96 | -37.16 |
|  | ParkRunning3 | -31.53 | -18.89 | -16.40 | -25.96 | -22.82 | -19.69 | -23.30 | -19.85 | -17.41 |
|  | Average | -37.43 | -35.62 | -29.95 | -35.44 | -38.37 | -32.28 | -34.29 | -36.61 | -31.50 |
| Class B | MarketPlace | -32.06 | -33.91 | -40.09 | -29.28 | -27.71 | -28.04 | -28.81 | -32.15 | -33.57 |
|  | RitualDance | -27.99 | -30.08 | -37.09 | -27.14 | -29.30 | -31.95 | -26.57 | -31.84 | -34.86 |
|  | Cactus | -32.93 | -39.15 | -31.87 | -36.29 | -44.90 | -41.35 | -32.50 | -39.60 | -34.96 |
|  | BasketballDrive | -32.50 | -41.11 | -41.53 | -35.29 | -43.09 | -39.45 | -29.14 | -40.47 | -37.55 |
|  | BQTerrace | -31.07 | -40.71 | -40.11 | -32.91 | -45.74 | -50.67 | -26.67 | -39.38 | -42.59 |
|  | Average | -31.31 | -36.99 | -38.14 | -32.18 | -38.15 | -38.29 | -28.74 | -36.69 | -36.71 |
| Overall Average |  | -33.35 | -37.19 | -36.98 | -31.78 | -36.28 | -35.26 | -29.99 | -37.24 | -36.19 |

4. Complexity analysis

The following complexity analysis considers the encoding time required for the AS-RA and MR-RA test conditions compared to the CTC. Both AS-RA and MR-RA consider only classes A1, A2 and B and Random Access main10 configuration. Results measured by the total encoding time (for all test rate-points) are shown in Table 4.1. The percentage between the proposed test conditions and the CTC are also shown in the last two columns.

The total encoding time required for AS-RA is approximately 44% of the required for the current RA CTC for the tested classes. In case of MR-RA, this figure is approximately 178%. When considering all RA classes: A1, A2, B, C, D and F, these numbers are reduced to 38% and 168% for AS-RA and MR-RA, respectively.

Table 4.1 Total encoding time required for the AS-RA, MR-RA and the JVET RA CTC

|  |  | JVET RA CTC | AS-RA | MR-RA | AS-RA | MR-RA |
| --- | --- | --- | --- | --- | --- | --- |
| Class | Sequence | Total Enc T (h) | Total Enc T (h) | Total Enc T (h) | Enc T [%] | Enc T [%] |
| Class A1 | Tango2 | 437 | 136 | 744 | 31.1% | 170.3% |
|  | FoodMarket4 | 347 | 189 | 654 | 54.5% | 188.6% |
|  | Campfire | 680 | 291 | 1216 | 42.8% | 178.8% |
|  | Total | 1464 | 616 | 2614 | 42.1% | 178.6% |
| Class A2 | CatRobot1 | 371 | 195 | 647 | 52.6% | 174.5% |
|  | DaylightRoad2 | 482 | 226 | 795 | 46.9% | 165.0% |
|  | ParkRunning3 | 680 | 141 | 1225 | 20.7% | 180.0% |
|  | Total | 1533 | 562 | 2667 | 36.7% | 174.0% |
| Class B | MarketPlace | 226 | 124 | 426 | 55.0% | 188.8% |
|  | RitualDance | 210 | 135 | 449 | 64.1% | 213.3% |
|  | Cactus | 179 | 127 | 310 | 71.1% | 173.5% |
|  | BasketballDrive | 241 | 132 | 428 | 54.9% | 178.0% |
|  | BQTerrace | 229 | 105 | 358 | 45.8% | 156.2% |
|  | Total | 1084.52 | 623.00 | 1971.00 | 57.4% | 181.7% |
| Overall total |  | 4080.96 | 1801.00 | 7252.00 | 44.1% | 177.7% |

Furthermore, considering that the encoding is performed in parallel for different chunks of the input test videos and considering the encoding time required for the other configurations, All Intra and Low Delay in the CTC, the addition of AS-RA would not increase the longest time for encoding a video sequence under CTC.

5. Experimental results - ALF and SAO - Tool off

In order to understand the impact of using AS-RA for individual tools and the benefit they bring to the adaptive streaming use case, tool off experiments using two adopted tools in VTM 4.0 were conducted. In these tests, the anchor codec is VTM 4.0 and the test codec is VTM 4.0 without the specific tool. Two tools were considered: Adaptive Loop Filter (ALF) and Sample Adaptive Offset (SAO).

5.1 ALF off

Table 5.1 presents the results of the ALF off tests. On average, by employing the CTC for the tested classes, the codec with ALF off achieves BD-rate losses of 4.28 %. However, when using AS-RA as the test condition, this loss is reduced to nearly half, 2.13 %. Therefore, although significant benefits might be achieved by enabling ALF for the broadcast scenario, the reported results indicate that a reduced benefit is expected in the adaptive streaming case.

Table 5.1 BD-rate between VTM 4.0 (anchor) and VTM 4.0 with ALF off using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Sequence | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | 4.39 | 3.64 | 1.50 | 1.66 | -0.97 | -1.86 |
|  | FoodMarket4 | 3.67 | 1.57 | 0.81 | 2.26 | 0.66 | 0.06 |
|  | Campfire | 3.20 | 0.70 | 2.22 |  |  |  |
|  | Average | 3.75 | 1.97 | 1.51 | 1.96 | -0.16 | -0.90 |
| Class A2 | CatRobot1 | 4.48 | 1.19 | 2.66 | 2.03 | -0.84 | -0.21 |
|  | DaylightRoad2 | 4.68 | 1.32 | 3.73 | 1.26 | -1.01 | -0.95 |
|  | ParkRunning3 | 7.01 | 3.24 | 2.32 | 1.00 | 3.85 | 3.24 |
|  | Average | 5.39 | 1.92 | 2.90 | 1.43 | 0.67 | 0.70 |
| Class B | MarketPlace | 3.70 | 1.67 | 2.44 | 2.06 | -1.00 | -0.61 |
|  | RitualDance | 2.04 | 0.96 | 0.39 | 1.10 | -0.03 | -0.51 |
|  | Cactus | 3.48 | 0.89 | 2.10 | 2.81 | -0.32 | 0.42 |
|  | BasketballDrive | 3.81 | 5.61 | 6.07 | 2.45 | 1.49 | 1.73 |
|  | BQTerrace | 6.64 | 20.84 | 10.34 | 4.71 | 8.70 | 4.30 |
|  | Average | 3.93 | 5.99 | 4.27 | 2.62 | 1.77 | 1.07 |
| Overall Average |  | 4.28 | 3.78 | 3.14 | 2.13 | 1.05 | 0.56 |

5.2 SAO off

Furthermore, Table 5.2 presents the results of the SAO off tests. It can be observed that the overall average BD-rate losses using CTC is similar to AS-RA, 0.62 % compared to 0.56 % using AS-RA. However, for individual sequences, differences can be found. For instance, for DaylightRoad2, the BD-rate is 1.12 % by using the CTC compared to 0.70 % using AS-RA. Nonetheless, overall, enabling SAO is likely to be beneficial for the adaptive streaming use case.

Table 5.2 BD-rate between VTM 4.0 (anchor) and VTM 4.0 with SAO off using the JVET CTC, AS-RA and MR-RA test conditions for classes A1, A2 and B.

|  |  | JVET RA CTC |  |  | AS-RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class | Sequence | Y [%] | U [%] | V [%] | Y [%] | U [%] | V [%] |
| Class A1 | Tango2 | 0.08 | 0.52 | 0.32 | 0.27 | -0.12 | 0.34 |
|  | FoodMarket4 | 0.17 | 0.10 | -0.02 | 0.36 | 0.38 | 0.10 |
|  | Campfire | 0.28 | 0.50 | 0.90 | 0.37 | 0.03 | 0.47 |
|  | Average | 0.18 | 0.37 | 0.40 | 0.33 | 0.10 | 0.30 |
| Class A2 | CatRobot1 | 1.07 | 3.70 | 3.08 | 0.88 | 1.61 | 1.90 |
|  | DaylightRoad2 | 1.12 | 0.30 | 0.20 | 0.70 | 0.11 | 0.09 |
|  | ParkRunning3 | 0.60 | 0.41 | 0.40 | 0.58 | -0.15 | 0.16 |
|  | Average | 0.93 | 1.47 | 1.23 | 0.72 | 0.52 | 0.72 |
| Class B | MarketPlace | 0.47 | 0.48 | 0.95 | 0.44 | 0.01 | 0.51 |
|  | RitualDance | 0.24 | 0.19 | 0.20 | 0.20 | -0.04 | 0.19 |
|  | Cactus | 0.72 | 1.46 | 0.93 | 0.58 | 0.12 | 0.56 |
|  | BasketballDrive | 0.31 | 0.66 | 0.47 | 0.20 | 0.76 | 0.16 |
|  | BQTerrace | 1.73 | 1.20 | 1.62 | 1.61 | 0.83 | 1.19 |
|  | Average | 0.69 | 0.80 | 0.83 | 0.61 | 0.34 | 0.52 |
| Overall Average |  | 0.62 | 0.87 | 0.82 | 0.56 | 0.32 | 0.52 |

References

[1] M. Afonso, A. Norkin, A. Aaron, J. Sole, K. Swanson, Y. Ye, W. Jiang, J. Kim, K. Kolarov, D. Singer, A. M. Tourapis, “Adaptive Streaming Test Conditions for VTM” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0466, Marrakesh, Morocco, January 2019.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-M0010, Marrakesh, Morocco, January 2019.

[3] FFMPEG, https://ffmpeg.org/

[4] HDRTools, https://gitlab.com/standards/HDRTools

Results reported in JVET-N0003