| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0447-v1 |
| --- | --- |

| Title: | Non-CE4: On triangle merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Chun-Chi Chen, Wei-Jung Chien, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, chunchic, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In current VTM, the maximum number of triangle merge candidates in uni-prediction candidate list is specified as five without an indicator in higher-level syntax. This contribution proposes to add a tile group syntax element to allow the maximum number of triangle merge candidates to be in the range of 2 to 5, inclusive. As the maximum number of triangle merge candidates specified as 2, it is further proposed to skip the signaling of the merge candidate index for the second triangle block, i.e., merge_triangle_idx1, because merge_triangle_idx1 must be an opposite index of the merge candidate index for the first triangle block, i.e., merge_triangle_idx0. Compared to VTM-4.0 with the number of triangle merge candidates specified as 2, the proposed signaling skip method shows -0.06%/-0.17%/-0.12% YUV-BDrate with 100/100% encoder/decoder runtime for RA configuration, and -0.xx%/-0.xx%/-0.xx% YUV-BDrate with 100/100% encoder/decoder runtime for LB configuration.

# Introduction

VVC introduces a triangle partition mode for inter prediction to evenly split a CU into two triangle-shaped partitions. A triangle merge candidate list is constructed with 5 triangle merge candidates. Each triangle-shaped partition block selects its own triangle merge candidate from the triangle merge candidate list. In current VTM, the number of triangle merge candidates is enforced to be five. There is no higher-level syntax element to define the maximum number of triangle merge candidates. Note that there are already tile group-level indexes to define the maximum number of normal merge candidates MaxNumMergeCand and the maximum number of subblock merge candidates MaxNumSubblockMergeCand by the syntax elements six_minus_max_num_merge_cand and five_minus_max_num_subblock_merge_cand. The smaller number of merge candidates leads to lower complexity at encoder and decoder side. It is desirable the VVC can support to flexibly set the maximum number of triangle merge candidates, defined as MaxNumTriangleMergeCand. Therefore, this contribution proposes a tile group flag to allow MaxNumTriangleMergeCand to be in the range of 2 to 5, inclusive. To further reduce the signaling overhead in the case of MaxNumTriangleMergeCand = 2, it is further proposed to skip the signaling of the merge candidate index for the second triangle block, i.e., merge_triangle_idx1.

# Proposed design

It is proposed to add a tile-level index five_minus_max_num_triangle_merge_cand to indicate the maximum number of triangle merge candidates, MaxNumTriangleMergeCand. MaxNumTriangleMergeCand can be derived as follows: MaxNumTriangleMergeCand = 5 - five_minus_max_num_triangle_merge_cand, and the minimum value of MaxNumTriangleMergeCand could be defined as 2. Therefore, five_minus_max_num_triangle_merge_cand could be in the range of 0 to 3, inclusive. Another proposal is related to the case of MaxNumTriangleMergeCand = 2. In the current design, merge_triangle_idx0 and merge_triangle_idx1 are always parsed. It is proposed to skip the parsing of merge_triangle_idx1 as MaxNumTriangleMergeCand = 2 since merge_triangle_idx1 must be the opposite number of merge_triangle_idx0, i.e., merge_triangle_idx1 must be 0 as merge_triangle_idx0 = 1, and merge_triangle_idx1 must be 1 as merge_triangle_idx0 = 0. The proposed change to VVC Text 4 is shown in the table below with yellow change mark:

Section 7.3.4.1 tile_group_header ( ) {; Descriptor. …. if ( tile_group_type != I ) {. if( sps_temporal_mvp_enabled_flag ). tile_group_temporal_mvp_enabled_flag; u(1). if( tile_group_type = = B ). mvd_l1_zero_flag; u(1). if( tile_group_temporal_mvp_enabled_flag ) {. if( tile_group_type = = B ). collocated_from_l0_flag; u(1). }. if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ). pred_weight_table( ). six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_triangle_enabled_flag ). five_minus_max_num_triangle_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). tile_group_fpel_mmvd_enabled_flag; u(1). } else if ( sps_ibc_enabled_flag ). six_minus_max_num_merge_cand; ue(v). …. }. five_minus_max_num_triangle_merge_cand specifies the maximum number of triangle merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 5. The maximum number of merging MVP candidates, MaxNumTriangleMergeCand is derived as follows: MaxNumTriangleMergeCand = 5 − five_minus_max_num_triangle_merge_cand The value of MaxNumTriangleMergeCand shall be in the range of 2 to 5, inclusive. Section 7.3.6.8 in VVC Draft 4 merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). } else {. mmvd_flag[ x0 ][ y0 ]; ae(v). if( mmvd_flag[ x0 ][ y0 ] = = 1 ) {. mmvd_merge_flag[ x0 ][ y0 ]; ae(v). mmvd_distance_idx[ x0 ][ y0 ]; ae(v). mmvd_direction_idx[ x0 ][ y0 ]; ae(v). } else {. if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ). merge_subblock_flag[ x0 ][ y0 ]; ae(v). if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumSubblockMergeCand > 1 ). merge_subblock_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) {. ciip_flag[ x0 ][ y0 ]; ae(v). if( ciip_flag[ x0 ][ y0 ] ) {. if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ). ciip_luma_mpm_flag[ x0 ][ y0 ]; ae(v). if( ciip_luma_mpm_flag[ x0 ][ y0 ] ). ciip_luma_mpm_idx[ x0 ][ y0 ]; ae(v). }. }. if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ). merge_triangle_flag[ x0 ][ y0 ]; ae(v). if( merge_triangle_flag[ x0 ][ y0 ] ) {. merge_triangle_split_dir[ x0 ][ y0 ]; ae(v). merge_triangle_idx0[ x0 ][ y0 ]; ae(v). if(MaxNumTriangleMergeCand > 2). merge_triangle_idx1[ x0 ][ y0 ]; ae(v). } else if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). }. }. }. }.

# Simulation results

The following experiments were conducted using the VTM-4.0 under the common test conditions [1]. The anchor is VTM-4.0 with MaxNumTriangleMergeCand = 2. The proposed signal skipping method for merge_triangle_idx1 is tested, and the RA and LB results are shown in Table 1.

Table 1: Summary results over VTM-4.0 with maximum number of triangle merge candidates specified as 2.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.19% | -0.08% | 99% | 99% |
| Class A2 | -0.04% | -0.18% | -0.11% | 100% | 99% |
| Class B | -0.04% | -0.11% | -0.08% | 100% | 100% |
| Class C | -0.10% | -0.24% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.17% | -0.12% | 100% | 100% |
| Class D | -0.06% | 0.13% | 0.16% | 100% | 101% |
| Class F | -0.03% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.21% | -0.53% | -0.57% | 100% | 99% |
| Class E | -0.17% | 0.09% | -0.07% | 99% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.12% | -0.24% | -0.02% | 99% | 100% |
| Class F | -0.08% | -0.09% | 0.11% | 100% | 100% |

# Conclusion

A tile group syntax element five_minus_max_num_triangle_merge_cand is proposed to indicate the maximum number of triangle merge candidates, MaxNumTriangleMergeCand. This new parsing index benefits to the video codec especially video encoder to trade-off the complexity and bitrate. A parsing skip method is also proposed to skip the paring of merge_triangle_idx1 as MaxNumTriangleMergeCand specified as 2, and shows a good bitrate saving without encoder and decoder runtime change. It is recommended to adopt the proposals into VVC standard.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakech, MA.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).