| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0448-v1 |
| --- | --- |

| Title: | Non-CE4: On MMVD signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Wei-Jung Chien, Chun-Chi Chen, and Marta Karczewicz 10160 Pacific Mesa Blvd, San Diego, CA 92121 | Email: | {yjchang, wchien, chunchic, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In current VTM, the maximum number of MMVD merge candidates is specified as two. These MMVD merge candidates directly uses 2 available merge candidates derived in normal merge candidate list, and are used as MV basis for MV refinement in MMVD. However, it is ambiguous in the current VVC design if the maximum number of normal merge candidates is specified as 1, i.e, MaxNumMergeCand = 1. Therefore, this contribution presents three categories of solutions to address this issue. First category is to derive max(2, MaxNumMergeCand) normal merge candidates if MMVD_Flag is enabled, and derive MaxNumMergeCand normal merge candidates if MMVD_Flag is disabled. Second category is to set MaxNumMergeCand as 2 if the maximum number of MMVD merge candidates is specified as 2. Third category is to set the maximum number of MMVD merge candidates as 1 if MaxNumMergeCand = 1. A tile group flag max_num_mmvd_merge_base_cand could be added in category 2 and category 3 to indicate the number of MMVD merge candidates. The proposed solutions were experimented on top of VTM-4.0.1 with MaxNumMergeCand specified as 1. The solution in category 1 shows -0.xx%/-0.xx%/-0.xx% YUV-BDrate with 100/100% encoder/decoder runtime for RA configuration, and -0.xx%/-0.xx%/-0.xx% YUV-BDrate with 100/100% encoder/decoder runtime for LB configuration. The CU-level solution in category 3 shows -0.xx%/-0.xx%/-0.xx% YUV-BDrate with 100/100% encoder/decoder runtime for RA configuration, and -0.xx%/-0.xx%/-0.xx% YUV-BDrate with 100/100% encoder/decoder runtime for LB configuration.

# Introduction

The merge mode with motion vector differences (MMVD) is introduced in VVC. In MMVD, after a merge candidate is selected from the normal merge candidate list, it is further refined by the signaled MVDs information. The further information includes a merge candidate flag, an index to specify motion magnitude, and an index for indication of motion direction. In MMVD mode, one out of the first two normal merge candidates in the normal merge list is selected to be used as MV basis. The merge candidate flag is signaled to specify which one is used. However, it is ambiguous in the current design if the maximum number of normal merge candidates is specified as 1, i.e., MaxNumMergeCand = 1. Since the maximum number of MMVD merge candidates is enforced to be 2 in VTM, i.e., MMVD_BASE_MV_NUM = 2, the MV basis directly derived from the normal merge candidate list is not enough as MaxNumMergeCand = 1. This proposal provides several solutions to address the issue.

# Proposed solution

Solution 1: In the current VTM design, the maximum number of normal merging candidates is derived by the tile group index six_minus_max_num_merge_cand, i.e., MaxNumMergeCand = 6 - six_minus_max_num_merge_cand. The maximum number of MMVD merging candidates, i.e., the number of MV basis for MMVD is enforced to be set as 2. It is proposed, if MaxNumMergeCand = 1 and mmvd_flag enabled in this CU, the number of normal merging candidates in the mergeCandList for this CU is inferred to be 2, instead of MaxNumMergeCand; Otherwise, the number of normal merging candidates for this CU remains as MaxNumMergeCand.

Solution 2.1: It is proposed to add a tile group-level flag max_num_mmvd_merge_base_cand to specify the maximum number of MMVD merging candidates supported in the tile group subtracting 1. The maximum number of MMVD base merging candidates, MaxNumMmvdBaseMergeCand, is derived as follows:

MaxNumMmvdBaseMergeCand = 1 + max_num_mmvd_merge_base_cand

The value of MaxNumMmvdBaseMergeCand shall be in the range of 1 to 2, inclusive. It is further proposed to restrict the smallest number of six_minus_max_num_merge_cand to be MaxNumMmvdBaseMergeCand. This proposed change can guarantee at least two normal merging candidates can be created and provided for MMVD merging candidates as MaxNumMmvdBaseMergeCand = 2. A condition check to skip the parsing of mmvd_merge_flag is further proposed as MaxNumMmvdBaseMergeCand equal to 1. An SPS-level flag sps_mmvd_enabled_flag is also used to specify that the MMVD may be used in decoding of pictures in the CVS. A condition check sps_mmvd_enabled_flag could be added prior to mmvd_flag to reduce the redundant signaling overhead. A new tile group-level index six_minus_max_num_ibc_merge_cand is proposed to specify the number of merging cnadidates for IBC mode, i.e., MaxNumIbcMergeCand. This change can avoid the ambiguous between MaxNumMergeCand, MaxNumIbcMergeCand and MaxNumMmvdBaseMergeCand. The proposed change to VVC Text 4 is shown in the table below with yellow change mark:

Section 7.3.2.1 seq_parameter_set_rbsp( ) {; Descriptor. …. sps_temporal_mvp_enabled_flag; u(1). if( sps_temporal_mvp_enabled_flag ). sps_sbtmvp_enabled_flag; u(1). …. sps_ibc_enabled_flag; u(1). sps_ciip_enabled_flag; u(1). sps_fpel_mmvd_enabled_flag; u(1). sps_triangle_enabled_flag; u(1). sps_mmvd_enabled_flag; u(1). …. }. Section 7.3.4.1 tile_group_header ( ) {; Descriptor. …. if ( tile_group_type != I ) {. if( sps_temporal_mvp_enabled_flag ). tile_group_temporal_mvp_enabled_flag; u(1). if( tile_group_type = = B ). mvd_l1_zero_flag; u(1). if( tile_group_temporal_mvp_enabled_flag ) {. if( tile_group_type = = B ). collocated_from_l0_flag; u(1). }. if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ). pred_weight_table( ). if(sps_mmvd_enabled_flag). max_num_mmvd_merge_base_cand; u(1). six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). tile_group_fpel_mmvd_enabled_flag; u(1). } else if ( sps_ibc_enabled_flag ). six_minus_max_num_merge_cand; ue(v). if ( sps_ibc_enabled_flag ). six_minus_max_num_ibc_merge_cand; ue(v). …. }. max_num_mmvd_merge_base_cand specifies the maximum number of MMVD merging motion vector prediction (MVP) candidates supported in the tile group subtracting 1. The maximum number of merging MVP candidates, MaxNumMmvdBaseMergeCand is derived as follows: MaxNumMmvdBaseMergeCand = max_num_mmvd_merge_base_cand + 1 The value of MaxNumMmvdBaseMergeCand shall be in the range of 1 to 2, inclusive. six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows: MaxNumMergeCand = 6 − six_minus_max_num_merge_cand The value of MaxNumMergeCand shall be in the range of max_num_mmvd_merge_base_cand to 6, inclusive. six_minus_max_num_ibc_merge_cand specifies the maximum number of IBC merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 6. The maximum number of IBC merging MVP candidates, MaxNumIbcMergeCand is derived as follows: MaxNumIbcMergeCand = 6 − six_minus_max_num_ibc_merge_cand The value of MaxNumIbcMergeCand shall be in the range of 1 to 6, inclusive. Section 7.3.6.8 merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). if( MaxNumIbcMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_mmvd_enabled_flag ). mmvd_flag[ x0 ][ y0 ]; ae(v). if( mmvd_flag[ x0 ][ y0 ] = = 1 ) {. if(MaxNumMmvdBaseMergeCand > 1). mmvd_merge_flag[ x0 ][ y0 ]; ae(v). mmvd_distance_idx[ x0 ][ y0 ]; ae(v). mmvd_direction_idx[ x0 ][ y0 ]; ae(v). } else {. if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ). merge_subblock_flag[ x0 ][ y0 ]; ae(v). if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumSubblockMergeCand > 1 ). merge_subblock_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) {. ciip_flag[ x0 ][ y0 ]; ae(v). if( ciip_flag[ x0 ][ y0 ] ) {. if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ). ciip_luma_mpm_flag[ x0 ][ y0 ]; ae(v). if( ciip_luma_mpm_flag[ x0 ][ y0 ] ). ciip_luma_mpm_idx[ x0 ][ y0 ]; ae(v). }. }. if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ). merge_triangle_flag[ x0 ][ y0 ]; ae(v). if( merge_triangle_flag[ x0 ][ y0 ] ) {. merge_triangle_split_dir[ x0 ][ y0 ]; ae(v). merge_triangle_idx0[ x0 ][ y0 ]; ae(v). merge_triangle_idx1[ x0 ][ y0 ]; ae(v). } else if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). }. }. }. }.

Solution 2.2: An alternative solution to Solution 2.1 is proposed to disallow MaxNumMergeCand to be 1 as sps_mmvd_enabled_flag enabled. Compared to Solution 2.1, there is no new tile group syntax element. The proposed change to VVC Text 4 is shown in the table below with yellow change mark:

Section 7.3.4.1 tile_group_header ( ) {; Descriptor. …. if ( tile_group_type != I ) {. if( sps_temporal_mvp_enabled_flag ). tile_group_temporal_mvp_enabled_flag; u(1). if( tile_group_type = = B ). mvd_l1_zero_flag; u(1). if( tile_group_temporal_mvp_enabled_flag ) {. if( tile_group_type = = B ). collocated_from_l0_flag; u(1). }. if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ). pred_weight_table( ). six_minus_max_num_merge_cand; ue(v). if( sps_affine_enabled_flag ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). tile_group_fpel_mmvd_enabled_flag; u(1). } else if ( sps_ibc_enabled_flag ). six_minus_max_num_merge_cand; ue(v). …. }. six_minus_max_num_merge_cand specifies the maximum number of merging motion vector prediction (MVP) candidates supported in the tile group subtracted from 6. The maximum number of merging MVP candidates, MaxNumMergeCand is derived as follows: MaxNumMergeCand = 6 − six_minus_max_num_merge_cand The value of MaxNumMergeCand shall be in the range of 1+ sps_mmvd_enabled_flag to 6, inclusive.

Solution 3.1: Similar to Solution 2.1, the same tile group flag max_num_mmvd_merge_base_cand is added to specify the maximum number of MMVD merging candidates supported in the tile group subtracting 1. The same SPS flag sps_mmvd_enabled_flag is added as condition checks to reduce the signaling overhead. A tile group flag MaxNumMergeCand > 1 could be added as a condition check prior to the parsing of max_num_mmvd_merge_base_cand to further reduce the signaling overhead. The proposed change to VVC Text 4 is shown in the table below with yellow change mark:

Section 7.3.2.1 seq_parameter_set_rbsp( ) {; Descriptor. …. sps_temporal_mvp_enabled_flag; u(1). if( sps_temporal_mvp_enabled_flag ). sps_sbtmvp_enabled_flag; u(1). …. sps_ibc_enabled_flag; u(1). sps_ciip_enabled_flag; u(1). sps_fpel_mmvd_enabled_flag; u(1). sps_triangle_enabled_flag; u(1). sps_mmvd_enabled_flag; u(1). …. }. Section 7.3.4.1 tile_group_header ( ) {; Descriptor. …. if ( tile_group_type != I ) {. if( sps_temporal_mvp_enabled_flag ). tile_group_temporal_mvp_enabled_flag; u(1). if( tile_group_type = = B ). mvd_l1_zero_flag; u(1). if( tile_group_temporal_mvp_enabled_flag ) {. if( tile_group_type = = B ). collocated_from_l0_flag; u(1). }. if( ( weighted_pred_flag && tile_group_type = = P ) ¦ ¦ ( weighted_bipred_flag && tile_group = = B ) ). pred_weight_table( ). six_minus_max_num_merge_cand; ue(v). if( sps_mmvd_enabled_flag && MaxNumMergeCand > 1 ). max_num_mmvd_merge_base_cand; u(1). if( sps_affine_enabled_flag ). five_minus_max_num_subblock_merge_cand; ue(v). if( sps_fpel_mmvd_enabled_flag ). tile_group_fpel_mmvd_enabled_flag; u(1). } else if ( sps_ibc_enabled_flag ). six_minus_max_num_merge_cand; ue(v). …. }. max_num_mmvd_merge_base_cand specifies the maximum number of MMVD merging motion vector prediction (MVP) candidates supported in the tile group subtracting 1. If it is not present, it is inferred to be 0. The maximum number of merging MVP candidates, MaxNumMmvdBaseMergeCand is derived as follows: MaxNumMmvdBaseMergeCand = max_num_mmvd_merge_base_cand + 1 The value of MaxNumMmvdBaseMergeCand shall be in the range of 1 to 2, inclusive. Section 7.3.6.8 merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_mmvd_enabled_flag ). mmvd_flag[ x0 ][ y0 ]; ae(v). if( mmvd_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumMmvdBaseMergeCand > 1 ). mmvd_merge_flag[ x0 ][ y0 ]; ae(v). mmvd_distance_idx[ x0 ][ y0 ]; ae(v). mmvd_direction_idx[ x0 ][ y0 ]; ae(v). } else {. if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ). merge_subblock_flag[ x0 ][ y0 ]; ae(v). if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumSubblockMergeCand > 1 ). merge_subblock_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) {. ciip_flag[ x0 ][ y0 ]; ae(v). if( ciip_flag[ x0 ][ y0 ] ) {. if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ). ciip_luma_mpm_flag[ x0 ][ y0 ]; ae(v). if( ciip_luma_mpm_flag[ x0 ][ y0 ] ). ciip_luma_mpm_idx[ x0 ][ y0 ]; ae(v). }. }. if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ). merge_triangle_flag[ x0 ][ y0 ]; ae(v). if( merge_triangle_flag[ x0 ][ y0 ] ) {. merge_triangle_split_dir[ x0 ][ y0 ]; ae(v). merge_triangle_idx0[ x0 ][ y0 ]; ae(v). merge_triangle_idx1[ x0 ][ y0 ]; ae(v). } else if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). }. }. }. }.

Solution 3.2: Another alternative solution to Solution 3.1 is to implicitly infer the number of MMVD merge candidate to be 1 as MaxNumMergeCand = 1 (without tile group syntax change). The syntax element mmvd_merge_flag is inferred to be 0 if it is not present. The proposed change to VVC Text 4 is shown in the table below with yellow change mark:

Section 7.3.6.8 merge_data( x0, y0, cbWidth, cbHeight ) {; Descriptor. if ( CuPredMode[ x0 ][ y0 ] = = MODE_IBC ) {. if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). } else {. mmvd_flag[ x0 ][ y0 ]; ae(v). if( mmvd_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumMergeCand > 1 ). mmvd_merge_flag[ x0 ][ y0 ]; ae(v). mmvd_distance_idx[ x0 ][ y0 ]; ae(v). mmvd_direction_idx[ x0 ][ y0 ]; ae(v). } else {. if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 ). merge_subblock_flag[ x0 ][ y0 ]; ae(v). if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) {. if( MaxNumSubblockMergeCand > 1 ). merge_subblock_idx[ x0 ][ y0 ]; ae(v). } else {. if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) {. ciip_flag[ x0 ][ y0 ]; ae(v). if( ciip_flag[ x0 ][ y0 ] ) {. if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ). ciip_luma_mpm_flag[ x0 ][ y0 ]; ae(v). if( ciip_luma_mpm_flag[ x0 ][ y0 ] ). ciip_luma_mpm_idx[ x0 ][ y0 ]; ae(v). }. }. if( sps_triangle_enabled_flag && tile_group_type = = B && ciip_flag[ x0 ][ y0 ] = = 0 && cbWidth * cbHeight >= 64 ). merge_triangle_flag[ x0 ][ y0 ]; ae(v). if( merge_triangle_flag[ x0 ][ y0 ] ) {. merge_triangle_split_dir[ x0 ][ y0 ]; ae(v). merge_triangle_idx0[ x0 ][ y0 ]; ae(v). merge_triangle_idx1[ x0 ][ y0 ]; ae(v). } else if( MaxNumMergeCand > 1 ). merge_idx[ x0 ][ y0 ]; ae(v). }. }. }. }.

# Simulation results

The following experiments were conducted using the VTM-4.0.1 under the common test conditions [1]. The anchor is VTM-4.0.1 with MaxNumMergeCand = 1. The proposed CU-level solutions, including Solution 1 and Solution 3.2, were experimented on top of VTM-4.0.1 with MaxNumMergeCand = 1. The simulation results are presented in Table 1 and Table 2.

Table 1: Summary results of Solution 1 over VTM-4.0.1 with MaxNumMergeCand=1.

Table 1: Summary results of Solution 1 over VTM-4.0.1 with MaxNumMergeCand=1.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.95% | -0.77% | -0.93% | 96% | 100% |
| Class C | -0.82% | -0.77% | -0.72% | 96% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.98% | -0.86% | -0.94% | 98% | 101% |
| Class F | -0.36% | -0.32% | -0.32% | 97% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.81% | -0.73% | -0.92% | 103% | 102% |
| Class E | -0.65% | -0.76% | -0.49% | 102% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.86% | -1.63% | -1.56% | 101% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2: Summary results of Solution 3.2 over VTM-4.0.1 with MaxNumMergeCand=1.

Table 2: Summary results of Solution 3.2 over VTM-4.0.1 with MaxNumMergeCand=1.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.01% | 0.00% | -0.05% | 93% | 100% |
| Class C | 0.03% | 0.06% | 0.10% | 93% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.08% | -0.01% | 0.12% | 95% | 101% |
| Class F | -0.03% | -0.05% | 0.05% | 93% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.21% | -0.06% | -0.16% | 100% | 102% |
| Class E | -0.27% | -0.51% | 0.19% | 98% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.14% | -0.40% | 0.79% | 97% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusion

Several solutions are provided to address the scenario of MaxNumMergeCand specified as 1. Basically, the solutions can be divided into three categories. First category is to set the number of merging candidates as 2 if MMVD_Flag is enabled. Second category is to set MaxNumMergeCand as 2 if the maximum number of MMVD merge candidates specified as 2. Third category is to set the maximum number of MMVD merge candidates as 1 if MaxNumMergeCand = 1. It is recommended to adopt one of solutions into VVC standard.

# Reference

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakech, MA.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).