| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0449 |
| --- | --- |

| Title: | CE4-related: Simplification on MMVD distance table |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang, Yao-Jen Chang, Chun-Chi Chen, Wei-Jung Chien and Marta Karczewicz | Email: | {yzh, yjchang chunchic, wchien, martak} @qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to simplify MMVD by removing MMVD candidates with the largest two distance offsets in the distance table. Both encoder-only change method and normative change method are tested. With the above mentioned changes on top of VTM-4.0, the reported average Y BD-rate numbers are 0.00% in RA and -0.02% in LDB for encoder-only change method and 0.00% in RA and -0.03% for normative change method. The encoder runtime is 97% in RA and 97% in LDB for encoder-only change method and 97% in RA and 98% in LDB for normative change method.

# Introduction

Currently in VTM4.0, each MMVD candidates are generated by adding a distance offset in a given direction to a base position. The MMVD base positions are derived by taking the first two available merge candidates from the merge list. The distance offset table is defined as [1/4, 1/2, 1, 2, 4, 8, 16, 32] with each of the entry representing the how far the mmvd candidate deviate from the base position in number of luma samples. Since the merge candidate is already a good estimation of the best motion vector, it is unlikely that the final best motion vector is located far from the base position. Hence, the possibility for those large distance offset to be selected should be quite low. For this reason, it is proposed to reduce the number of mmvd candidates from 64 to 48 by removing the largest two distance offset entries in the distance offset table. Encoder only change and normative change are investigated together in this contribution.

# Proposed Method

## Method 1: encoder only change method

In this method, MMVD candidates with distance offsets of 16 and 32 will be skipped for testing in the encoder side to avoid it being selected. The total number of MMVD candidates tested in the encoder side is reduced from 64 to 48.

## Method 2: normative change method

## On top of method 1, the distance offset table is modified from [1/4, 1/2, 1, 2, 4, 8, 16, 32] to [1/4, 1/2, 1, 2, 4, 8]. The binarization of the distance offset index is also modified accordingly. Table 1 shows the binarization of each of the distance offset index. Compare to the original binarization design, distance offset indices 6 and 7 are removed. Codeword for distance offset index 5 is also changed accordingly based on truncated unary binarization scheme from 111110 to 11111.

Table 1. Distance offset index binarization

| Distance list index | binarization |
| --- | --- |
| 0 | 0 |
| 1 | 10 |
| 2 | 110 |
| 3 | 1110 |
| 4 | 11110 |
| 5 | 11111 |

# Test results

The Random Access (RA) and Low Delay B (LDB) conditions as pecified in the common test conditions [1] are used in this contribution. The simulation results are reported in Table 2 and Table 3:

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.07% | 0.19% | 96% | 102% |
| Class A2 | -0.02% | -0.04% | 0.07% | 96% | 102% |
| Class B | 0.02% | 0.08% | 0.07% | 97% | 104% |
| Class C | -0.01% | 0.02% | -0.10% | 98% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.05% | 97% | 102% |
| Class D | -0.02% | -0.11% | -0.10% | 98% | 100% |
| Class F | 0.01% | 0.06% | 0.05% | 100% | 104% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.03% | 0.19% | 97% | 105% |
| Class C | -0.03% | 0.00% | 0.17% | 96% | 106% |
| Class E | -0.07% | -0.01% | -0.35% | 97% | 114% |
| Overall | -0.02% | -0.02% | 0.05% | 97% | 107% |
| Class D | 0.00% | 0.54% | 0.13% | 97% | 103% |
| Class F | -0.04% | 0.24% | 1.08% | 98% | 113% |

Table 2. Simulation result for proposed encoder only change method

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.14% | 96% | 102% |
| Class A2 | -0.01% | -0.01% | 0.02% | 96% | 103% |
| Class B | -0.01% | 0.01% | 0.05% | 97% | 103% |
| Class C | 0.01% | 0.04% | -0.08% | 97% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.03% | 97% | 102% |
| Class D | -0.04% | 0.03% | -0.01% | 97% | 99% |
| Class F | -0.02% | 0.02% | -0.02% | 100% | 104% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.31% | 0.08% | 98% | 106% |
| Class C | -0.03% | 0.20% | 0.10% | 99% | 107% |
| Class E | -0.05% | -0.01% | -0.06% | 97% | 115% |
| Overall | -0.03% | -0.06% | 0.05% | 98% | 108% |
| Class D | -0.06% | 0.03% | 0.06% | 100% | 104% |
| Class F | -0.05% | 0.22% | 0.01% | 99% | 115% |

Table 3. Simulation result for proposed normative change method

# Conclusion

In this contribution, it is proposed to simplify MMVD by removing MMVD candidates with the largest two distance offsets using both encoder only change method and normative change method. The resulting BD-rate numbers for encoder only change method are 0.00% and -0.02% on average for RA and LDB with 3% encoding time reduction. The resulting BD-rate numbers for normative change method are 0.00% and -0.03 on average for RA and LDB with 3% and 2% encoding time reduction respectively. We would like to recommend the committees to consider adopting the proposed techniques in the corresponding VTM reference software.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li, Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video”, document JVET-M1010, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.