| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0450 |
| --- | --- |

| Title: | CE3-3.4-related: unified MPM list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Bossen1, K. Misra2 | Email: | fbossen@sharplabs.com misrak@sharplabs.com |
| Source: | 1Sharp Electronics of Canada, 2Sharp Labs of America |  |  |

_____________________________

# Abstract

This contribution considers a variation of experiment CE3-3.4. CE3-3.4 proposes an approach to coding intra modes by signaling DC and planar modes separate from the angular modes. The angular modes are signaled using a MPM list. Here, normative constants are modified in that MPM list construction process and are asserted to have the benefit of: (i) enabling an implementation that is even more compact than the one described in JVET-N0436, (ii) reducing the number of pages required to express the list construction process to approximately 1 page of specification text, (iii) provide a coding efficiency benefit, and (iv) reduce encoder run time. Specifically, the coding efficiency relative to VTM 4 for the proposed method, including some encoder adjustments, is -0.27% (AI) and -0.09% (RA) with a slight reduction in encoder run time of about 3% (AI).

# Introduction

JVET-N0436 describes optimized software implementations for experiments CE3-3.4. After considering such implementations, modifications are made to further simplify the MPM list construction. For example, when constructing the list where neighbours are two different angular modes A and B, several list variants are provided based on the difference D between A and B. In these variants, the Nth entry in the list may be equal to an offset plus either A or B, depending on the value of D. In the proposed modified method, the dependency on D is eliminated.

# Proposal

The proposal is based on experiment CE3-3.4.1, and has the following features:

- Same as CE3-3.4.1

  - DC and planar modes are coded using separate flags (angular_flag and planar_flag)

  - The MPM and remainder lists contain exclusively angular modes

  - There are three cases for constructing the list, based on top-right and bottom-left neighbours:

    - Both neighbours use different angular modes

    - One neighbour uses an angular or both neighbours use the same angular mode

    - Neither neighbour uses an angular mode

  - For combined intra/inter prediction (CIIP), the angular modes are coded using a single flag as only two angular modes are supported in VVC draft 4: horizontal and vertical

- Additional features

  - Normative constants for the list construction process are modified, enabling a more compact implementation and easier to understand description.

  - Only the first entry in the MPM list may be equal to VDIA_IDX. Therefore all other entries may be represented using 6 bits (there are 64 angular modes other than VDIA_IDX)

  - The encoder search is modified to more closely match other proposals in CE3-3. For instance, if both neighbours are different and angular, then one neighbour plus the planar mode are added to the RD search list instead of both neighbours. More details are provided in section 2.1.

  - The lambda value for the initial mode search is doubled

A detailed description of the approach is provided in Section 8 (Appendix).

## Search modification

The search is modified such that the procedure for determining which candidates to add to the full RD search is as:

mpm[0] = leftIntraDir;

mpm[1] = aboveIntraDir;

numCand = leftIntraDir == aboveIntraDir ? 1 : 2;

if (ispType != NOT_INTRA_SUBPARTITIONS)

{

if (mpm[0] == DC_IDX) mpm[0] = PLANAR_IDX;

if (mpm[1] == DC_IDX) mpm[1] = PLANAR_IDX;

}

if (numCand == 2 && mpm[0] > DC_IDX && mpm[1] > DC_IDX)

{

mpm[0] = std::max(mpm[0], mpm[1]);

mpm[1] = PLANAR_IDX;

}

Note that the number of candidates numCand added to the full RD search list is unchanged. It is asserted that this search method is more similar to what is done in other CE3-3 experiments.

# Results

The approach was integrated into VTM4.0, and results are shown in the Tables below.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.49% | -0.39% | -0.32% | -0.49% | 94% | 100% |
| Class A2 | -0.21% | -0.08% | -0.20% | -0.20% | 97% | 100% |
| Class B | -0.26% | -0.20% | -0.17% | -0.25% | 98% | 102% |
| Class C | -0.16% | -0.15% | -0.09% | -0.16% | 99% | 101% |
| Class E | -0.27% | -0.11% | -0.21% | -0.25% | 93% | 101% |
| Overall | -0.27% | -0.18% | -0.19% | -0.26% | 97% | 101% |
| Class D | -0.15% | -0.17% | -0.31% | -0.16% | 95% | 99% |
| Class F | 0.00% | -0.07% | -0.17% | -0.03% | 92% | 99% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.17% | -0.14% | -0.13% | -0.19% | 100% | 99% |
| Class A2 | -0.08% | 0.17% | 0.08% | -0.06% | 97% | 99% |
| Class B | -0.09% | -0.03% | -0.16% | -0.09% | 98% | 101% |
| Class C | -0.03% | -0.09% | -0.24% | -0.04% | 98% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.09% | -0.03% | -0.12% | -0.09% | 98% | 100% |
| Class D | -0.05% | -0.14% | -0.08% | -0.05% | 97% | 100% |
| Class F | -0.07% | -0.19% | -0.12% | -0.08% | 99% | 101% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.44% | 0.07% | 0.01% | 100% | 99% |
| Class E | -0.04% | 0.77% | 0.95% | 0.03% | 98% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.02% | 0.37% | -0.25% | 0.02% | 99% | 102% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-4.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.01% | -0.15% | -0.36% | -0.01% | 99% | 101% |
| Class E | 0.04% | 0.97% | 1.16% | 0.13% | 98% | 103% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.04% | -0.06% | 0.22% | 0.04% | 98% | 101% |
| Class F | 0.05% | -0.06% | -0.22% | 0.09% | 96% | 100% |

# Implementation aspects

## MPM list construction

One benefit of the modified MPM list construction process is that it can be implemented in ways that are efficient in software. For example, the following code implements the list contribution process for all cases:

const uint64_t offsetMpms[10] = {

0x3e023f010000ull, 0x023e013f0000ull, 0x023f01010000ull, 0x01013f3f0000ull,

0x3d023e013f00ull, 0x090310013f00ull, 0x373d30013f00ull,

0x183f20001030ull, 0x26303f081710ull, 0x1a1001382930ull

};

const bool sameDir = leftIntraDir == aboveIntraDir;

const int  maxDir  = std::max(leftIntraDir, aboveIntraDir) - 2;

const int  minDir  = std::min(leftIntraDir, aboveIntraDir) - 2;

int        idx     = ispType + 7;

uint64_t   mpms    = 0;

if (!sameDir && minDir >= 0) {

idx  = Clip3(0, 3, ((maxDir - minDir + 2) & 63) - 2);

mpms = minDir * 0x000100010100ull + maxDir * 0x010001000001ull;

} else if (maxDir >= 0) {

idx  = sameDir && ((maxDir >= 32) == (ispType == 1)) ? ispType + 4 : 4;

mpms = maxDir * (idx == 4 ? 0x010101010101ull : 0x000000010101ull);

}

mpms = (mpms + offsetMpms[idx]) & 0x3f3f3f3f3fffull;

It should be noted that in an implementation such as the above, where a single register/variable is used to represent all elements in the list, the list size doesn’t impact the operation count (i.e., the operation count is the same regardless of whether 4 or 6 MPMs are in the list).

We note that the equivalent code spans over 200 lines in VTM 4.0.  (And, thus, we achieve a 90+% reduction in code size!)

At a high level, the code distinguishes between 3 cases:

- Both neighbours use different angular modes

- One neighbour uses an angular or both neighbours use the same angular mode

- Neither neighbour uses an angular mode

For each case an index is computed. This index is then used to address a lookup table to determine the MPM list using simple arithmetic.

## MPM mode reconstruction

Reconstructing the mode from the MPM index is still straightforward using the single register/variable representation:

mode = (uint8_t)(mpms >> (8 * mpmIdx)) + 2

## Non-MPM mode reconstruction

Given that in this proposal DC and planar modes are coded separately in this proposal, there are only 65 modes that need consideration when reconstructing a non-MPM mode. This reduced number of modes can be beneficial when using bit arithmetic and 64-bit registers.

While the reference software and the text describe the reconstruction process as using a sort on a vector of modes, no such sorting is actually needed. Below is a software implementation that uses the BMI2 instruction set to efficiently reconstruct the mode from a remainder index:

uint64_t mask = mpms & 64 ? 0 : 1ull << (mpms & 63);

for (int i = 1; i < 6; i++)

mask |= 1ull << ((mpms >> (8 * i)) & 63);

ipred_mode = 2 + _tzcnt_u64(_pdep_u64(1ull << ipred_mode, ~mask));

# Recommendation

In the current VVC WD there exists four different luma intra MPM list construction process. This contribution proposes the use of a unified list construction process. It is recommended to adopt the proposed luma intra MPM list construction process in the next version of the VVC WD and VTM.

# References

[1] F. Bossen, K. Misra, “Non-CE3: A unified luma intra mode list construction process,” JVET-M0528, 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0 VTM-4.0

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Modifications to VVC WD are highlighted in yellow:

## SYNTAX CHANGES:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 ) |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| … |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY ¦ ¦ cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_angular_mode_flag[ x0 ][ y0 ] != 0) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] == 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] == 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } else if( intra_subpartitions_mode_flag[ x0 ][ y0 ] == 0 ) |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER */ |  |
| … |  |

Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| … |  |
| } else { |  |
| if( sps_ciip_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 ) { |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] ) { |  |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_angular_mode_flag[ x0 ][ y0 ] ) |  |
| ciip_intra_luma_vert_flag[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_planar_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| … |  |
| } |  |

## SEMANTIC CHANGES:

intra_luma_angular_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction mode for luma samples is angular. intra_luma_angular_mode_flag[ x0 ][ y0 ] equal to  0 specifies that the intra prediction mode for luma samples is not angular. When intra_luma_angular_mode_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 1.

intra_luma_planar_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction mode for luma samples is PLANAR. intra_luma_planar_flag[ x0 ][ y0 ] equal to  0 specifies that the intra prediction mode for luma samples is DC. When intra_luma_planar_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 1.

When ciip_flag[ x0 ][ y0 ] is equal to 0, The syntax elements intra_luma_mpm_flag[ x0 ][ y0 ], intra_luma_mpm_idx[ x0 ][ y0 ] and intra_luma_mpm_remainder[ x0 ][ y0 ] specify the angular intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_luma_angular_mode_flag[ x0 ][ y0 ] is equal to 1 and intra_luma_mpm_flag[ x0 ][ y0 ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.4.2

When intra_luma_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

ciip_intra_luma_vert_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction mode for luma samples is INTRA_ANGULAR50. ciip_intra_luma_vert_flag[ x0 ][ y0 ] equal to  0 specifies that the intra prediction mode for luma samples is INTRA_ANGULAR18. When intra_luma_vert_flag[ x0 ][ y0 ] is not present it is inferred to be equal to 0.

When ciip_flag[ x0 ][ y0 ] is equal to 1, the syntax elements intra_luma_angular_mode_flag[ x0 ][ y0 ], ciip_intra_luma_vert_flag[ x0 ][ y0 ], and intra_luma_planar_flag[ x0 ][ y0 ] specify the intra prediction mode for luma samples used in combined inter-picture merge and intra-picture prediction. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. The intra prediction mode IntraPredModeY is derived as follows:

if ( intra_angular_mode_flag[ x0 ][ y0 ] != 0 ) {
	IntraPredModeY = ( ciip_intra_luma_vert_flag[ x0 ][ y0 ] )? INTRA_ANGULAR50 : INTRA_ANGULAR18
} else {
	IntraPredModeY = ( intra_luma_planar_flag[ x0 ][ y0 ] )? INTRA_PLANAR : INTRA_DC
}

## DECODING PROCESS CHANGES:

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1, provided above, specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )

maxAB = Max( candIntraPredModeA, candIntraPredModeB )

- If maxAB is greater than INTRA_DC, and minAB is equal to maxAB or less than or equal to INTRA_DC candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ x ] with x = 0..2 is set equal to maxAB - 2

  - If one of the following conditions is true,

  - IntraSubPartitionsSplitType is equal to ISP_HOR_SPLIT and candIntraPredModeA  is less than INTRA_ANGULAR34 and minAB is not equal to maxAB,

  - IntraSubPartitionsSplitType is equal to ISP_VER_SPLIT and candIntraPredModeA  is greater than or equal to INTRA_ANGULAR34 and minAB is not equal to maxAB,

  - IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT,

the following applies:

candModeList[ x ] with x = 3..5 is set equal to maxAB - 2

The variable mpmTblIdx is set to 4

  - Otherwise, the following applies:

candModeList[ x ] with x = 3..5 is set equal to 0

The variable mpmTblIdx is set to 4 + IntraSubPartitionsSplitType

- Otherwise if minAB is greater than INTRA_DC, the following applies:

  - candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ x ] with x = 0, 3, 5 is set equal to maxAB - 2

candModeList[ x ] with x = 1, 2, 4 is set equal to minAB - 2

  - The variable mpmTblIdx is set to Clip3(0, 3, ((maxAB - minAB + 2) & 63) – 2)

- Otherwise, the following applies:

  - candModeList[ x ] with x = 0..5 is set equal to 0

  - The variable mpmTblIdx is set to 7 + IntraSubPartitionsSplitType

- candModeList[ x ] with x = 0..5 is updated according to:

  - candModeList[ x ] = ((candModeList[ x ] + mpmTbl[ mpmTblIdx ][ x ]) & (x == 0 ? 127 : 63)) + 2, where mpmTbl is an array defined as:

| mpmTblIdx | x |  |  |  |  |  | mpmTblIdx | x |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |  | 0 | 1 | 2 | 3 | 4 | 5 |
| 0 | 0 | 0 | 1 | 63 | 2 | 62 | 5 | 0 | 63 | 1 | 16 | 3 | 9 |
| 1 | 0 | 0 | 63 | 1 | 62 | 2 | 6 | 0 | 63 | 1 | 48 | 61 | 55 |
| 2 | 0 | 0 | 1 | 1 | 63 | 2 | 7 | 48 | 16 | 0 | 32 | 63 | 24 |
| 3 | 0 | 0 | 63 | 63 | 1 | 1 | 8 | 16 | 23 | 8 | 63 | 48 | 38 |
| 4 | 0 | 63 | 1 | 62 | 2 | 61 | 9 | 48 | 41 | 56 | 1 | 16 | 26 |

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_angular_mode_flag[ xCb ][ yCb ] is equal to 0 and intra_luma_planar[ xCb ][ yCb ] is equal to 1, then IntraPredModeY[ xCb ][ yCb ] is the set equal to INTRA_PLANAR

- Otherwise if intra_angular_mode_flag[ xCb ][ yCb ] is equal to 0 and intra_luma_planar[ xCb ][ yCb ] is equal to 0, then IntraPredModeY[ xCb ][ yCb ] is the set equal to INTRA_DC

- Otherwise, if intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ] + 2.

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

Table 9-4 – Syntax elements and associated binarizations

| coding_unit( ) | intra_luma_ref_idx[ ][ ] |  | TR |  | cMax = 2, cRiceParam = 0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | intra_subpartitions_mode_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_subpartition_split_flag |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_luma_angular_mode_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_luma_planar_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_luma_mpm_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | intra_luma_mpm_idx[ ][ ] |  | TR |  | cMax = 5, cRiceParam = 0 |  |  |  |  |  |
|  | intra_luma_mpm_remainder[ ][ ] |  | TB |  | cMax = 58 |  |  |  |  |  |
|  | intra_chroma_pred_mode[ ][ ] |  | 9.5.3.7 |  | - |  |  |  |  |  |
|  | … |  | … |  | … |  |  |  |  |  |
| merge_data( ) | … |  | … |  | … |  |  |  |  |  |
|  | ciip_intra_luma_vert_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | ciip_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | ciip_luma_mpm_flag[ ][ ] |  | FL |  | cMax = 1 |  |  |  |  |  |
|  | ciip_luma_mpm_idx[ ][ ] |  | TR |  | cMax = 2, cRiceParam = 0 |  |  |  |  |  |
|  | … |  | … |  | … |  |  |  |  |  |
| Table 9-10 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |  |  |  |
| Syntax element |  | binIdx |  |  |  |  |  |  |  |
|  |  | 0 |  | 1 |  | 2 | 3 | 4 | >= 5 |
| .. |  | .. |  | .. |  | .. | .. | .. | .. |
| intra_luma_ref_idx[ ][ ] |  | 0 |  | 1 |  | na | na | na | na |
| intra_subpartitions_mode_flag |  | 0 |  | na |  | na | na | na | na |
| intra_subpartition_split_flag |  | 0 |  | na |  | na | na | na | na |
| intra_luma_ref_idx[ ][ ] |  | 0 |  | 1 |  | 2 | na | na | na |
| intra_luma_angular_mode_flag[ ][ ] |  | When ciip_flag[ ][ ] is equal to 0, ctxInc is between 0..4 and derived per clause a.b.c.d.e. When ciip_flag[ ][ ] is equal to 1, ctxInc is ( cbWidth > 2*cbHeight ¦¦ cbHeight > 2*cbWidth) ? 6 : 7 |  | na |  | na | na | na | na |
| intra_luma_planar_flag[ ][ ] |  | ciip_intra_flag[ ][ ] ? 0 : 1 |  | na |  | na | na | na | na |
| intra_luma_mpm_flag[ ][ ] |  | 0 |  | na |  | na | na | na | na |
| intra_luma_mpm_idx[ ][ ] |  | Bypass |  | bypass |  | bypass | bypass | bypass | na |
| intra_luma_mpm_remainder[ ][ ] |  | Bypass |  | bypass |  | bypass | bypass | bypass | bypass |
| .. |  | .. |  | .. |  | .. | .. | .. | .. |
| ciip_flag[ ][ ] |  | 0 |  | na |  | na | na | na | na |
| ciip_intra_luma_vert_flag[ ][ ] |  | bypass |  | na |  | na | na | na | na |
| ciip_luma_mpm_flag[ ][ ] |  | 0 |  | na |  | na | na | na | na |
| ciip_luma_mpm_idx[ ][ ] |  | bypass |  | bypass |  | na | na | na | na |
| … |  | … |  | … |  | … | … | … | … |

a.b.c.d.e Derivation process of ctxInc using left-bottom and above-right syntax elements

Input to this process is the luma location ( x0, y0 ) specifying the top-left luma sample of the current luma block relative to the top-left sample of the current picture, the colour component cIdx, the current coding quadtree depth cqDepth, and the width and the height of the current coding block in luma samples cbWidth and cbHeight.

Output of this process is ctxInc.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 − 1, y0 + cbHeight − 1 ) as inputs, and the output is assigned to availableLB.

The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to availableAR.

The same CTU checking process for a block as specified in clause 6.Y.X [Same CTU checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( x0, y0 ) and the neighbouring location ( xNbY, yNbY ) set equal to ( x0 + cbWidth − 1, y0 − 1 ) as inputs, and the output is assigned to sameCtuAR.

The assignment of ctxInc is specified as follows with condL, condLB, condA and condAR for the syntax elements intra_luma_angular_mode_flag[ x0 ][ y0 ] specified in Table 9-xx:

ctxInc = ( condLB  &&  availableLB ) + ( condAR  &&  availableAR && sameCtuAR ) + (intra_subpartitions_mode_flag[ x0 ][ y0 ] != 0 ? 3 : 0)	(9-yy)

Table 9-xx – Specification of ctxInc using left-bottom and above-right syntax elements

| Syntax element | condLB | condAR |
| --- | --- | --- |
| intra_luma_angular_mode_flag[ x0 ][ y0 ] | CuPredMode[ xNbLB ][ yNbLB ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbLB ][ yNbLB ] is equal to 1 | CuPredMode[ xNbAR ][ yNbAR ] is equal to MODE_INTRA && intra_luma_angular_mode_flag[ xNbAR ][ yNbAR ] is equal to 1 |