Decoding process for coding units coded in intra prediction mode

General decoding process for coding units coded in intra prediction mode

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components.

Output of this process is a modified reconstructed picture before in-loop filtering.

The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, the decoding process for luma samples is specified as follows:

- If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SL[ xCb + i ][ yCb + j ] = 
		pcm_sample_luma[ ( cbHeight * j ) + i ]  <<  ( BitDepthY − PcmBitDepthY ),	(8-6)
		with i = 0..cbWidth − 1, j = 0..cbHeight − 1

- Otherwise, the following applies:

- The derivation process for the luma intra prediction mode as specified in clause 8.4.2 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The general decoding process for intra blocks as specified in clause 8.4.4.1 is invoked with the luma location ( xCb, yCb ), the tree type treeType, the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the variable predModeIntra set equal to IntraPredModeY[ xCb ][ yCb ], and the variable cIdx set equal to 0 as inputs, and the output is a modified reconstructed picture before in-loop filtering.

When treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_CHROMA, the decoding process for chroma samples is specified as follows:

- If pcm_flag[ xCb ][ yCb ] is equal to 1, the reconstructed picture is modified as follows:

SCb[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] =
		pcm_sample_chroma[ ( cbHeight / SubWidthC * j ) + i ]  <<  ( BitDepthC − PcmBitDepthC ),
		with i = 0.. cbWidth / SubWidthC − 1 and j = 0.. cbHeight / SubHeightC − 1	(8-7)

SCr[ xCb / SubWidthC + i ][ yCb / SubHeightC + j ] =
		pcm_sample_chroma[ ( cbHeight / SubWidthC * ( j + cbHeight / SubHeightC ) ) + i ]  <<
		( BitDepthC − PcmBitDepthC ),
		with i = 0..cbWidth / SubWidthC − 1 and j = 0..cbHeight / SubHeightC − 1	(8-8)

- Otherwise, the following applies:

- The derivation process for the chroma intra prediction mode as specified in clause 8.4.3 is invoked with the luma location ( xCb, yCb ) , the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight as input.

- The general decoding process for intra blocks as specified in clause 8.4.4.1 is invoked with the chroma location ( xCb / 2, yCb / 2 ), the tree type treeType, the variable nTbW set equal to ( cbWidth / 2 ), the variable nTbH set equal to ( cbHeight / 2 ), the variable predModeIntra set equal to IntraPredModeC[ xCb ][ yCb ], and the variable cIdx set equal to 1, and the output is a modified reconstructed picture before in-loop filtering.

- The general decoding process for intra blocks as specified in clause 8.4.4.1 is invoked with the chroma location ( xCb / 2 , yCb / 2 ), the tree type treeType, the variable nTbW set equal to ( cbWidth / 2 ), the variable nTbH set equal to ( cbHeight / 2 ), the variable predModeIntra set equal to IntraPredModeC[ xCb ][ yCb ], and the variable cIdx set equal to 2, and the output is a modified reconstructed picture before in-loop filtering.

Derivation process for luma intra prediction mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

Table 8-1 specifies the value for the intra prediction mode IntraPredModeY[ xCb ][ yCb ] and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |
| 81..83 | INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM |

NOTE – : The intra prediction modes INTRA_LT_CCLM, INTRA_L_CCLM and INTRA_T_CCLM are only applicable to chroma components.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb + cbHeight − 1 ) and ( xCb + cbWidth − 1, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If one or more of the following conditions are true, candIntraPredModeX is set equal to INTRA_PLANAR.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-9)

candModeList[ 1 ] = INTRA_PLANAR	(8-10)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )		(8-11)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 4 ] = DC_IDX	(8-13)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-14)

-

-

-

-

-

-

-

-

-

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-24)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-25)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-26)

candModeList[ 1 ] = INTRA_PLANAR	(8-27)

candModeList[ 2 ] = candIntraPredModeB	(8-28)

candModeList[ 3 ] = INTRA_DC		(8-29)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-30)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-31)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-33)

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If minAB is equal to INTRA_PLANAR

candModeList[ 0 ] = INTRA_PLANAR	(8-52)

candModeList[ 1 ] = maxAB	(8-53)

  - Otherwise

candModeList[ 0 ] = candIntraPredModeA	(8-54)

candModeList[ 1 ] = candIntraPredModeB	(8-54)

candModeList[ 2 ] = 1 − minAB		(8-54)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-55)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-56)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-57)

-

-

-

-

-

-

-

-

-

-

-

-

- Otherwise, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-70)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-71) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-72)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-73)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-74)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-75)

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

-

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_luma_mpm_flag[ xCb ][ yCb ] is equal to 1, the IntraPredModeY[ xCb ][ yCb ] is set equal to  candModeList[ intra_luma_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candModeList[ i ] is greater than candModeList[ j ] for i = 0..4 and for each i, j = ( i + 1 )..5, both values are swapped as follows:

( candModeList[ i ], candModeList[ j ] ) = Swap( candModeList[ i ], candModeList[ j ] )	(8-94)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_luma_mpm_remainder[ xCb ][ yCb ].

  - For i equal to 0 to 5, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

Binarization process

General

Input to this process is a request for a syntax element.

Output of this process is the binarization of the syntax element.

Table 9-9 specifies the type of binarization process associated with each syntax element and corresponding inputs.

The specification of the truncated Rice (TR) binarization process, the truncated binary (TB) binarization process, the k-th order Exp-Golomb (EGk) binarization process and the fixed-length (FL) binarization process are given in clauses 9.5.3.3 through 9.5.3.7, respectively.

| Table 9-9 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| tile_group_data( ) | end_of_tile_one_bit | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_tree( ) | split_cu_flag | FL | cMax = 1 |
|  | split_qt_flag | FL | cMax = 1 |
|  | mtt_split_cu_vertical_flag | FL | cMax = 1 |
|  | mtt_split_cu_binary_flag | FL | cMax = 1 |
| coding_unit( ) | cu_skip_flag[ ][ ] | FL | cMax = 1 |
|  | pred_mode_ibc_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_ref_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | intra_subpartitions_mode_flag | FL | cMax = 1 |
|  | intra_subpartition_split_flag | FL | cMax = 1 |
|  | intra_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | intra_luma_mpm_idx[ ][ ] | TR | cMax = 5, cRiceParam = 0 |
|  | intra_luma_mpm_remainder[ ][ ] | TB | cMax = 60 |
|  | intra_chroma_pred_mode[ ][ ] | 9.5.3.8 | - |
|  | merge_flag[ ][ ] | FL | cMax = 1 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.5.3.9 | cbWidth, cbHeight |
|  | inter_affine_flag[ ][ ] | FL | cMax = 1 |
|  | cu_affine_type_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l0[ ][ ] | TR | cMax = NumRefIdxActive[ 0 ] − 1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = NumRefIdxActive[ 1 ] − 1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
|  | avmr_flag[ ][ ] | FL | cMax = 1 |
|  | amvr_precision_flag[ ][ ] | FL | cMax = 1 |
|  | gbi_idx[ ][ ] | TR | cMax = NoBackwardPredFlag ? 4: 2 |
|  | cu_cbf | FL | cMax = 1 |
|  | cu_sbt_flag | FL | cMax = 1 |
|  | cu_sbt_quad_flag | FL | cMax = 1 |
|  | cu_sbt_horizontal_flag | FL | cMax = 1 |
|  | cu_sbt_pos_flag | FL | cMax = 1 |
| merge_data( ) | mmvd_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_merge_flag[ ][ ] | FL | cMax = 1 |
|  | mmvd_distance_idx[ ][ ] | TR | cMax = 7, cRiceParam = 0 |
|  | mmvd_direction_idx[ ][ ] | FL | cMax = 3 |
|  | ciip_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_flag[ ][ ] | FL | cMax = 1 |
|  | ciip_luma_mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | merge_subblock_flag[ ][ ] | FL | cMax = 1 |
|  | merge_subblock_idx[ ][ ] | TR | cMax = MaxNumSubblockMergeCand − 1, cRiceParam = 0 |
|  | merge_triangle_flag[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx[ ][ ] | FL | cMax = 1 |
|  | merge_triangle_idx0[ ][ ] | TR | cMax = 4, cRiceParam = 0 |
|  | merge_triangle_idx1[ ][ ] | TR | cMax = 3, cRiceParam = 0 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | tu_cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | tu_cbf_cr[ ][ ][ ] | FL | cMax = 1 |
|  | cu_qp_delta_abs | 9.5.3.10 | - |
|  | cu_qp_delta_sign_flag | FL | cMax = 1 |
|  | transform_skip_flag[ ][ ] | FL | cMax = 1 |
|  | tu_mts_idx[ ][ ] | TR | cMax = 4, cRiceParam = 0 |
| residual_coding( ) | last_sig_coeff_x_prefix | TR | cMax = ( log2TbWidth << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_y_prefix | TR | cMax = ( log2TbHeight << 1 ) − 1, cRiceParam = 0 |
|  | last_sig_coeff_x_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_x_prefix >> 1 ) − 1 ) − 1 ) |
|  | last_sig_coeff_y_suffix | FL | cMax = ( 1 << ( ( last_sig_coeff_y_prefix >> 1 ) − 1 ) − 1 ) |
|  | coded_sub_block_flag[ ][ ] | FL | cMax = 1 |
|  | sig_coeff_flag[ ][ ] | FL | cMax = 1 |
|  | par_level_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt1_flag[ ] | FL | cMax = 1 |
|  | abs_level_gt3_flag[ ] | FL | cMax = 1 |
|  | abs_remainder[ ] | 9.5.3.11 | cIdx, current sub-block index i, x0, y0 |
|  | dec_abs_level[ ] | 9.5.3.12 | cIdx, x0, y0, xC, yC, log2TbWidth, log2TbHeight |
|  | coeff_sign_flag[ ] | FL | cMax = 1 |

