| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0451-v3 |
| --- | --- |

| Title: | CE3-3.1.2 : Harmonization on MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Ling Li Jangwon Choi Jungah Choi Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {jin78.heo, l.li, jangwon84.choi, jungah.choi, sunmi.yoo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of CE3-3.1.2, which is about the unification of MPM list generation described in JVET-M0815 [1]. All sub-tests use the same MPM list which is generated from the MPM list construction process for zero reference line. In CE3-3.1.2.a and CE3-3.1.2.b, one or two MPM candidates are removed for non-zero reference line and intra sub-partition (ISP) mode [2] as shown below Table 1. In CE3-3.1.2.c, a unified MPM generation process is used for all intra coding including zero and non-zero reference line and ISP mode.

Table 1. CE sub-tests

| CE sub-tests | Number of MPM candidates |  |  |
| --- | --- | --- | --- |
|  | Intra prediction | MRL | ISP |
| CE3-3.1.2.a | 6 | 5 | 4 |
| CE3-3.1.2.b | 6 | 5 | 5 |
| CE3-3.1.2.c | 6 | 6 | 6 |

The experimental results from three sub-tests from CE3-3.1.2 are reported as follows (BD-rate Y, enc. time, dec. time):

- CE3-3.1.2.a: -0.04%, 98%, 99% (AI), 0.01%, 100%, 99% (RA)

- CE3-3.1.2.b: -0.04%, 99%, 99% (AI), 0.01%, 101%, 99% (RA)

- CE3-3.1.2.c: 0.07%, 100%, 99% (AI), 0.07%, 101%, 99% (RA)

Proposed method

All sub-tests use the same MPM list which is generated from the MPM list construction process for zero reference line. In case of non-zero reference line and ISP mode, difference non-angular intra mode (i.e. PLANAR and DC) is excluded from the initial list. It is asserted that the proposed method simplified the MPM list generation process, especially, MPM list buffer for ISP mode by half from unifying the MPM list construction process in every intra coding cases.

This contribution changes MPM list order for the following three cases for zero reference line.

- If two neighboring modes are same and two neighboring modes are greater than DC.

mpm[0] = leftIntraDir

mpm[1] = PLANAR_IDX

mpm[2] = 2 + ( (leftIntraDir + 61) % 64)

mpm[3] = 2 + ( (leftIntraDir – 1 ) % 64)

mpm[4] = DC_IDX

mpm[5] = 2 + ( (leftIntraDir + 60) % 64)

- If two neighboring modes are different and two neighboring modes are greater than DC.

mpm[0] = leftIntraDir

mpm[1] = PLANAR_IDX

mpm[2] = aboveIntraDir

mpm[3] = DC_IDX

if maxAB – minAB is in the range of 2 to 62

mpm[4] = 2 + ( (maxAB + 61) % 64)

mpm[5] = 2 + ( (maxAB – 1 ) % 64)

otherwise

mpm[4] = 2 + ( (maxAB + 60) % 64)

mpm[5] = 2 + ( maxAB % 64)

- If one neighboring mode is PLANAR and the other neighboring mode is angular mode.

mpm[0] = PLANAR_IDX

mpm[1] = maxAB

mpm[2] = DC_IDX

mpm[3] = 2 + ( (maxAB + 61) % 64)

mpm[4] = 2 + ( (maxAB – 1 ) % 64)

mpm[5] = 2 + ( (maxAB + 60) % 64)

CE3-3.1.2.a

The size of MPM list for zero reference line is 6. Since the DC mode is excluded for MPM list of ISP mode, the size of MPM list for ISP mode is 5. The size of MPM list for non-zero reference line is 4, where Planar and DC modes are excluded. All other angular modes are still used for MPM list for both ISP mode and non-zero reference line with the same order.

CE3-3.1.2.b

The size of MPM list for zero reference line is 6. In order to generate MPM list for non-zero reference line, DC mode is maintained and only PLANAR mode is removed. DC mode is excluded for MPM list of ISP mode. As a result, the size of MPM list for both non-zero reference line and ISP mode is 5.

CE3-3.1.2.c

In this sub-test, there exists a single MPM list construction process for zero reference line. Both of MPM lists for non-zero reference line and ISP mode share the same MPM list which is generated for zero reference line.

Experimental results

The proposed method is implemented on top of VTM4.0 reference software and evaluated under common test conditions [4].

CE3-3.1.2.a

Table 2. Experimental results for CE3-3.1.2.a AI

Table 3. Experimental results for CE3-3.1.2.a RA

Table 4. Experimental results for CE3-3.1.2.a AI over codebase

Table 5. Experimental results for CE3-3.1.2.a RA over codebase

CE3-3.1.2.b

Table 6. Experimental results for CE3-3.1.2.b AI

Table 7. Experimental results for CE3-3.1.2.b RA

Table 8. Experimental results for CE3-3.1.2.b AI over codebase

Table 9. Experimental results for CE3-3.1.2.b RA over codebase

CE3-3.1.2.c

Table 10. Experimental results for CE3-3.1.2.c AI

Table 11. Experimental results for CE3-3.1.2.c RA

Table 12. Experimental results for CE3-3.1.2.c AI over codebase

Table 13. Experimental results for CE3-3.1.2.c RA over codebase

Conclusion

In this contribution, unification methods on MPM list construction are proposed. It is reportedly shows that, CE3-3.1.2.a, CE3-3.1.2.b, and CE3-3.1.2.c provide an average BD rate performance of -0.04% (AI) and 0.01% (RA), -0.04% (AI) and 0.01% (RA), and 0.07% (AI) and 0.07% (RA) without increasing the complexity of encoding and decoding. It is recommended to adopt the proposed method in the next VTM.

Reference

- Ling Li, et al., “CE3-related: Harmonization on MPM list”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0815, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- Santiago De-Luxán-Hernández, et al., “CE3: Intra Sub-Partitions Coding Modes”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0102, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).