| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0452-v2 |
| --- | --- |

| Title: | CE3-related : Harmonization between WAIP and ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Hyeongmun Jang Ling Li Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {jin78.heo, hm.jang, l.li, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a modification with which wide-angle intra prediction (WAIP) is appropriately applied to intra sub-partitions (ISP). In current VTM4.0, intra prediction mode for ISP is determined based on the aspect ratio of a coding block. However, actual intra prediction is applied to each sub-partition of which aspect ratio is different from that of the coding block. Therefore, proposed method determines the intra mode based on aspect ratio of sub-partition block. The experimental results reportedly show 0.01% (AI) and 0.00% (RA) average BD rate changes without increasing the complexity of encoding and decoding.

Introduction

WAIP [1] was proposed to allow extended intra prediction angles beyond the traditional 45 degree top-right and bottom-left directions to improve the accuracy of prediction blocks using closer reference samples for non-square blocks. WAIP replaces several conventional angular intra prediction modes with wide angular modes according to aspect ratio of non-square block.

ISP [2] divides an intra luma coding block vertically or horizontally into 2 or 4 sub-partitions which share the same intra mode. Each sub-partition is sequentially encoded/decoded to generate a reconstructed block and used as reference samples of the next sub-partition. All sub-partitions share the same intra mode which is determined based on a coding block.

In the VTM4.0, ISP determined the intra mode considering the aspect ratio of the current coding block. However, since the aspect ratio of the coding block and sub-partition are different, it is not appropriate to apply the intra mode determined based on the coding block to sub-partition. Therefore, this contribution proposed a method of determining the intra mode of sub-partition in ISP block.

Proposed method

For ISP blocks, it is proposed to determine intra mode considering the aspect ratio of sub-partitions. Since all sub-partitions have the same aspect ratio, all sub-partitions share the same intra mode. In addition, since the intra mode is determined in units of sub-partitions, reference samples can be efficiently managed.

Experimental results

The proposed method is implemented on top of VTM4.0 reference software and evaluated under common test conditions [3].

Table 1. Experimental results for AI

Table 2. Experimental results for RA

Conclusion

In this contribution, harmonization method between WAIP and ISP is proposed. It is reported that the proposed method provides an average BD rate performance of 0.01% (AI) and 0.00% (RA) without increasing the complexity of encoding and decoding. It is recommended to adopt the proposed method in the next VTM.

Reference

- Fabien Racapé, et al., “CE3-related: Wide-angle intra prediction for non-square blocks”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0500, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- Santiago De-Luxán-Hernández, et al., “CE3: Intra Sub-Partitions Coding Modes”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0102, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).