| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0454 |
| --- | --- |

| Title: | CE4-4.6: Simplification for merge list derivation in triangular prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Timofey Solovyev, Semih Esenlik, Sergey Ikonin, Jianle Chen | Tel: Email: | +7-925-008-9652 solovyev.timofey@huawei.com semih.esenlik@huawei.com sergey.ikonin@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of simplified merge list derivation methods for the triangular prediction mode. The additional uni-prediction merge candidate list dedicated to triangular prediction mode is removed. Instead, regular merge list is used by converting bi-prediction candidates to uni-prediction candidates.

Two methods of converting bi-prediction candidates to uni-prediction candidates are tested:

- Using L0 part of bi-prediction candidate (test 4.4.6.a)

- Using both L0 and L1 parts of bi-prediction candidate (test 4.4.6.b)

The first method shows on average 0.12%, 0.16% luma BD-rate loss for RA and LDB configurations respectively without significant change in encoding and decoding times.

The second option shows on average 0.00% BD-rate change for RA configuration and 0.11% luma BD-rate loss for LDB configuration without significant change in encoding and decoding times.

# Introduction

During the 12th JVET meeting in Macau, the triangular prediction mode has been adopted into the VVC specification draft [1]. If triangular mode is selected by a CU, it is split diagonally and each resulting triangular part is predicted using a different motion vector. The predictions generated for each triangular part is restricted to be uni-prediction.

The motion candidates used to predict the triangular parts are selected based on a uni-prediction candidate list. The uni-prediction candidate list consists of five candidates. It is derived from seven neighboring blocks including five spatial neighboring blocks (1 to 5) and two temporal co-located blocks (6 to 7). The motion vectors are collected and put into the uni-prediction candidate list according the following order:

- uni-prediction motion vectors

- L0 motion vector of bi-prediction motion vectors

- L1 motion vector of bi-prediction motion vectors

- averaged motion vectors of the L0 and L1 motion vectors of bi-prediction motion vectors.

If the number of candidates is less than five, zero motion vector(s) is added to the list.

# Proposed methods

The additional uni-prediction merge candidate list dedicated to triangular prediction mode is removed. Instead, regular merge list is used by converting bi-prediction candidates to uni-prediction candidates.

Two methods are tested in this core experiment.

### Method 1 (Test 4.4.6.A)

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list,

- Starting from the beginning of the regular merge candidate list the following is performed:

  - If the candidate is bi-predictive, the candidate is converted to uni-predictive by keeping the L0 motion information (motion information here means motion vector and reference index).

Described method of conversion bi-prediction merge candidate to the uni-prediction is same as used for 4x4 blocks in merge mode.

### Method 2 (Test 4.4.6.B)

The uni-prediction candidate list is constructed according to the steps of:

- Construct the regular merge candidate list,

- Starting from the beginning of the regular merge candidate list, the following is performed:

  - If a candidate in the regular merge list is uni-predictive, insert it into uni-prediction candidate list.

  - If a candidate in the regular merge candidate list is bi-prediction candidate:

    - insert L0 motion information into uni-prediction candidate list,

    - insert L1 motion information if it is not identical to the L0 motion information as additional candidate.

Motion information here means motion vector and reference index.

In this method L1 motion information of a bi-prediction candidate is compared with the L0 motion information before insertion into the uni-prediction candidate list. The L0-L1 motion information comparison can be performed in parallel to the other pruning checks of the merge list construction process.

The number of candidates in the uni-prediction candidate list in both methods is set as 5.

# Complexity analysis

Complexity analysis is performed according to the description of Core Experiment 4 [3]. Summary of complexity analysis for tests 4.4.6.a and 4.4.6.b is presented in Table 1 – Table 2. In this analysis it is assumed that regular merge list is already constructed, so complexity of deriving triangular merge candidates list based on the regular merge candidates list is analysed.

Table 1. Summary of complexity analysis for Test 4.4.6.a

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5 | 0 | 0 | 1 | 0 | 5 | – |

Table 2. Summary of complexity analysis for Test 4.4.6.b

| Merge list size | Max number of potential candidates | Max number of candidate comparison | Max number of MV scaling | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 5*2 | 4 | 0 | 1 | 0 | 5 | – |

# Simulation results

The proposed method is evaluated for random-access (RA) and low delay B (LDB) configurations using VTM-4.0 software, according to the common test conditions in JVET-M1010 [2] and the description of Core Experiment 4 [3]. The detailed experimental results are provided in Table 3 – Table 6.

Table 3. Coding performance of test 4.4.6.a over VTM-4.0 for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.13% | 0.18% | 102% | 101% |
| Class A2 | 0.06% | 0.00% | 0.18% | 102% | 101% |
| Class B | 0.09% | 0.10% | 0.05% | 101% | 103% |
| Class C | 0.23% | 0.15% | 0.27% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.10% | 0.16% | 101% | 102% |
| Class D | 0.18% | 0.12% | 0.25% | 100% | 101% |
| Class F | 0.29% | 0.17% | 0.12% | 100% | 101% |

Table 4. Coding performance of test 4.4.6.a over VTM-4.0 for LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11% | -0.14% | 0.10% | 101% | 101% |
| Class C | 0.19% | 0.41% | 0.28% | 101% | 105% |
| Class E | 0.19% | 0.45% | 0.16% | 101% | 105% |
| Overall | 0.16% | 0.19% | 0.18% | 101% | 103% |
| Class D | 0.12% | 0.68% | -0.02% | 100% | 102% |
| Class F | -0.04% | 0.19% | 0.52% | 101% | 102% |

Table 5. Coding performance of test 4.4.6.b over VTM-4.0 for RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.01% | 102% | 101% |
| Class A2 | -0.06% | -0.14% | -0.11% | 102% | 101% |
| Class B | 0.00% | -0.12% | -0.08% | 101% | 103% |
| Class C | 0.03% | -0.04% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.07% | -0.09% | 101% | 102% |
| Class D | 0.00% | -0.16% | -0.03% | 100% | 100% |
| Class F | 0.03% | 0.00% | -0.01% | 100% | 101% |

Table 6. Coding performance of test 4.4.6.b over VTM-4.0 for LDB configuration

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.15% | 0.02% | 101% | 101% |
| Class C | 0.16% | 0.45% | 0.14% | 101% | 105% |
| Class E | 0.09% | 0.12% | -0.25% | 101% | 105% |
| Overall | 0.11% | 0.12% | -0.01% | 101% | 103% |
| Class D | 0.16% | 0.06% | -0.24% | 100% | 104% |
| Class F | -0.10% | -0.37% | -0.31% | 101% | 103% |

# Conclusion

The proposed methods allow to use regular merge list construction procedure for deriving merge candidates list in triangular prediction mode. For bi-predictive candidates from the regular merge list conversion to uni-predictive is performed.

Two methods of converting bi-predictive candidates to uni-predictive ones are tested:

- Using L0 part of bi-prediction candidate (test 4.4.6.a)

- Using both L0 and L1 parts of bi-prediction candidate (test 4.4.6.b)

Simulation results for test 4.4.6.a reportedly show on average 0.12%, 0.16% luma BD-rate loss for RA and LDB configurations respectively.

Simulation results for test 4.4.6.b reportedly show on average no BD-rate difference for RA configuration and 0.11% luma BD-rate loss for LDB configuration.

# References

- R.-L. Liao, C. S. Lim, “CE10.3.1.b: Triangular prediction unit mode”, JVET-L0124, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019

- H. Yang, G. Li, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding,” JVET-M1024, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 13th Meeting: , Marrakech, MA, January, 2019.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).