| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0455 |
| --- | --- |

| Title: | CE8-related: Sign context modelling and level mapping for TS residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Muhammed Coban | Tel: Email: | +1 858 658 3973 martak@qti.qualcomm.com mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a local neighbor-based context derivation for sign coding and a level mapping scheme for TS residual coding proposed in JVET-M0464 input document. The simulations results show Class F: -3.27% AI, -2.37% RA, -1.94% LB BD-Rate, and Class SCC: -5.20% AI, -3.44% RA, -2.84% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions for sign context modelling case. For both proposed methods combined, the simulations results show Class F: -3.92% AI, -2.83% RA, -2.22% LB BD-Rate, and Class SCC: -5.83% AI, -4.06% RA, -3.15% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions.

# Proposal

## Sign context modelling

Sign coding context derivation in JVET-M0464 is replaced by the following method that utilize the sign information of above neighboring coefficient and left neighboring coefficient to derive the sign coding context offset.

Figure 1: Current coefficient value X, neighboring left coefficient value X0 and neighboring above coefficient value X1

X0 is the left neigboring coefficient value, X1 is above neighboring coefficient value as shown in Figure 1. If both neighboring coefficients are zero or both nonzero but with opposite signs, then context offset 0 is used, otherwise if both are non-negative and then context offset 1 is used, for all other cases context offset 2 is used. This can be summarized by the following table.

| (X0, X1) | Context offset |
| --- | --- |
| (0, 0), (+, -), (-, +) | 0 |
| (+, +), (+, 0), (0, +) | 1 |
| (-, -), (-, 0), (0, -) | 2 |

## Level mapping

The absolute coefficient level, absCoeffLevel, is mapped to a modified level to be coded by the following method that depends on the values of quantized residual samples to the left and above of the current residual sample.  Let X0 denote the absolute coefficient level to the left of the current coefficient and let X1 denote the absolute coefficient level of above coefficient. For representing a coefficient with absolute coefficient level absCoeff, a mapped absCoeffMod is coded which is derived as follows:

pred = max(X0, X1);

if (absCoeff == pred)

{

absCoeffMod = 1;

}

else

{

absCoeffMod = (absCoeff < pred) ? absCoeff + 1 : absCoeff;

}

# Results

The proposed scheme is implemented on top of CE8-3.1 software with BDPCM turned off and 3 regular bins per coefficient setting.  The simulations were carried out under the JVET common test conditions [4] with Class F coding settings applied to SCC class.

The test results under JVET common test conditions is tabulated in Table 1.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.21% | 0.41% | 105% | 119% |
| Class A2 | -0.01% | 0.07% | -0.03% | 99% | 120% |
| Class B | -0.05% | 0.03% | -0.01% | 85% | 99% |
| Class C | -0.30% | -0.32% | -0.21% | 90% | 97% |
| Class E | -0.09% | -0.10% | -0.17% | 82% | 96% |
| Overall | -0.15% | -0.03% | -0.01% | 91% | 104% |
| Class D | -0.24% | -0.18% | -0.26% | 88% | 99% |
| Class F | -3.27% | -1.91% | -2.08% | 85% | 97% |
| Class SCC | -5.20% | -2.75% | -2.83% | 108% | 109% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | 0.10% | 0.25% | 115% | 120% |
| Class A2 | -0.03% | -0.20% | -0.04% | 110% | 120% |
| Class B | 0.00% | -0.03% | -0.22% | 86% | 98% |
| Class C | -0.10% | -0.21% | -0.45% | 84% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.09% | -0.15% | 95% | 107% |
| Class D | 0.08% | -0.56% | -0.32% | 83% | 96% |
| Class F | -2.37% | -1.46% | -1.47% | 85% | 102% |
| Class SCC | -3.44% | -1.38% | -1.29% | 112% | 113% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.06% | 0.29% | 0.00% | 83% | 97% |
| Class E | -0.08% | 1.06% | 0.73% | 84% | 105% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.28% | 0.21% | -0.62% | 82% | 97% |
| Class F | -1.94% | -1.54% | -1.84% | 87% | 103% |
| Class SCC | -2.84% | -1.76% | -1.69% | 112% | 116% |

Table 1: Average Bjøntegaard delta rate savings of the proposed signed context modelling method relative to VTM-4.0.

For sign context modelling in combination with level mapping, the test results under JVET common test conditions is tabulated in Table 2.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | 0.25% | 0.40% | 104% | 118% |
| Class A2 | -0.01% | 0.06% | -0.06% | 99% | 122% |
| Class B | -0.05% | 0.00% | -0.02% | 87% | 98% |
| Class C | -0.29% | -0.36% | -0.34% | 84% | 97% |
| Class E | -0.07% | -0.14% | -0.22% | 85% | 99% |
| Overall | -0.14% | -0.05% | -0.06% | 91% | 105% |
| Class D | -0.23% | -0.16% | -0.40% | 89% | 99% |
| Class F | -3.92% | -2.49% | -2.65% | 84% | 94% |
| Class SCC | -5.82% | -3.35% | -3.37% | 113% | 110% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.06% | 0.26% | 113% | 119% |
| Class A2 | -0.02% | -0.13% | -0.07% | 110% | 119% |
| Class B | 0.00% | -0.04% | -0.03% | 85% | 99% |
| Class C | -0.06% | -0.30% | -0.42% | 84% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.10% | -0.08% | 94% | 106% |
| Class D | 0.10% | -0.27% | -0.77% | 83% | 97% |
| Class F | -2.83% | -1.98% | -1.83% | 85% | 99% |
| Class SCC | -4.06% | -1.83% | -1.85% | 112% | 115% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0-(CTC+TGM) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.20% | -0.29% | 82% | 99% |
| Class C | 0.09% | 0.14% | -0.14% | 82% | 103% |
| Class E | -0.02% | 0.40% | 0.14% | 84% | 98% |
| Overall | 0.04% | 0.06% | -0.13% | 82% | 100% |
| Class D | 0.27% | -0.05% | 0.33% | 82% | 94% |
| Class F | -2.22% | -1.54% | -1.60% | 86% | 100% |
| Class SCC | -3.15% | -2.14% | -1.99% | 114% | 118% |

Table 2: Average Bjøntegaard delta rate savings of the proposed signed context modelling in combination with level mapping method relative to VTM-4.0.

Measured run times are not accurate.

# References

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE 8): Screen content coding,” JVET-M1028, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4)”, JVET-M1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

- https://vcgit.hhi.fraunhofer.de/jvet-m-ce8/VVCSoftware_VTM/tree/CE8-3.1

- “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, January 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).