| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0457 |
| --- | --- |

| Title: | CE8-1.1: Block vector prediction for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the CE test results for CE8-1.1a and CE8-1.1b. In CE8-1.1a, alternative candidates using default positions, i.e. (-2*width, 0) and (0, -2*height), are tested. In CE8-1.1b, a validation check is added on top of CE8-1.1a. The validation check removes a candidate of which motion information is out of search range for IBC. From experiment results, following BD rate changes are observed over VTM-4.0 with IBC:

For CE8-1.1a:

- AI: 0.00% (CTC), 0.02% (Class F), 0.02% (TGM)

- RA: -0.03% (CTC), -0.02% (Class F), -0.06% (TGM)

- LDB: -0.04% (CTC), -0.11% (Class F), -0.05% (TGM)

For CE8-1.1b:

- AI: -0.04% (CTC), -0.20% (Class F), -0.72% (TGM)

- RA: -0.04% (CTC), -0.18% (Class F), -0.51% (TGM)

- LDB: -0.02% (CTC), -0.29% (Class F), -0.33% (TGM)

Introduction

In JVET-M0332 [1], alternative candidates for block vector prediction in IBC mode were proposed. Zero candidate within MVP for IBC mode is inadequate because block vector of IBC mode must be larger than width and height of the current coding block. There is no zero candidate for merge list for IBC as same manner with MVP. In order to replace zero candidate, alternative candidates using default positions and validation check are tested.

Proposed method

CE 8-1.1a: Alternative candidates using default positions

When IBC mode is used, two alternative candidates are added for both MVP and merge mode. For MVP mode, alternative candidates are inserted into MVP candidate list. The position is before zero candidates. As similar, alternative candidates for merge mode are added into merge candidate list at the last position. If alternative candidates are out of current CTU, proposed candidates are clipped to boundary position.

If only one candidate for MVP and merge candidate list is allowed, the first one of default candidates is added. The maximum number of candidates for MVP and merge mode are unchanged and there is no syntax changes introduced by the proposed method also.

CE 8-1.1b: Alternative candidates using default positions with validation check

CE8-1.1b adds validation check on top of CE8-1.1a. The validation check removes a candidate, which includes motion vector out of search range for IBC, from candidate list. The validation check for merge list is conducted for each candidate. For MVP, the validation check is performed before inserting alternative candidates.

Experimental results

The proposed methods have been implemented on top of VTM-4.0 and tested under CE8 test conditions [2].

Table 1. Performance of test 8-1.1a in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.02% | -0.05% | 102% | 99% |
| Class A2 | 0.00% | 0.03% | 0.00% | 100% | 101% |
| Class B | 0.00% | -0.04% | 0.01% | 100% | 99% |
| Class C | 0.00% | 0.07% | -0.08% | 101% | 98% |
| Class E | -0.01% | -0.02% | 0.05% | 100% | 97% |
| Overall | 0.00% | 0.00% | -0.02% | 101% | 99% |
| Class D | -0.02% | 0.01% | 0.12% | 102% | 98% |
| Class F | 0.02% | -0.03% | -0.09% | 101% | 100% |
| Class SCC | 0.02% | -0.01% | -0.01% | 100% | 100% |

Table 2. Performance of test 8-1.1a in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.01% | -0.26% | 0.00% | 101% | 100% |
| Class A2 | -0.03% | 0.04% | -0.04% | 101% | 100% |
| Class B | -0.02% | -0.10% | -0.20% | 102% | 100% |
| Class C | -0.06% | 0.15% | -0.02% | 104% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.04% | -0.08% | 102% | 101% |
| Class D | -0.03% | -0.05% | 0.09% | 105% | 105% |
| Class F | -0.02% | -0.04% | -0.08% | 103% | 105% |
| Class SCC | -0.06% | -0.01% | 0.00% | 102% | 101% |

Table 3. Performance of test 8-1.1a in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.07% | -0.32% | 102% | 101% |
| Class C | -0.07% | -0.08% | -0.05% | 104% | 101% |
| Class E | -0.03% | 0.15% | -0.08% | 101% | 100% |
| Overall | -0.04% | -0.02% | -0.17% | 102% | 101% |
| Class D | -0.11% | 1.19% | 0.27% | 105% | 102% |
| Class F | -0.11% | 0.51% | -0.45% | 100% | 99% |
| Class SCC | -0.05% | -0.06% | -0.06% | 101% | 100% |

Table 4. Performance of test 8-1.1b in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.01% | -0.02% | -0.07% | 104% | 101% |
| Class A2 | -0.01% | -0.01% | -0.02% | 102% | 101% |
| Class B | -0.04% | -0.05% | -0.06% | 102% | 102% |
| Class C | -0.06% | 0.05% | -0.08% | 105% | 106% |
| Class E | -0.06% | -0.12% | -0.06% | 103% | 103% |
| Overall | -0.04% | -0.03% | -0.06% | 103% | 103% |
| Class D | -0.04% | -0.02% | 0.01% | 104% | 107% |
| Class F | -0.20% | -0.22% | -0.32% | 104% | 105% |
| Class SCC | -0.72% | -0.67% | -0.68% | 102% | 104% |

Table 5. Performance of test 8-1.1b in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.01% | -0.14% | -0.04% | 102% | 100% |
| Class A2 | -0.04% | -0.06% | 0.00% | 102% | 99% |
| Class B | -0.05% | -0.21% | -0.08% | 103% | 99% |
| Class C | -0.05% | 0.19% | 0.02% | 104% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.06% | -0.03% | 103% | 100% |
| Class D | -0.03% | -0.16% | 0.15% | 106% | 105% |
| Class F | -0.18% | -0.27% | -0.31% | 104% | 104% |
| Class SCC | -0.51% | -0.45% | -0.54% | 103% | 100% |

Table 6. Performance of test 8-1.1b in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.05% | 0.07% | 102% | 100% |
| Class C | -0.04% | -0.22% | -0.35% | 104% | 101% |
| Class E | -0.04% | -0.11% | 0.08% | 101% | 98% |
| Overall | -0.02% | -0.08% | -0.07% | 102% | 100% |
| Class D | -0.07% | 0.69% | -0.22% | 105% | 101% |
| Class F | -0.29% | -0.48% | -0.69% | 102% | 99% |
| Class SCC | -0.33% | -0.28% | -0.46% | 102% | 100% |

Conclusion

In this contribution, two additional candidates for block vector prediction on IBC mode are tested in CE8-1.1 as a solution for invalid zero candidate insertion. It is recommended to adopt the proposed candidates for IBC.

Reference

- J. Nam, et al., “CE8: Block vector prediction for CPR (test 8.1.1a and test 8.1.1b),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0333, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- X. Xu, et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” , Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1028, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).