| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0458 |
| --- | --- |

| Title: | CE8-1.2: Block vector coding for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

Proposed text

Binarization process

General

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | (Input == MODE_IBC) ? EG3 : EG1 | CuPredMode[ ][ ] |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

Derivation process for ctxTable, ctxIdx and bypassFlag

        - General

| Table 9-15 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_greater0_flag[ ] | (CuPredMode[ ][ ] != MODE_IBC) ? (0: x ? 1 : 2) | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | (CuPredMode[ ][ ] != MODE_IBC) ? (0: x ? 1 : 2) | na | na | na | na | na |