| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0463 |
| --- | --- |

| Title: | CE11-2.2 Disable deblocking filter for 4xN for vertical edge and Nx4 for Horizontal edge on 4x4 grid. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports a coding efficiency about disabling deblock filtering process on 4xN block boundary for vertical edge and on Nx4 block boundary for horizontal edge in order to parallel deblock processing on 4x4 grid. Experimental results reportedly show -0.05%, -0.02%, -0.04%, 0.00% BD-rate changes for AI, RA, LD-B and LD-P CTC configurations respectively and -0.01%, 0.00%, -0.01%, 0.02% BD-rate changes for AI, RA, LD-B and LD-P CTC configurations without ALF.

Proposed method

The proposed method handles deblocking filter on 4x4 grid with following aspects

- For the luma component, deblocking filter is not applied on left block boundary of 4xN block for vertical edge and deblocking filter is also not applied on above block boundary of Nx4 block for horizontal edge.

- For the chroma component, deblocking filter is not applied on left block boundary of 2xN block for vertical edge and deblocking filter is also not applied on above block boundary of Nx2 block for horizontal edge.

Experimental results

#1. Experiment result for CTC configuration.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.31% | 0.01% | 100% | 99% |
| Class A2 | -0.05% | 0.22% | 0.37% | 100% | 98% |
| Class B | -0.06% | 0.41% | 0.29% | 101% | 99% |
| Class C | -0.06% | 0.14% | 0.03% | 98% | 95% |
| Class E | -0.04% | -0.12% | -0.05% | 98% | 96% |
| Overall | -0.05% | 0.22% | 0.14% | 99% | 98% |
| Class D | -0.05% | 0.16% | 0.14% | 99% | 93% |
| Class F (optional) | -0.01% | 0.20% | 0.32% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.24% | -0.08% | 100% | 99% |
| Class A2 | -0.03% | 0.05% | 0.54% | 100% | 99% |
| Class B | -0.02% | 0.51% | 0.30% | 99% | 97% |
| Class C | -0.02% | 0.18% | 0.00% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.28% | 0.19% | 100% | 98% |
| Class D | -0.03% | 0.39% | 0.21% | 99% | 97% |
| Class F (optional) | 0.01% | 0.42% | 0.55% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.17% | -0.38% | 100% | 98% |
| Class C | -0.02% | 0.11% | -0.26% | 99% | 99% |
| Class E | -0.07% | -0.12% | -0.58% | 99% | 101% |
| Overall | -0.04% | -0.06% | -0.39% | 100% | 99% |
| Class D | -0.02% | 0.14% | -0.35% | 99% | 99% |
| Class F (optional) | -0.01% | 0.09% | 0.19% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.11% | 100% | 99% |
| Class C | -0.02% | -0.36% | -0.62% | 99% | 97% |
| Class E | 0.00% | -0.08% | -0.57% | 99% | 98% |
| Overall | 0.00% | -0.26% | -0.39% | 99% | 98% |
| Class D | 0.02% | 0.01% | -0.34% | 99% | 96% |
| Class F (optional) | 0.11% | -0.05% | 0.38% | 99% | 98% |

#2. Experiment result for CTC configuration without ALF

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | -0.40% | 99% | 99% |
| Class A2 | -0.02% | -0.39% | -0.19% | 100% | 99% |
| Class B | -0.03% | 0.03% | -0.26% | 100% | 99% |
| Class C | -0.03% | -0.28% | -0.50% | 100% | 98% |
| Class E | 0.02% | -0.85% | -1.11% | 99% | 98% |
| Overall | -0.01% | -0.27% | -0.47% | 100% | 98% |
| Class D | -0.02% | -0.21% | -0.40% | 100% | 97% |
| Class F (optional) | 0.01% | 0.09% | 0.21% | 100% | 97% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.19% | 100% | 99% |
| Class A2 | 0.03% | 0.48% | 0.91% | 100% | 99% |
| Class B | -0.01% | 1.05% | 0.74% | 100% | 99% |
| Class C | 0.00% | 0.56% | 0.29% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.60% | 0.47% | 100% | 99% |
| Class D | 0.03% | 0.74% | 0.47% | 100% | 98% |
| Class F (optional) | 0.01% | 0.41% | 0.64% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.14% | -0.13% | 100% | 98% |
| Class C | 0.00% | 0.09% | -0.06% | 100% | 99% |
| Class E | -0.01% | -1.01% | -0.95% | 100% | 101% |
| Overall | -0.01% | -0.17% | -0.31% | 100% | 99% |
| Class D | 0.14% | 0.62% | 0.13% | 100% | 99% |
| Class F (optional) | 0.13% | 0.60% | 0.24% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.16% | 100% | 100% |
| Class C | 0.00% | -0.04% | 0.06% | 100% | 99% |
| Class E | 0.06% | -0.77% | -1.60% | 100% | 99% |
| Overall | 0.02% | -0.17% | -0.31% | 100% | 99% |
| Class D | -0.07% | 0.07% | -0.73% | 100% | 98% |
| Class F (optional) | 0.00% | 0.87% | 0.05% | 101% | 99% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).