| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0464 |
| --- | --- |

| Title: | Non-CE8 : MMVD Motion vector rounding for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In JVET-M0255, MMVD without Fractional Distances for SCC was introduced. MMVD distances are multiplied by 4 and it is interpreted as integer precision when fractional distance MMVD offset is disabled. However the predicted motion vector from merge candidate still has 1/16 precision, then the final MMVD motion vector can be fractional-Pel. In this proposal, it is proposed to round MMVD motion vector to integer precision when the fractional distance disable flag is enabled. Simulation results reportedly show 0.14% and 0.03% luma BD-rate saving on SCC sequences in Random Access configuration and Low Delay B configuration respectively over VTM-4.0. With the proposed method, motion compensation of MMVD does not need to interpolate the reference picture for fractional pel.

Introduction

MMVD without fractional-pel distance restrict MVD precision to integer pel only. Since the predicted motion vector from merge candidate and MVD after scaling operation are still 1/16 precision, final motion vector of MMVD can be 1/16 precision. Fractional motion vector for MMVD can be inefficient for screen contents when fractional-pel distances are disabled.

Proposed method

The proposed method suggests to apply rounding operation for MMVD motion vector based on tile_group_fpel_mmvd_enabled_flag. If tile_group_fpel_mmvd_enabled_flag is 1, the motion vector is rounded from 1/16 to integer precision. In addition, in this case, motion compensation for MMVD does not need to interpolate the reference picture to fractional pel.

tile_group_fpel_mmvd_enabled_flag equal to 1 specifies that merge mode with motion vector difference uses integer sample precision in the current tile group. tile_group_fpel_mmvd_enabled_flag equal to 0 specifies that merge mode with motion vector difference can use fractional sample precision in the current tile group. When not present, the value of tile_group_fpel_mmvd_enabled_flag is inferred to be 0. The variable MmvdMvShift is derived as follows: MmvdMvShift = (tile_group_fpel_mmvd_enabled_flag)<<2

Derivation process for motion vector components and reference indices Derivation process for luma motion vectors for merge mode - When mmvd_flag[ xCb ][ yCb ] is equal to 1, the following applies: - The derivation process for merge motion vector difference as specified in 8.5.2.7 is invoked with the luma location ( xCb, yCb ), the luma motion vectors mvL0[ 0 ][ 0 ], mvL1[ 0 ][ 0 ], the reference indices refIdxL0, refIdxL1 and the prediction list utilization flags predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] as inputs, and the motion vector differences mMvdL0 and mMvdL1 as outputs. - The motion vector difference mMvdLX is added to the merge motion vectors mvLX for X being 0 and 1 as follows: mvLX[ 0 ][ 0 ][ 0 ] += mMvdLX[ 0 ] (8-292) mvLX[ 0 ][ 0 ][ 1 ] += mMvdLX[ 1 ] (8-293) - The rounding process for merge motion vectors as specified in cluase 8.5.2.14 is invoked with mvLX for X being 0 and 1, rightShift set equal to MmvdMvShift, and leftShift set equal to MmvdMvShift as inputs and the rounded mvLX for X being 0 and 1 as outputs.

- Experimental results

#1. Experimental result for CTC configuration over VTM-4.0

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.12% | 0.11% | 101% | 100% |
| Class A2 | 0.17% | 0.07% | 0.29% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.04% | 0.08% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.01% | 0.02% | -0.01% | 100% | 101% |
| Class SCC 1080p | -0.14% | -0.01% | -0.05% | 100% | 101% |

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  | 101% | 100% |
| Class A2 |  |  |  | 101% | 100% |
| Class B | #VALUE! | #VALUE! | #VALUE! | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | -0.12% | 0.03% | -0.94% | 100% | 101% |
| Class SCC 1080p | -0.03% | 0.09% | 0.11% | 100% | 101% |

#2. Experimental result for CTC configuration over VTM-4.0 without encoder option from JVET-M0854.

In the last JVET meeting, it is adopted at the same time that encoder option is applied in order to force tile_group_fpel_mmvd_enabled_flag to 1 based on picture resolution and it is applied automatically tile_group_fpel_mmvd_enabled_flag enabling scheme which is based on the Hash ratio. By the picture resoltion based encoding scheme, only ClassA1 and ClassA2 enabled tile_group_fpel_mmvd_enabled_flag is forced as 1 in spite of that it is disabled by automatical encoder decision scheme. this proposal also reports experimental result about proposed method without picture resolution based encoder scheme.

|  | Over VTM-4.0 without JVET-M0854 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.05% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.01% | 0.02% | -0.01% | 100% | 101% |
| Class SCC 1080p | -0.14% | -0.01% | -0.05% | 100% | 101% |

|  | Over VTM-4.0 without JVET-M0854 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 71% | 74% |
| Class E | 0.00% | 0.00% | 0.00% | 68% | 73% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 71% | 74% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class SCC 1080p | -0.03% | 0.09% | 0.11% | 72% | 75% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).