| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0469 |
| --- | --- |

| Title: | CE3-related : Disable ISP mode for 128xN, Nx128 Block. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Jin Heo Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

Intra Sub-Partitions(ISP) coding mode divides luma intra-predicted blocks vertically and horizontally. This proposal suggests to disable ISP for large block sizes, such as 128xN and Nx128 to align VPDU processing on single tree structure and to handle worst case which is not allowed in current VVC draft specification. Experimental result for the proposed method shows 0.00%, 0.00% BD-rate changes for CTC configuration.

Problem state

Current VVC draft4, in order to align VPDU processing between luma component and chroma components, when current block structure is dual tree structure and the block size is larger than VPDU(64x64) then current block is split by quad-tree implicitly. On the other hand when current block structure is single tree structure, implicit quad-tree partitioning is not necessary because luma and chroma components already aligned with VPDU processing. However if the ISP is enabled on the block of which height or width is greater than VPDU, then miss-aligned VPDU processing occurs. For example, on a single tree structure, a 128x64 block is implicitly split to four 32x64 sub-partitions for luma component, and 64x32 Chroma block is coded without Sub-partitions.

Figure 1-1 shows an example about aligned VPDU processing for 128x64 block which is not coded on single tree structure in current VVC text specification.

Figure 1-2 shows an example of the problem state regarding with miss-aligned VPDU processing when block is bigger than VPDU and the block is coded by ISP on single tree structure.

And here another problem occurs when the ISP is enabled for 128xN, Nx128 block on single tree structure. Current VVC draft4 describes Maximum Transform Size for Chroma block is 32x32 in 4:2:0 video format as described in Table1-1. If current block is 128xN or Nx128 block and it is coded by ISP on single tree structure then 64x(N/2) or (N/2)x64 chroma block corresponded with luma block is not allowed without implicit or ISP sub-partitioning by current VVC draft.

8.4.4 Decoding process for intra blocks 8.4.4.1 General decoding process for intra blocks Inputs to this process are: - a sample location ( xTb0, yTb0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, - a variable nTbW specifying the width of the current transform block, - a variable nTbH specifying the height of the current transform block, - a variable predModeIntra specifying the intra prediction mode, - a variable cIdx specifying the colour component of the current block. Output of this process is a modified reconstructed picture before in-loop filtering. The maximum transform block size maxTbSize is derived as follows: maxTbSize = ( cIdx = = 0 ) ? MaxTbSizeY : MaxTbSizeY / 2 (8-95)

Proposed method

This contribution suggest to handle problem case, it is suggest that ISP coding is disabled for 128xN, Nx128 block.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( tile_group_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && tile_group_type != I ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( tile_group_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( tile_group_type != I && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_ibc_enabled_flag ) |  |
| pred_mode_ibc_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| if( sps_pcm_enabled_flag && cbWidth >= MinIpcmCbSizeY && cbWidth <= MaxIpcmCbSizeY && cbHeight >= MinIpcmCbSizeY && cbHeight <= MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( cbWidth, cbHeight, treeType) |  |
| } else { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( y0 % CtbSizeY ) > 0 ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if (intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY )) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| …. |  |
| } |  |

intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 1 specifies that the current intra coding unit is partitioned into NumIntraSubPartitions[ x0 ][ y0 ] rectangular transform block subpartitions. intra_subpartitions_mode_flag[ x0 ][ y0 ] equal to 0 specifies that the current intra coding unit is not partitioned into rectangular transform block subpartitions. When intra_subpartitions_mode_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0. intra_subpartitions_split_flag[ x0 ][ y0 ] specifies whether the intra subpartitions split type is horizontal or vertical. When intra_subpartitions_split_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0 as follows: - If cbHeight is greater than MaxTbSizeY, intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 0. - Otherwise (cbWidth is greater than MaxTbSizeY), intra_subpartitions_split_flag[ x0 ][ y0 ] is inferred to be equal to 1. The variable IntraSubPartitionsSplitType specifies the type of split used for the current luma coding block as illustrated in Table 7-8. IntraSubPartitionsSplitType is derived as follows: - If intra_subpartitions_mode_flag[ x0 ][ y0 ] is equal to 0, IntraSubPartitionsSplitType is set equal to 0. - Otherwise, the IntraSubPartitionsSplitType is set equal to 1 + intra_subpartitions_split_flag[ x0 ][ y0 ]. Table 7-8 – Name association to IntraSubPartitionsSplitType IntraSubPartitionsSplitType; Name of IntraSubPartitionsSplitType. 0; ISP_NO_SPLIT. 1; ISP_HOR_SPLIT. 2; ISP_VER_SPLIT.

Experimental results

The experimental result for the proposed method shows no BD-rate change and negligible encoding/decoding time change.

#1. Test result for AI CTC configuration.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 100% |

#2. Test result for AI CTC configuration.

|  | Over VTM-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 101% | 100% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).