| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0470 |
| --- | --- |

| Title: | AhG2: Mismatch between text specification and reference software on SMVD. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Naeri Park Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution provides a fix to align specification and software (VTM4) regarding symmetric MVD flag (sym_mvd_flag). Specifically, the parsing condition for sym_mvd_flag and BDOF applying condition based on sym_mvd_flag are updated to match VTM4.

Introduction

In current VVC WD4 [1], mismatches between specification and reference software are found as bellow.

- Mismatch on sym_mvd_flag signaling condition The sym_mvd_flag is only signaled when mvd_l1_zero_flag is equal to 0 in the reference SW.

- Mismatch on BDOF applying condition, the reference SW allows BDOF when sym_mvd_flag is equal to 0. This part is missing on current text specification.

Proposed draft

#1. Mismatch on sym_mvd_flag signaling condition.

According to the VTM4.0 and JVET-M0444, sym_mvd_flag is added as follows in 7.3.6.5. Changes are highlighted by yellow color.

| if( ! mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| --- | --- |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |

#1.1. Alternative suggestion to solve mismatch on sym_mvd_flag signaling condition.

Alternative editorail suggestion has less condition check compared to #1.

- Decoding process for symmetric motion vector difference reference indices

Output of this process are RefIdxSymL0 and RefIdxSymL0 specifying the list 0 and list 1 reference picture indices for symmetric motion vector differences, i.e., when sym_mvd_flag is equal to 1 for a coding unit.

If mvd_l1_zero_flag is equal to 1, RefIdxSymL0 and RefIdxSymL1 is set equal to -1

Otherwise, the following applies:

The variable RefIdxSymLX with X being 0 and 1 is derived as follows:

- The variable currPic specifies the current picture.

- RefIdxSymL0 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 0 ], the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- RefIdxSymL1 is set equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ], the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

- When RefIdxSymL1 is equal to −1 or RefIdxSymL1 is equal to −1, the following applies:

- For each index i with i = 0..NumRefIdxActive[ 0 ], the following applies:

- When all of the following conditions are true, RefIdxSymL0 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) < 0,

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ i ] ) > DiffPicOrderCnt( currPic, RefPicList[ 0 ][ RefIdxSymL0 ] ) or RefIdxSymL0 is equal to −1.

- For each index i with i = 0..NumRefIdxActive[ 1 ], the following applies:

- When all of the following conditions are true, RefIdxSymL1 is set to i:

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) > 0,

- DiffPicOrderCnt( currPic, RefPicList[ 1 ][ i ] ) < DiffPicOrderCnt( currPic, RefPicList[ 1 ][ RefIdxSymL1 ] ) or RefIdxSymL1 is equal to −1.

| if(inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && !inter_affine_flag[ x0 ][ y0 ] && RefIdxSymL0 > −1 && RefIdxSymL1 > −1 ) |  |
| --- | --- |
| sym_mvd_flag[ x0 ][ y0 ] | ae(v) |

#2. Mismatch on BDOF applying condition based on sym_mvd_flag

According to the VTM4.0 and JVET-M0444, sym_mvd_flag is added as follows in 8.5.7.1. Changes are highlighted by yellow color.

- The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

- If all of the following conditions are true, bdofFlag is set equal to TRUE.

- sps_bdof_enabled_flag is equal to 1.

- predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

- MotionModelIdc[ xCb ][ yCb ] is equal to 0.

- merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

- GbiIdx[ xCb ][ yCb ] is equal to 0.

- sym_mvd_flag[ xCb ][ yCb ] is equal to 0

- cIdx is equal to 0.

- Otherwise, bdofFlag is set equal to FALSE.

Conclusion

In this contribution, mismatches between specification and software are fixed regarding SMVD. It is recommended to consider these changes into the next WD.

Reference

- Benjamin Bross, et al., “Versatile Video Coding (Draft 4)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1001, 13th meeting, Marrakech, MA, 9-18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).