| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0472_v2 |
| --- | --- |

| Title: | Non-CE8: On IBC reference buffer design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document analyzes several issues in the current IBC reference buffer design and presents a different design to address the issues. An independent IBC reference buffer is proposed instead of mixing with decoding memory. With the design, most bitstream conformance constrains can be removed. Compared with the current anchor, the proposed scheme shows -0.99%/-0.71%/-0.79% AI/RA/LD-B luma BD-rate for class F and -2.57%/-1.81%/-1.36% for 4:2:0 TGM, with 6.7% memory reduction; or -1.31%/-1.01%/-0.81% for class F and -3.23%/-2.33%/-1.71% for 4:2:0 TGM with 6.7% memory increase.

# Introduction

Intra block copy, i.e. IBC (or current picture referencing, i.e. CPR previously) coding mode, was adopted into VTM at the 12th JVET meeting. It was realized that IBC reference samples should be stored in on-chip memory and thus a limited reference area of one CTU is defined. To restrict the extra on-chip memory for the buffer, the current design reuses the 64x64 memory for decoding the current VPDU so that only 3 additional 64x64 blocks’ memory is needed to support IBC. When CTU size is 128x128, currently the reference area is shown in Fig. 1, as proposed in JVET-M0407.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Figure 1. Reference area defined in VTM-4.0, where a block denotes a 64x64 area.

In the current draft (VVC draft 4), the area is defined as

Thus, the total reference size is a CTU.

# Potential issues of the current design

The current design assumes to reuse the 64x64 memory for decoding the current VPDU and the IBC reference is aligned to VPDU memory reuse accordingly. Such a design bundles VPDU decoding memory with the IBC buffer. There might be several issues:

- To handle smaller CTU size might be an issue. Suppose that CTU size is 32x32, it is not clear whether the current 64x64 memory for decoding the current VPDU can support 32x32 level memory reuse efficiently in different architectures.

- The reference area varies significantly. Accordingly, too many bitstream conformance constrains are introduced. It places extra burden to encoder to exploit reference area efficiently and avoid generating legal bitstreams. It also increases the possibility to have invalid BVs in different modules, e.g. merge list. To handle those invalid BVs may introduce extra logics or extra conformance constrains. It not only introduces burdens to encoder or decoder, it may also create divergence between BV coding and MV coding.

- The design does not scale well. Because VPDU decoding is mixed with IBC buffer, it is not easy to increase or decrease reference area relative to the current one 128x128 CTU design. It may limit the flexibility to exploit a better coding efficiency vs. on-chip memory trade-off in the later development, e.g. a lower or higher profile.

- The bit-depth of IBC reference buffer is linked with decoding buffer. Even though screen contents usually have a lower bit-depth than internal decoding bit-depth, the buffer still needs to spend memory to store bits mostly representing rounding or quantization noises. The issue becomes even severe when considering higher decoding bit-depth configurations.

# A clear IBC buffer design

To address issues listed in the above section, we propose to have a dedicated IBC buffer, which is not mixed with decoding memory.

For 128x128 CTU, the buffer is defined as 128x128 with 8-bit samples, when a CU (x, y) with size wxh has been decoded, its reconstruction before loop-filtering is converted to 8-bit and written to the wxh block area starting from position (x%128, y%128). Here the modulo operator % always returns a positive number, i.e. for x<0,  x%L≜-(-x%L), e.g. -3%128=125.

Assume that a pixel (x,y) is coded in IBC mode with BV=(BVx, BVy), it is prediction sample in the IBC reference buffer locates at ((x+BVx)%128, (y+BVy)%128) and the pixel value will be converted to 10-bit before prediction.

When the buffer is considered as (W, H), after decoding a CTU or CU starting from (x, y), the reconstructed pixels before loop-filtering will be stored in the buffer starting from (x%W, y%H). Thus, after decoding a CTU, the corresponding IBC reference buffer will be updated accordingly. Such setting might happen when CTU size is not 128x128. For example, for 64x64 CTU, with the current buffer size, it can be considered as a 256x64 buffer. For 64x64 CTU, figure 2 shows the buffer status.

Figure 2. Illustration of IBC reference buffer status, where a block denotes a 64x64 CTU.

In such a design, because the IBC buffer is different from the VPDU decoding memory, all the IBC reference buffer can be used as reference.

When the bit-depth of the IBC buffer is 8-bit, compared with the current design that needs 3 additional 10-biot 64x64 buffer, the on-chip memory increase is (8*4)/(10*3)-100%=6.7%.

If we further reduce the bit-depth. The memory requirement can be further reduced. For example, for 7-bit buffer, the on-chip memory saving is 100%-(7*4)/(10*3)=6.7%.

With the design, the only bitstream conformance constrain is that the reference block shall be within the reconstructed area in the current CTU row of the current Tile.

When initialization with 512 is allowed at the beginning of each CTU row, all bitstream conformance constrains can be removed.

# Experimental results

We implemented the proposed scheme based on VTM-4.0 software.

For a 10-bit buffer implementation and CTC, the decoder is fully compatible to the current VTM4.0 encoder, which means that the proposed decoder can exactly decode the VTM-4.0 CTC bitstreams.

For a 7-bit buffer implementation, the results shown in Table I.

For a 8-bit buffer implementation, the results shown in Table II.

Table I. Performance with a 7-bit buffer. The anchor is VTM-4.0 with IBC on for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.09% | -0.10% | 132% | 101% |
| Class A2 | 0.05% | 0.00% | 0.06% | 135% | 100% |
| Class B | 0.00% | -0.02% | 0.01% | 135% | 100% |
| Class C | -0.02% | 0.01% | 0.03% | 130% | 98% |
| Class E | -0.13% | -0.16% | -0.04% | 135% | 99% |
| Overall | -0.02% | -0.05% | 0.00% | 133% | 100% |
| Class D | 0.04% | 0.04% | 0.12% | 127% | 107% |
| Class F | -0.99% | -1.14% | -1.18% | 115% | 99% |
| 4:2:0 TGM | -2.57% | -2.73% | -2.67% | 104% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.01% | 109% | 100% |
| Class A2 | 0.00% | -0.04% | 0.03% | 111% | 100% |
| Class B | -0.01% | -0.10% | -0.22% | 113% | 101% |
| Class C | -0.01% | 0.17% | 0.12% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | -0.04% | 112% | 100% |
| Class D | 0.05% | 0.16% | 0.20% | 117% | 101% |
| Class F | -0.71% | -0.77% | -0.77% | 109% | 99% |
| 4:2:0 TGM | -1.81% | -1.65% | -1.64% | 107% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.36% | 0.30% | 114% | 95% |
| Class C | -0.01% | -0.12% | -0.10% | 120% | 98% |
| Class E | 0.10% | 0.20% | 0.18% | 107% | 99% |
| Overall | 0.03% | 0.16% | 0.13% | 114% | 97% |
| Class D | -0.01% | 1.07% | 0.18% | 123% | 104% |
| Class F | -0.79% | -0.89% | -1.01% | 110% | 100% |
| 4:2:0 TGM | -1.36% | -1.30% | -1.26% | 109% | 102% |

Table II. Performance with a 8-bit buffer. The anchor is VTM-4.0 with IBC on for all sequences.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.10% | 129% | 102% |
| Class A2 | 0.02% | -0.06% | -0.02% | 134% | 102% |
| Class B | -0.04% | -0.02% | -0.07% | 135% | 101% |
| Class C | -0.03% | 0.04% | 0.00% | 130% | 98% |
| Class E | -0.16% | -0.14% | -0.08% | 134% | 100% |
| Overall | -0.04% | -0.03% | -0.05% | 133% | 100% |
| Class D | 0.00% | 0.04% | 0.02% | 126% | 101% |
| Class F | -1.31% | -1.27% | -1.29% | 114% | 98% |
| 4:2:0 TGM | -3.23% | -3.27% | -3.24% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.08% | 0.04% | 107% | 99% |
| Class A2 | -0.03% | -0.16% | 0.06% | 110% | 99% |
| Class B | -0.01% | -0.14% | -0.22% | 111% | 99% |
| Class C | -0.01% | 0.15% | 0.09% | 115% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.05% | -0.03% | 111% | 99% |
| Class D | 0.01% | 0.19% | 0.22% | 116% | 101% |
| Class F | -1.01% | -0.99% | -1.01% | 108% | 99% |
| 4:2:0 TGM | -2.33% | -2.14% | -2.19% | 105% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.04% | -0.14% | 113% | #NUM! |
| Class C | -0.05% | -0.28% | -0.15% | 119% | 98% |
| Class E | 0.04% | -0.16% | 0.43% | 107% | #NUM! |
| Overall | 0.00% | -0.11% | 0.00% | 113% | #NUM! |
| Class D | -0.07% | 1.14% | 0.13% | 122% | 99% |
| Class F | -0.81% | -0.92% | -0.96% | 111% | 99% |
| 4:2:0 TGM | -1.71% | -1.67% | -1.71% | 106% | 95% |

Table III. Performance for a 8-bit 96x128 buffer

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.43% | -0.32% | -0.38% | 109% | 105% |
| 4:2:0 TGM | -0.93% | -0.94% | -0.94% | 97% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-4.0 w/ IBC on |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.35% | -0.38% | -0.42% | 105% | 99% |
| 4:2:0 TGM | -1.19% | -0.96% | -1.02% | 103% | 101% |

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).