| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0473 |
| --- | --- |

| Title: | Non-CE11: On ISP transform boundary deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Andrew Segall1, Masaru Ikeda2, Teruhiko Suzuki2 | Email: | misrak@sharplabs.com asegall@sharplabs.com masaru.ikeda@sony.com teruhiko.s@sony.com |
| Source: | 1Sharp Labs of America, 2Sony Corp |  |  |

_____________________________

# Abstract

In the VTM software and VVC WD, transform boundaries inside an intra sub partition (ISP) luma coding block do not undergo deblocking while transform boundaries resulting from an equivalent BT partitioning do undergo deblocking.  It is asserted to be desirable that the deblocking of ISP partitioning and regular binary tree (BT) partitioning be consistent.  This contribution proposes that transform boundaries inside an ISP luma coding block aligned with the 8x8 deblocking grid should also undergo deblocking.

# Introduction

In the current VTM4.0 [1] software and VVC WD [2], transform boundaries inside an intra sub partition (ISP) luma coding block do not undergo deblocking while an equivalent partitioning using the MTT would undergo deblocking. The ISP internal boundaries are shown in red in Figure below.

It is asserted that this behavior is not desirable. This contribution proposes that transform boundaries inside an intra sub partition (ISP) luma coding block should also undergo deblocking. This is achieved by modifying the transform boundary marking process to mark the edges inside a ISP luma coding block. The modifications to VVC WD corresponding to this change is listed in document attached to this contribution.

# Recommendation

It is recommended to adopt the proposed ISP deblocking process in the next version of the VVC WD and VTM.

# References

[1] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-4.0 VTM-4.0

[2] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 4),” JVET-M1001 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010 13th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, 9–18 Jan. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).