Text modification highlighted in yellow below. The changes are shown on top of WD text provided for the adoption of JVET-M0471:

#### Deblocking filter process for one direction

Inputs to this process are:

- the variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the reconstructed picture prior to deblocking, i.e., the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are the modified reconstructed picture after deblocking, i.e:

- when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL,

- when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr.

For each coding unit with coding block width log2CbW, coding block height log2CbH and location of top-left sample of the coding block ( xCb, yCb ), when edgeType is equal to EDGE_VER and xCb % 8 is equal 0 or when edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The coding block width nCbW is set equal to 1  <<  log2CbW and the coding block height nCbH is set equal to 1  <<  log2CbH

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- Otherwise if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile group and tile_group_loop_filter_across_tile_groups_enabled_flag is equal to 0.

- Otherwise, filterEdgeFlag is set equal to 1.

[Ed. (BB): Adapt syntax once tiles are integrated.]

- All elements of the two-dimensional (nCbW)x(nCbH) arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterlengthPs and a two-dimensional (nCbW/SubWidthC)x(nCbH/SubHeightC) array maxFilterLengthPCbCrs are initialized to be equal to zero. The variable isSBTSubblock is initialized to 0.

- The derivation process of transform block boundary specified in clause 8.8.2.3 is invoked with the location ( xB0, yB0 ) set equal to ( 0, 0 ), the block width nTbW set equal to nCbW, the block height nTbH set equal to nCbH, the variable treeType, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable isISPSubblock as inputs, and the modified arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs as output.

- The derivation process of coding subblock boundary specified in clause 8.8.2.4 is invoked with the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs, and the variable edgeType as inputs, and the modified arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs and maxFilterLengthPs as output.

- The picture sample array recPicture is derived as follows:

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, recPicture is set equal to the reconstructed luma picture sample array prior to deblocking recPictureL.

- Otherwise (treeType is equal to DUAL_TREE_CHROMA), recPicture is set equal to the reconstructed chroma picture sample array prior to deblocking recPictureCb.

- The derivation process of the boundary filtering strength specified in clause 8.8.2.5 is invoked with the picture sample array recPicture, the luma location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, the variable edgeType, and the array edgeFlagYs, edgeFlagCbCrs as inputs, and an (nCbW)x(nCbH) array bSLuma, and  (nCbW/SubWidthC)x(nCbH/SubHeightC) arrays bSCb and bSCr as output.

- The edge filtering process is invoked as follows:

- If edgeType is equal to EDGE_VER, the vertical edge filtering process for a coding unit as specified in clause 8.8.2.6.1 is invoked with the variable treeType, the reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the arrays bSLuma, bSCb, bSCr, maxFilterLengthPs, maxFilterLengthQs and maxFilterLengthCbCrs as inputs, and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

- Otherwise if edgeType is equal to EDGE_HOR, the horizontal edge filtering process for a coding unit as specified in clause 8.8.2.6.2 is invoked with the variable treeType, the modified reconstructed picture prior to deblocking, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, the location ( xCb, yCb ), the coding block width nCbW, the coding block height nCbH, and the arrays bSLuma, bSCb, bSCr, maxFilterLengthPs, maxFilterLengthQs and maxFilterLengthCbCrs as inputs and the modified reconstructed picture, i.e., when treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the array recPictureL and, when ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the arrays recPictureCb and recPictureCr, as output.

#### Derivation process of transform block boundary

Inputs to this process are:

- a location ( xB0, yB0 ) specifying the top-left sample of the current block relative to the top-left sample of the current coding block,

- a variable nTbW specifying the width of the current block,

- a variable nTbH specifying the height of the current block,

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- a variable filterEdgeFlag,

- the two-dimensional (nCbW)x(nCbH) arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and a two-dimensional (nCbW/SubWidthC)x(nCbH/SubHeightC) array maxFilterLengthCbCrs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

- a variable isSBTSubblock specifying if current block is a SBT subblock.

- a variable ispSubblockIdx specifying index of a ISP subblock

Output of this process is the modified two-dimensional (nCbW)x(nCbH) arrays edgeFlagYs, edgeFlagCbCrs,  maxFilterLengthQs, maxFilterLengthPs and the two-dimensional (nCbW/SubWidthC)x(nCbH/SubHeightC) array maxFilterLengthCbCrs.

The maximum transform block size maxTbSize is derived as follows:

maxTbSize = ( treeType  = =  DUAL_TREE_CHROMA ) ? MaxTbSizeY / 2 : MaxTbSizeY	(8-1055)

Depending on maxTbSize, the following applies:

- If nTbW is greater than maxTbSize or nTbH is greater than maxTbSize, the following ordered steps apply.

- The variables newTbW and newTbH are derived as follows:

newTbW = ( nTbW  >  maxTbSize ) ? ( nTbW / 2 ) : nTbW	(8-1056)

newTbH = ( nTbH   >  maxTbSize ) ? ( nTbH / 2 ) :  nTbH	(8-1057)

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0, yB0 ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

- If nTbW is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xB0, yB0 ) set equal to ( xB0 + newTbW, yB0 ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs,  the variable edgeType, and the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

- If nTbH is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xB0, yB0 ) set equal to ( xB0, yB0 + newTbH ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

- If nTbW is greater than maxTbSize and nTbH is greater than maxTbSize, the derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xB0, yB0 ) set equal to ( xB0 + newTbW, yB0 + newTbH ), the variables nTbW set equal to newTbW and nTbH set equal to newTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

–	Otherwise, if edgeType is equal to EDGE_VER, cu_sbt_flag[ xB0 ][ yB0 ] is equal to 1, and cu_sbt_horizontal_flag[ xB0 ][ yB0 ] is equal to 0, and isSBTSubblock is equal to 0, the following ordered steps apply:

- The variable isSBTSubblock is set to 1

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0, yB0 ), the variable nTbW set equal to nTbW * SbtNumFourthsTb0 / 4, the variable nTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

- The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xB0 + nTBW * SbtNumFourthsTb0 / 4, yB0 ), the variable nTbW set equal to (nTbW - nTbW * SbtNumFourthsTb0 / 4), the variable nTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock , and the variable ispSubblockIdx and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

–	Otherwise, if edgeType is equal to EDGE_HOR, cu_sbt_flag[ xB0 ][ yB0 ] is equal to 1, and cu_sbt_horizontal_flag[ xB0 ][ yB0 ] is equal to 1, and isSBTSubblock is equal to 0, the following ordered steps apply:

- The variable isSBTSubblock is set to 1

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0, yB0 ), the variable nTbW, the variable nTbH set equal to nTbH * SbtNumFourthsTb0 / 4, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

- The derivation process of transform block boundary as specified in this clause is invoked with the luma location ( xB0 , yB0 + nTBH * SbtNumFourthsTb0 / 4 ), the variable nTbW, the variable nTbH set equal to (nTbH - nTbH * SbtNumFourthsTb0 / 4), the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

–	Otherwise, if edgeType is equal to EDGE_VER, treeType is not equal to DUAL_TREE_CHROMA, intra_subpartitions_mode_flag[ xB0 ][ yB0 ] is equal to 1, and IntraSubPartitionsSplitType[ xB0 ][ yB0 ] is equal to ISP_VER_SPLIT, and ispSubblockIdx is equal to 0, the following step applies:

- For k = 0 .. (NumIntraSubPartitions – 1), the following ordered steps apply:

- The variable ispSubblockIdx is set to (k + 1)

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0 + k * nTbW / NumIntraSubPartitions, yB0 ), the variable nTbW set equal to nTbW / NumIntraSubPartitions, the variable nTbH, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagsCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

–	Otherwise, if edgeType is equal to EDGE_HOR, treeType is not equal to DUAL_TREE_CHROMA, intra_subpartitions_mode_flag[ xB0 ][ yB0 ] is equal to 1, and IntraSubPartitionsSplitType[ xB0 ][ yB0 ] is equal to ISP_HOR_SPLIT, and ispSubblockIdx is equal to 0, the following step applies:

- For k = 0 .. (NumIntraSubPartitions – 1), the following applies:

- The variable ispSubblockIdx is set to (k + 1)

- The derivation process of transform block boundary as specified in this clause is invoked with the location ( xB0, yB0 + k * nTbH / NumIntraSubPartitions ), the variable nTbW, the variable nTbH set equal to nTbH / NumIntraSubPartitions, the variable filterEdgeFlag, the arrays edgeFlagYs, edgeFlagsCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs, the variable edgeType, the variable isSBTSubblock, and the variable ispSubblockIdx as inputs, and the output is the modified version of arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs, maxFilterLengthPs and maxFilterLengthCbCrs.

–	Otherwise, the following applies:

- If treeType is equal to DUAL_TREE_CHROMA, set both variables cW and cH to 1,

- Otherwise, assign SubWidthC to variable cW, and SubHeightC to variable cH

- If edgeType is equal to EDGE_VER, the value of edgeFlagYs[ xB0 ][ yB0 + k ] and edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] for k = 0..nTbH − 1 is derived as follows:

- If xB0 is equal to 0, edgeFlagYs[ xB0 ][ yB0 + k ] and edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] is set equal to filterEdgeFlag.

- Otherwise,

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA edgeFlagYs[ xB0 ][ yB0 + k ] is set equal to 1.

- If ispSubblockIdx is greater than 1, edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] is set equal to 0

- Otherwise, edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] is set equal to 1

    - If edgeFlagYs[ xB0 ][ yB0 + k ] is equal to 1

- If width in luma samples of transform block at luma position (xB0, yB0+k) is equal to or greater than 32, maxFilterLengthQs[ xB0 ][ yB0 + k ] = 7

- Otherwise, maxFilterLengthQs[ xB0 ][ yB0 + k ] = 3

- If width in luma samples of transform block at luma position (xB0 − 1, yB0+k) is equal to or greater than 32, maxFilterLengthPs[ xB0 ][ yB0 + k ] = 7

- Otherwise, maxFilterLengthPs[ xB0 ][ yB0 + k ] = 3

    - If edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] is equal to 1

- If width in chroma samples of transform block at chroma position (xB0/cW, (yB0+k)/cH) and chroma position (xB0/cW − 1, (yB0+k)/cH) both are equal to or greater than 8, maxFilterLengthCbCrs[ xB0/cW ][ (yB0+k)/cH ] = 3

- Otherwise, maxFilterLengthCbCrs[ xB0/cW ][ (yB0+k)/cH] = 1

- Otherwise (edgeType is equal to EDGE_HOR), the value of edgeFlagYs[ xB0 + k ][ yB0 ] and edgeFlagCbCrs[ (xB0 + k)/cW ][ yB0/cH ] for k = 0..nTbW − 1 is derived as follows:

- If yB0 is equal to 0, edgeFlagYs[ xB0 + k ][ yB0 ] and edgeFlagCbCrs[ (xB0 + k)/cW ][ yB0/cH ] is set equal to filterEdgeFlag.

- Otherwise,

- If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA edgeFlagYs[ xB0 + k ][ yB0 ] is set equal to 1.

- If ispSubblockIdx is greater than 1, edgeFlagCbCrs[ (xB0 + k)/cW ][ yB0/cH ] is set equal to 0

- Otherwise, edgeFlagCbCrs[ (xB0 + k)/cW ][ yB0/cH ] is set equal to 1

- If edgeFlagYs[ xB0+k ][ yB0 ] is equal to 1,

- If height in luma samples of transform block at luma position (xB0+k, yB0) is equal to or greater than 32, maxFilterLengthQs[ xB0+k ][ yB0 ] = 7

- Otherwise, maxFilterLengthQs[ xB0+k ][ yB0 ] = 3

- If height in luma samples of transform block at luma position (xB0+k, yB0 − 1) is equal to or greater than 32, maxFilterLengthPs[ xB0+k ][ yB0 ] = 7

- Otherwise, maxFilterLengthPs[ xB0+k ][ yB0 ] = 3

- If edgeFlagCbCrs[ xB0/cW ][ (yB0 + k)/cH ] is equal to 1

- If height in chroma samples of transform block at chroma position ((xB0+k)/cW, yB0/ cH) and chroma position ((xB0+k)/cW, yB0/cH − 1) both are equal to or greater than 8,

- Set yC as the top position of the current block in chroma samples relative to the top position of the current picture

- If  (yC %  (1<<(log2_ctu_size_minus2 +2))/cH  is equal to 0) (when the edge type is EDGE_HOR and the edge overlaps with the chroma CTB boundary), maxFilterLengthCbCrs[ (xB0+k)/cW ][ yB0/ cH ] = 1

- Otherwise, maxFilterLengthCbCrs[ (xB0+k)/cW][ yB0/ cH ] = 3

#### Derivation process of coding subblock boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- the two-dimensional (nCbW)x(nCbH) arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs and maxFilterLengthPs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Output of this process is the modified two-dimensional (nCbW)x(nCbH) arrays edgeFlagYs, edgeFlagCbCrs, maxFilterLengthQs and  maxFilterLengthPs.

The number of coding subblock in horizontal direction numSbX and in vertical direction numSbY are derived as follows:

- If CuPredMode[ xCb ][ yCb ]  !=  MODE_INTER, numSbX and numSbY are both set equal to 1.

- Otherwise, numSbX and numSbY are set equal to NumSbX[ xCb ][ yCb ] and NumSbY[ xCb ][ yCb ], respectively.

Depending on the value of edgeType the following applies:

- If edgeType is equal to EDGE_VER and numSbX is greater than 1,

  - an array of edgeTransformFlags is created by making a copy of the array of edgeFlagYs

  - the following applies for i = 1..min( ( nCbW / 8 ) − 1, numSbX − 1), k = 0..nCbH − 1:

edgeFlagYs[ i * Max( 8, nCbW / numSbX ) ][ k ] = 1	(8-1058)

edgeFlagCbCrs[ i * Max( 8, nCbW / (numSbX*SubWidthC) ) ][ k/SubHeightC ] = 1	(8-xxxx)

  - the following applies for i = 0..min( ( nCbW / 8 ) − 1, numSbX − 1), k = 0..nCbH − 1:

If edgeFlagYs[ i * Max( 8, nCbW / numSbX ) ][ k ]  is equal to 1,

If luma component

    - If  i is equal to 0,

      - If  numSbX at location (xCb-1,yCb)  is greater than 1,

maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] =min(5, maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] )

maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] =min(5,         maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] )

    - Otherwise if edgeTransfromFlags[i * Max( 8, nCbW / numSbX ) ][ k ] is equal to 1

maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] =min(5, maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] )

maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] =min(5, maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] )

    - Otherwise if  i is equal to 1 or i is equal to nCbW/8-1 or (edgeTransfromFlags[(i-1) * Max( 8, nCbW / numSbX ) ][ k ] is equal to 1 ) or (edgeTransfromFlags[(i+1) * Max( 8, nCbW / numSbX ) ][ k ] is equal to 1 )

maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] = 2

maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] = 2

    - Otherwise

maxFilterLengtPs[i * Max( 8, nCbW / numSbX ) ][ k ] = 3

maxFilterLengtQs[i * Max( 8, nCbW / numSbX ) ][ k ] = 3

- Otherwise if edgeType is equal to EDGE_HOR and numSbY is greater than 1,

  - an array of edgeTransformFlags is created by making a copy of the array of edgeFlagYs

  - the following applies for j = 1..min( ( nCbH / 8 ) − 1, numSbY − 1 ), k = 0..nCbW − 1:

edgeFlagYs[ k ][ j * Max( 8, nCbH / numSbY ) ] = 1	(8-1059)

edgeFlagCbCrs[ k/SubWidthC ][ j * Max( 8, nCbH / (numSbY*SubHeightC) ) ] = 1	(8-xxxx)

  - The following applies for j = 0..min( ( nCbH / 8 ) − 1, numSbY − 1), k = 0..nCbW − 1:

If edgeFlagYs[ k ][ j * Max( 8, nCbH / numSbY ) ] is equal to 1

If luma component

    - If  j is equal to 0,

      - If  numSbY at location (xCb,yCb-1)  is greater than 1,

maxFilterLengtPs[ k ][j * Max( 8, nCbH / numSbY ) ] =min(5,         maxFilterLengtPs[ k ] [j * Max( 8, nCbH / numSbY ) ])

maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ] = min(5, 	                           maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ] )

    - Otherwise if edgeTransformFlags[ k ][ j * Max( 8, nCbH / numSbY ) ] is equal to 1

maxFilterLengtPs[ k ] [j * Max( 8, nCbH / numSbY ) ] = min(5, maxFilterLengtPs[ k ] [j * Max( 8, nCbH / numSbY ) ] )

maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ] = min(5, maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ])

    - Otherwise if  j is equal to 1 or j is equal to nCbH/8-1 or (edgeTransformFlags[ k ][ (j-1) * Max( 8, nCbH / numSbY ) ] is equal to 1) or (edgeTransformFlags[ k ][ (j+1) * Max( 8, nCbH / numSbY ) ] is equal to 1)

maxFilterLengtPs[ k ] [j * Max( 8, nCbH / numSbY ) ] = 2

maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ] = 2

    - Otherwise

maxFilterLengtPs[ k ] [j * Max( 8, nCbH / numSbY ) ] = 3

maxFilterLengtQs[ k ] [j * Max( 8, nCbH / numSbY ) ] = 3.

…

If edgeType is equal to EDGE_VER, the following ordered steps apply:

…

- The variables sidePisLargeBlk and sideQisLargeBlk are derived as follows:

If any of the the following conditions are true, set sidePisLargeBlk is set equal to 1:

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 0 and p0,0 belongs to coding block with width in luma samples greater than or equal to 32

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 1, intraSubPartitionsSplitType[][] of coding block to which p0,0 belongs is equal to ISP_HOR_SPLIT, and p0,0 belongs to coding block with width in luma samples greater than or equal to 32

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 1, intraSubPartitionsSplitType[][] of coding block to which p0,0 belongs is equal to ISP_VER_SPLIT, and p0,0 belongs to transform with width in luma samples greater than or equal to 32

Otherwise, sidePisLargeBlk is set equal to 0.

If q0,0 belongs to coding block with width in luma samples greater than or equal to 32, sideQisLargeBlk is set equal to 1 otherwise sideQisLargeBlk is set equal to 0.

…

–    Otherwise (edgeType is equal to EDGE_HOR), the following ordered steps apply:

…

- The variables sidePisLargeBlk and sideQisLargeBlk are derived as follows:

If (luma position of sample p0,0 % CtbSizeY) is equal to 0, set sidePisLargeBlk to 0

Otherwise, If any of the the following conditions are true, set sidePisLargeBlk is set equal to 1:

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 0 and p0,0 belongs to coding block with height in luma samples greater than or equal to 32

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 1, intraSubPartitionsSplitType[][] of coding block to which p0,0 belongs is equal to ISP_VER_SPLIT, and p0,0 belongs to coding block with height in luma samples greater than or equal to 32

- intra_subpartitions_mode_flag[][] of coding block to which p0,0 belongs is equal to 1, intraSubPartitionsSplitType[][] of coding block to which p0,0 belongs is equal to ISP_HOR_SPLIT, and p0,0 belongs to transform with height in luma samples greater than or equal to 32

Otherwise, sidePisLargeBlk is set equal to 0.

If q0,0 belongs to coding block with height greater than or equal to 32, sideQisLargeBlk is set equal to 1 otherwise sideQisLargeBlk is set equal to 0.