| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0474 |
| --- | --- |

| Title: | Comments on algorithm description document for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sang-hyo Park Je-Won Kang | Email: | sanghyo.park@ewha.ac.kr jewonk@ewha.ac.kr |
| Source: | Ewha Womans University |  |  |

_____________________________

- Abstract

This contribution addresses comments on algorithm description for VTM4, especially on encoder aspects. The main point of comments is simply copying the same encoding process as HM has with minor modification that suits VVC-specific terms.

Introduction

Although many tools were added or modified in VTM in comparison with HM, there are still same or very similar processes in VTM with HM. For instance, cost functions like SAD, traditional integer-pel motion search (Diamond search), and fast encoder modes such as reduction of bi-predictive iterations. With minor modification on terms such as PU by replacing CU as in VTM, we can simply borrow the latest description of HM encoder [1] to the algorithm description for VTM [2] so that the algorithm description document could be more readable and helpful to future readers interested in VVC encoder. Accordingly, we suggest following description (similar with HM encoder description [1]) be included in the next version of algorithm description for VTM.

Comment 1: Cost Functions

There is no definition nor description for cost functions used in [2] although cost functions should be clarified. Thus, we recommend the following description (from [1] with minor modification highlighted in yellow) be added to Section 4 in [2]:

- Cost Functions Various cost functions are used in the VTM encoder to determine costs used in making encoder decisions. This section documents the cost functions used in the encoding process of the VTM software. - Sum of Square Error (SSE) The difference between two blocks with the same block size is produced using Diff(i,j) = BlockA(i,j) - BlockB(i,j) (7-1) SEE is computed using the following equation: (7-2) - Sum of Absolute Difference (SAD) SAD is computed using the following equation: (7-3) - Hadamard transformed SAD (SATD) Since the transformed coefficients are coded, an improved estimation of the cost of each mode can be obtained by estimating DCT with the Hadamard transform. SATD is computed using: (7-4) The Hadamard transform flag can be turned on or off. SA(T)D refers to either SAD or SATD depending on the status of the Hadamard transform flag. SAD is used when computing full-pel motion estimation while SA(T)D is used for sub-pel motion estimation.

- Comment 2: Motion Estimation

There is no definition nor description for integer-pel motion search in [2] even though very similar approach is utilized with HM [1]. Thus, we recommend the following description (from [1] with minor modification highlighted in yellow) be added to Section 4 in [2]:

Inter prediction mode and parameters Derivation of motion parameters To be added. Motion estimation To derive the motion vector(s) for each CU, a block matching algorithm is performed in the VTM encoder. The motion vector accuracy supported in VVC is quarter-pel. To generate half-pel and quarter-pel accuracy samples, interpolation filtering is performed for reference picture samples. Instead of searching all the positions for quarter-pel accuracy motion, a motion vector aligned to integer-pel positions is first obtained. For the half-pel search, the eight sample points at half-pel accuracy around the motion vector which has the minimum cost are searched. Similarly, for the quarter-pel search, the eight sample points at quarter-pel accuracy around the motion vector which has the minimum cost so far are searched. The motion vector which has the minimum cost is selected as the motion vector of the CU. To get the cost, SAD is used for integer-pel motion search and SA(T)D is used for half-pel and quarter-pel motion search. The rate for motion vector is obtained by utilizing a pre-calculated rate table. In the following sub-sections, algorithms for integer-pel motion search are described. Integer-pel accuracy motion search For AMVP, find the best candidate MV predictor for each ref_idx and ref_pic_list using xEstimateMvPredAMVP(), called from predInterSearch(). One of four supported integer search algorithms is selected using to the FastSearch configuration option, with the following options being available: - Full search - Diamond search - Selective search - Enhanced diamond search Then, aspects of the above selected search algorithm are modified according to the fast encoder modes (FEN) configuration option: - Fast mode disabled - Fast mode 1 - Fast mode 2 - Fast mode 3 The default minimum search range for the first search in the VTM encoder is 96 integer pixels, for random access configuration. However, low-delay or low-delay P use a value of 64. A search window is defined according to the search range, relative to the best candidate MV predictor. Firstly an integer-pel search is performed, followed by a fractional-pel refinement search. These searches are described in more detail below: Full search When the full search is selected, every integer location within the defined search range is tested to find the best candidate motion vector. When fast mode 1 or 3 is selected, a subsampled SAD is used to speed up distortion measurement. The SAD is subsampled to check only every second row for blocks with greater than eight rows. One predictor is used in the search; set to the best candidate MV predictor for the considered reference picture. Diamond and extended diamond searches A best starting point is selected from a set of candidates as generated by fillMvpCand(). If only one candidate was generated, that becomes the starting point. Otherwise, the candidate resulting in minimal cost, with distortion (SAD) measured in the luma channel, is selected as the optimal motion vector predictor. See the function xEstimateMvPredAMVP(). The motion vector corresponding to the PART_2Nx2N CU is also tested for other partition modes of the CU. If the enhanced diamond search is enabled then the neighbour predictors PRED_A, PRED_B and PRED_C are tested using xTZSearchHelp(). The zero motion vector is also tested using xTZSearchHelp(). The result of this testing is selection of the motion vector predictor. Then, a first search is performed to select an integer-pel accuracy motion vector. The first search begins with an iterative 8-point search. The 8-point search uses a distance parameter iDist to control the distance of the tested points relative to the chosen starting location. Additionally, the pattern of the test points also depends on the value of iDist. When iDist is equal to one, relative to the starting location (red), eight neighbour points (light and dark blue) are tested, as shown in Figure 6-5. Testing of the light blue points is enabled according to a function parameter and thus is dependent upon the invocation of the 8-point search. If disabled, then only four points (i.e. dark blue points) are tested. Figure 6-5. 8-point search with iDist equal to one. For values of iDist from two to eight, the search pattern is shown in Figure 6-6 is used. Figure 6-6. 8-point search with iDist from two to eight. For values of iDist greater than eight, the search pattern shown in Figure 6-7 is used. This search pattern includes diagonal test points located at horizontal and vertical offsets of ±¼, ±½ and ±¾ of iDist, rounded to integer accuracy. Figure 6-7. 8-point search with iDist greater than eight. Alternative square test patterns are also available when constant value bFirstSearchDiamond is set to false. The iterative 8-point search is performed by iterating a variable iDist over powers of two from one to the highest power of two not exceeding the specified search range. For each value of iDist, a set of points is tested as described above. Thus, the iterative 8-point search is the union of the sets of points above over all iterated values of iDist. An early exit for the iDist loop is available using the FastMEAssumingSmootherMVEnabled configuration option. When enabled, the iDist loop terminates after a maximum of three iterations from the last iteration for which a new best motion vector was found. For the diamond search, the corner points (light blue in Figure 6-5 are not tested) and for the enhanced diamond search, these points are tested as part of the iterative 8-point search. For the enhanced diamond search, an iterative 8-point search (with corners not tested) is performed, with iDist iterating to a maximum of half the defined search range, around the (0, 0) position. If iDist is equal to 1, then, due to the previous search at iDist equal to 0, there are only two untested points. These two points are tested. Then, for the next stage of the first search of a diamond search, perform a raster search and for the enhanced diamond search, perform an adaptive raster search. The raster search (enabled by constant value bEnableRasterSearch) is performed if the distance iDist corresponding to the current best motion vector is greater than five. When performing the raster search, iDist for the best motion vector is set to five, and the raster search tests blocks along a sparse grid within the defined search window, testing blocks at every five luma samples horizontally and vertically. For the adaptive raster search, the search window is halved in size). If the iDist of the current best motion vector is less than or equal to five then the current best distance is set to six. Then a raster search is performed using the halved search window, also testing blocks located every five luma samples horizontally and vertically. After the first search completes, a refinement search is performed. By default, a star refinement search is performed, although a raster refinement search is also available in the VTM source code. The refinement search only occurs if the iDist associated with the best motion vector is nonzero. If so, an iterative 8-point search is performed, starting from iDist equal to one, and doubling iDist until the search range is exceeded. By default the diamond pattern of test points is used, however a square pattern can also be used. Selective search The selective search begins by setting the median predictor value as the search starting point and the best MV. Then, predictors PRED_A, PRED_B and PRED_C are tested to see if they provide a better starting point, in which case the preferred predictor is selected. Then the zero motion vector is tested similarly. For an initial search, an initial search window is set, centred at the selected best predictor and one quarter the width and height of the defined search range. Then, the initial search window is sparsely searched, such that every fourth integer pel horizontally and vertically is tested. At each tested point, two 8-point diamond searches are performed, at iDist equal to one and two and with the corner four points disabled. If the L1 distance between the best motion vector and the chosen predictor exceeds a constant of eight, then a full search is performed over a search window relative to the predictor and sized according to the search range. Otherwise, if iDist resulting from the initial search is equal to one, a refinement search is performed. The refinement search performs an iterative 8-point diamond search, with corners always disabled and no early exit tests active. Fast encoder modes Table 6-2 provides a summary of the approaches selected according to the fast encoder mode in use. Table 6-2. Fast encoder mode summary. Fast mode 1; Fast mode 2; Fast mode 3. Subsampled SAD for selective searches; Enabled; Disabled; Enabled. Subsampled SAD for searches other than selective searches; Enabled; Disabled; Enabled. Bi-predictive iterations reduction; Enabled; Enabled; Disabled. Reference picture list adaptive selection; Enabled; Enabled; Disabled. SAD for selective searches To speed up the SAD operation, a subsampling approach is provided where the number of rows tested is reduced. When the selective search is used and RestrictMESampling is enabled, this approach operates as follows: Variable iSubShift results in testing every nth row, such that n = 1 << iSubShift. The variable iSubShift is initialized according to the number of rows in the block under test, such that the spacing between tested rows increases with the block size. Then, the SAD for the block under test is calculated iteratively, with iSubShift decremented on each iteration, until the SAD of the current iteration does not improve upon the best SAD resulting from earlier iterations (with adjustments to compensate for iSubShift variation). SAD for searches other than selective searches Depending on the fast encoder mode, the SAD can be subsampled to either check every row or every second row. Bi-predictive iterations reduction By default, the bi-predictive search performs four iterations. For fast modes 1 and 3, this is reduced to one iteration. Reference picture list adaptive selection By default, the reference picture list to be used for a given iteration of the bi-predictive search is alternatively selected with consecutive iterations, i.e., iterations 0, 2, …, use reference picture list 0 and iterations 1, 3, …, use reference picture list 1. When reference picture lists are adaptively selected, the list to be tested for the bi-predictive search is the opposing list to the best list resulting from the uni-predictive search. Fractional-pel search strategy Fractional refinement is performed in two steps, firstly testing half-pel positions and secondly testing quarter-pel positions. The eight half-pel positions surrounding the current best integer motion vector are tested, with the best (if any) being selected. Then, the eight quarter-pel positions surrounding the current best integer or half-pel motion vector are tested. Bi-predictive search strategy The objective of the bi-predictive motion search is to produce two motion vectors which result in minimum error between the original block (O) and the predicted block P with two predictions P0 and P1, such that P = P0 + P1. In the HM encoder, an iterative uni-predictive search is implemented, providing a reasonable compromise between search duration and minimising error. The bi-predictive search steps implemented in the HM encoder are as follows: - Search P1 which produces minimum error with (2O - P0), where O represents original block and P0 means predictor produced by the first motion vector. P0 is fixed in this step. To derive motion vector for P1, a uni-predictive motion search is performed with the reference samples set to (2O - P0). - Search P0 which produces minimum error with (2O − P1), where O represents original block and P1 means predictor produced by the second motion vector. P1 is the predictor obtained in step 1) and fixed in this step. To get P0, uni-predictive search is utilized after setting (2O − P1) as reference samples. - Repeat steps (1) and (2) until maximum number of iterations is reached. The maximum number of iterations is set to four, unless the fast search option is enabled (in which case steps (1) and (2) are performed once only). Behaviour of the bi-predictive search is also influenced by the fast encoder mode selection.

Conclusion

Herein, it is commented that the same or very similar descriptions and definitions with HM may be put to the algorithm description for VTM. To make the description document readable and clarified more, we recommend that the aforementioned comments be resolved in the next algorithm description document for VTM.

References

[1] “High Efficiency Video Coding (HEVC) Test Model 16 (HM 16) Encoder Description Update 9,” JCTVC-AB1002, Torino, IT, 15-21 July, 2017.

[2] “Algorithm description for Versatile Video Coding and Test Model 4 (VTM 4),” JVET-M1002, Marrakech, MA, 9-18 Jan, 2019.

Patent rights declaration(s)

Ewha Womans University does not have any current or pending patent rights relating to the technology described in this contribution.